| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0157 |
| --- | --- |

| Title: | AHG17: On picture order count for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The contribution proposes a syntax element (nuh_pic_id) in the NAL unit header, which has the same value with the LSB values of the picture order count. The proposed syntax element (nuh_pic_id) is an identifier for each picture and can be used for the access unit identification at the system level. A network element or a video player can easily access VCL NAL units belonging to the same access unit. It is asserted that, especially when multiple video components are coded with multiple layers (or with multiple coded vide sequences) and simultaneously contained in the same coded bitstream, exposing the value of the picture order count in NAL unit header layer is very helpful for a media player or a file writer to access pictures belonging the same access unit or the same time instance, and synchronize multiple NAL units corresponding to different components for extracting, editing, writing or playing back.

# Proposal

The contribution proposes a syntax element (nuh_pic_id) in a NAL unit header of a VCL NAL unit, which has the same value with the LSBs of the picture order count (POC) of the picture contained in the NAL unit. The proposed syntax element (nuh_pic_id) is an identifier for each picture or for each access unit. The value of nuh_pic_id of a VCL NAL unit shall not be equal to the value of nuh_group_id of any other VCL NAL unit in the same coded video sequence (CVS), as long as the number of pictures in the CVS is not greater than 255.

The proposed syntax element (nuh_pic_id) is an identifier for each picture and can be used for the access unit identification at the system level. A network element or a video player can easily access VCL NAL units belonging to the same access unit, for extracting, editing, writing or playing back.

#### NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(32) |
| nuh_reserved_zero_7bits | u(7) |
| nuh_pic_id | u(8) |
| } |  |

Or, in accordance with the NAL unit header design in JVET-M0520:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nuh_unit_class | u(4) |
| nuh_temporal_id_plus1 | u(3) |
| . . . |  |
| if( nuh_unit_class == 9 ) { /* Complex */ | u(1) |
| nuh_pic_id_present_flag | u(1) |
| if( nuh_pic_id_present_flag ) { |  |
| nuh_pic_id | u(8) |
| } |  |
| } |  |
| byte_align() |  |
| } |  |

nuh_pic_id specifies the picture id of the VCL NAL unit. The value of nuh_pic_id shall be equal to the value of LSB 8bits of PicOrderCntVal. The value of nuh_pic_id shall be in the range of 0 to 255, inclusive. When the corresponding NAL unit is a non-VCL NAL unit, the value of nuh_pic_id is set equal to 0.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).