| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0158-v1 |
| --- | --- |

| Title: | Non-CE3: LUT-free interpolation filters for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Jianle Chen | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co. |  |  |

_____________________________

# Abstract

This document proposes LUT-free interpolation filters for intra prediction. Replacing the Gaussian interpolation filter by its analytic representation reportedly shows the following overall results: -0.00% (Y), -0.02% (U), -0.01% (V) and -0.01% (Y), 0.03% (U), 0.09% (V) for AI and RA configurations, respectively. Replacing both the Gaussian and DCT-IF interpolation filters by their analytic representations reportedly shows the following overall results: -0.01% (Y), -0.02% (U), -0.05% (V) and -0.01% (Y), 0.03% (U), 0.02% (V) for AI and RA configurations, respectively. The running time is unchanged at both encoder and decoder sides for AI and RA configurations as compared with VTM-3.0.

# Introduction

In the JVET-L meeting, 4-tap interpolation filters for intra prediction were adopted into the VTM software and the VVC spec draft as proposed in [1]. Coefficients of both interpolation filters (DCT-IF and Gaussian filter) are stored in LUTs that contain 32 entries of 4 coefficients for each filter.

# Proposal

In this contribution, we propose to use analytic representations of filter coefficients for both interpolation filters (DCT-IF and Gaussian filter) rather than LUT-based ones as done in the VTM-3.0 software. To calculate the coefficients of the smoothing filter that replaces the Gaussian filter, the following formulas are used:

tmpDelta = deltaFract >> 1;

f[0] = 16 - tmpDelta;

f[1] = 32 - tmpDelta;                                                                                                      (1)

f[2] = 16 + tmpDelta;

f[3] =      tmpDelta;

where tmpDelta denotes fractional sample position.

Coefficients calculated using formula (1) are given in Table 1. The actual number of entries in this table is equal to 16 but not to 32, because rows in Table 1 are replicated.

Table 1 – Specification of intra-prediction interpolation filter coefficients of the smoothing interpolation filter

| deltaFract | f[0] | f[1] | f[2] | f[3] |
| --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 16 | 0 |
| 1 | 16 | 32 | 16 | 0 |
| 2 | 15 | 31 | 17 | 1 |
| 3 | 15 | 31 | 17 | 1 |
| 4 | 14 | 30 | 18 | 2 |
| 5 | 14 | 30 | 18 | 2 |
| 6 | 13 | 29 | 19 | 3 |
| 7 | 13 | 29 | 19 | 3 |
| 8 | 12 | 28 | 20 | 4 |
| 9 | 12 | 28 | 20 | 4 |
| 10 | 11 | 27 | 21 | 5 |
| 11 | 11 | 27 | 21 | 5 |
| 12 | 10 | 26 | 22 | 6 |
| 13 | 10 | 26 | 22 | 6 |
| 14 | 9 | 25 | 23 | 7 |
| 15 | 9 | 25 | 23 | 7 |
| 16 | 8 | 24 | 24 | 8 |
| 17 | 8 | 24 | 24 | 8 |
| 18 | 7 | 23 | 25 | 9 |
| 19 | 7 | 23 | 25 | 9 |
| 20 | 6 | 22 | 26 | 10 |
| 21 | 6 | 22 | 26 | 10 |
| 22 | 5 | 21 | 27 | 11 |
| 23 | 5 | 21 | 27 | 11 |
| 24 | 4 | 20 | 28 | 12 |
| 25 | 4 | 20 | 28 | 12 |
| 26 | 3 | 19 | 29 | 13 |
| 27 | 3 | 19 | 29 | 13 |
| 28 | 2 | 18 | 30 | 14 |
| 29 | 2 | 18 | 30 | 14 |
| 30 | 1 | 17 | 31 | 15 |
| 31 | 1 | 17 | 31 | 15 |

To compute the coefficients of the sharpening filter that replaces the DCT-IF, the following formulas are used:

tmpDelta = (deltaFract & 0x10 ? -(~(31 ^ deltaFract)) : deltaFract);

tmpDelta = ((tmpDelta << 1) + tmpDelta) >> 3;

f[0] =                        - tmpDelta;

f[1] = 64 - (deltaFract << 1) + tmpDelta;                                                                (2)

f[2] =      (deltaFract << 1) + tmpDelta;

f[3] =                        - tmpDelta;

Coefficients calculated using formulas (2) are given in Table 2.

Table 2 – Specification of intra-prediction interpolation filter coefficients of the sharpening filter

| deltaFract | f[0] | f[1] | f[2] | f[3] |
| --- | --- | --- | --- | --- |
| 0 | 0 | 64 | 0 | 0 |
| 1 | 0 | 62 | 2 | 0 |
| 2 | 0 | 60 | 4 | 0 |
| 3 | -1 | 59 | 7 | -1 |
| 4 | -1 | 57 | 9 | -1 |
| 5 | -1 | 55 | 11 | -1 |
| 6 | -2 | 54 | 14 | -2 |
| 7 | -2 | 52 | 16 | -2 |
| 8 | -3 | 51 | 19 | -3 |
| 9 | -3 | 49 | 21 | -3 |
| 10 | -3 | 47 | 23 | -3 |
| 11 | -4 | 46 | 26 | -4 |
| 12 | -4 | 44 | 28 | -4 |
| 13 | -4 | 42 | 30 | -4 |
| 14 | -5 | 41 | 33 | -5 |
| 15 | -5 | 39 | 35 | -5 |
| 16 | -6 | 38 | 38 | -6 |
| 17 | -5 | 35 | 39 | -5 |
| 18 | -5 | 33 | 41 | -5 |
| 19 | -4 | 30 | 42 | -4 |
| 20 | -4 | 28 | 44 | -4 |
| 21 | -4 | 26 | 46 | -4 |
| 22 | -3 | 23 | 47 | -3 |
| 23 | -3 | 21 | 49 | -3 |
| 24 | -3 | 19 | 51 | -3 |
| 25 | -2 | 16 | 52 | -2 |
| 26 | -2 | 14 | 54 | -2 |
| 27 | -1 | 11 | 55 | -1 |
| 28 | -1 | 9 | 57 | -1 |
| 29 | -1 | 7 | 59 | -1 |
| 30 | 0 | 4 | 60 | 0 |
| 31 | 0 | 2 | 62 | 0 |

We tested 2 configurations summarized in Table 3.

Table 3. Summary of 2 proposed test

| Test # | Description |
| --- | --- |
| 1 | Gaussian filter is replaced by its analytic representation according to (1) |
| 2 | Both DCT-IF and Gaussian filter are replaced by their analytic representations according to (1) and (2), respectively |

# Experiment Results

The results obtained using the common test conditions [2] are reportedly presented in Table 4 and Table 5 for AI and RA configurations.

Table 4. Experimental results for Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.06% | 100% | 99% |
| Class A2 | 0.00% | 0.05% | -0.01% | 99% | 100% |
| Class B | 0.00% | -0.02% | -0.05% | 99% | 100% |
| Class C | 0.00% | -0.09% | -0.11% | 100% | 99% |
| Class E | 0.01% | -0.05% | 0.10% | 99% | 100% |
| Overall | 0.00% | -0.02% | -0.01% | 99% | 100% |
| Class D | 0.01% | 0.08% | -0.13% | 99% | 99% |
| Class F | -0.01% | 0.02% | 0.09% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | 0.05% | 100% | 100% |
| Class A2 | -0.03% | 0.00% | 0.13% | 100% | 100% |
| Class B | -0.01% | 0.10% | 0.02% | 100% | 100% |
| Class C | -0.01% | -0.07% | 0.17% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | 0.09% | 100% | 100% |
| Class D | 0.03% | -0.40% | -0.42% | 99% | 99% |
| Class F | -0.03% | 0.09% | 0.14% | 100% | 100% |

Table 5. Experimental results for Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.01% | 100% | 99% |
| Class A2 | -0.02% | 0.06% | -0.02% | 100% | 99% |
| Class B | 0.00% | -0.02% | -0.08% | 100% | 99% |
| Class C | 0.02% | -0.01% | -0.13% | 100% | 100% |
| Class E | -0.04% | -0.10% | 0.00% | 100% | 100% |
| Overall | -0.01% | -0.02% | -0.05% | 100% | 99% |
| Class D | -0.02% | 0.02% | -0.10% | 100% | 100% |
| Class F | 0.07% | -0.02% | 0.05% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.00% | 99% | 99% |
| Class A2 | 0.00% | 0.10% | 0.09% | 99% | 100% |
| Class B | -0.02% | 0.16% | -0.06% | 99% | 100% |
| Class C | 0.00% | -0.14% | 0.11% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | 0.02% | 99% | 100% |
| Class D | -0.03% | -0.33% | 0.07% | 99% | 99% |
| Class F | 0.01% | 0.03% | 0.13% | 99% | 99% |

# Conclusion

We propose to adopt the LUT-free representation of the interpolation filters for intra prediction into the VTM software and the VVC spec draft.

# References

- A. Filippov, V. Rufitskiy, J. Chen, G. Van der Auwera, A.K. Ramasubramonian, V. Seregin, T. Hsieh, M. Karczewicz, “CE3: A combination of tests 3.1.2 and 3.1.4 for intra reference sample interpolation filter”, JVET-L0628, 12th JVET Meeting, Macao, China, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th JVET Meeting, Macao, China, October 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).