| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0161-v1 |
| --- | --- |

| Title: | AHG17: Signalling random access properties in the NAL unit header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to signal random access properties in the network abstraction layer (NAL) unit header. The overall NAL unit header remains a 16-bit structure. The random access properties indicate whether the leading pictures, if present, are decodable and/or used as references for the trailing pictures. The commonly considered closed or open group of pictures (GOP) may be signalled accordingly. The cases for instantaneous decoding refresh (IDR), dependent random access point (DRAP), and gradual decoding refresh (GDR) pictures are also addressed. The signalled properties may be used for random access processing with a desired quality. The suggested syntax and semantics are provided.

# Introduction

In the WD of VVC [1], the NAL unit header syntax and NAL unit type codes and NAL unit type classes are in 7.3.1.2 and Table 7-1, respectively.

7.3.1.2 NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_7bits | u(7) |
| } |  |

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | NON_IRAP_NUT | Coded slice of a non-IRAP picture slice_layer_rbsp( ) | VCL |
| 1 | IRAP_NUT | Coded slice of an IRAP picture slice_layer_rbsp( ) | VCL |
| 2-15 | RSV_VCL_NUT | Reserved VCL NAL Units | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20, 21 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 22-26 | RSV_NVCL | Reserved | non-VCL |
| 27-31 | UNSPEC | Unspecified | non-VCL |

The current NAL unit header has a 16-bit field to signal the NAL unit type and the temporal id. From system and application point of view, signalling or exposing more information, such as random access properties, of coded pictures at a higher syntax level are desired.

The following definitions referenced in this contribution are mainly from the HEVC, as below.

Definitions

intra random access point (IRAP) picture: A coded picture that does not refer to any pictures other than itself for inter prediction in its decoding process.

instantaneous decoding refresh (IDR) picture: An IRAP picture that is the first picture of a codec video sequence (CVS) in decoding order.

non-IRAP picture: A coded picture which is not an IRAP picture.

associated IRAP picture: The previous IRAP picture in decoding order.

leading picture: A non-IRAP picture that precedes the associated IRAP picture in output order.

trailing picture: A non-IRAP picture that follows the associated IRAP picture in output order.

random access: The act of starting the decoding process for a bitstream at a point other than the beginning of the stream.

dependent random access point (DRAP) picture: A trailing picture and it contains no references to pictures other than its associated IRAP picture. Any picture that follows the DRAP picture in both decoding order and output order shall not contain references to any picture that precedes the DRAP picture in decoding order or output order with the exception of the IRAP picture associated with the DRAP picture.

recovery point: A point in the bitstream at which the recovery of an exact or an approximate representation of the decoded pictures represented by the bitstream is achieved after a random access.

gradual decoding refresh (GDR) picture: A non-IRAP picture that contains the refreshed regions relating to the indicated recovery point.

# Proposed method

This contribution proposes to signal random access properties in the NAL unit header. The 16-bit length of the NAL unit header remains as in the current WD of VVC. For IRAP picture, DRAP picture and GDR picture (which may be used for random access processing), nuh_temporal_id_plus1 of the picture is inferred to be 1. The bit fields for IDR picture and DRAP picture ends early with end_nuh_field_flag equal to 1. Other types of picture signalling continues to use subsequent bit fields with end_nuh_field_flag equal to 0.

NAL unit header syntax

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_unit_type | u(5) |
| end_nuh_field_flag | f(1) |
| if( nal_unit_type == IRAP_NUT ) { |  |
| random_access_type | f(2) |
| num_leading_pic_plus1 | u(4) |
| nuh_reserved_zero_3bits | u(3) |
| } else if( nal_unit_type == GDR_NUT ) { |  |
| refreshed_region_flag | f(1) |
| gdr_pic_count | u(4) |
| nuh_reserved_zero_4bits | u(4) |
| } else { |  |
| nuh_temporal_id_plus1 | u(3) |
| nuh_reserved_zero_6bits | u(6) |
| } |  |
| } |  |

NAL unit header semantics

forbidden_zero_bit shall be equal to 0.

nal_unit_type specifies the type of RBSP data structure contained in the NAL unit as specified in Table 7-1 with the added nal_unit_type equal to 2 for GDR_NUT.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | NON_IRAP_NUT | Coded slice of a non-IRAP picture slice_layer_rbsp( ) | VCL |
| 1 | IRAP_NUT | Coded slice of an IRAP picture slice_layer_rbsp( ) | VCL |
| 2 | GDR_NUT | Coded slice of a GDR picture slice_layer_rbsp( ) | VCL |
| 3-15 | RSV_VCL_NUT | Reserved VCL NAL Units | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20, 21 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 22-26 | RSV_NVCL | Reserved | non-VCL |
| 27-31 | UNSPEC | Unspecified | non-VCL |

end_nuh_field_flag equal to 1 specifies that the subsequent bits shall be ignored, when nal_unit_type equal to IRAP_NUT, the content is coded slice of an instantaneous decoding refresh (IDR) picture; when nal_unit_type equal to NON_IRAP_NUT, the content is coded slice of a dependent random access point (DRAP) picture. end_nuh_field_flag equal to 0 specifies that the subsequent bits shall be used for the NAL unit type.

random_access_type specifies the random access type associated with the NAL unit as specified in Table 7-2.

Table 7-2 – random_access_type values

| random_access_type values | Name of random access type | Description |
| --- | --- | --- |
| 0 | RAP_TYPE1 | The leading pictures are decodable and may or may not be used as references for the decoding process of trailing pictures of the same associated IRAP picture. A closed GOP random access point at which a coded picture is an IRAP picture with num_leading_pic_plus1 greater than or equal to 1. |
| 1 | RAP_TYPE2 | The leading pictures may or may not be decodable and may or may not be used as references for the the decoding process of trailing pictures of the same associated IRAP picture. An open GOP random access point at which a coded picture is an IRAP picture with num_leading_pic_plus1 greater than 1. |
| 2 | RAP_TYPE3 | The leading pictures may or may not be decodable but are not used as references for the decoding process of trailing pictures of the same associated IRAP picture. An open GOP random access point at which a coded picture is an IRAP picture with num_leading_pic_plus1 greater than 1. |
| 3 | RAP_TYPE4 | Reserved. (RAP_TYPE3 is for IRAP similar to BLA and CRA in HEVC. RAP_TYPE4 may be used with RAP_TYPE3 to differentiate the two for defining corresponding decoding process and conformance test., i.e. RAP_TYPE3 indicates a BLK and RAP_TYPE4 indicates a CRA. The decoding process may be in a separate contribution) |

Note: RAP_TYPE1 may be used to signal a closed GOP that a seamless random access may be achievable. RAP_TYPE2 may be used to signal an open GOP that the random access may cause decoding errors as references to pictures that may not be present in the bitstream. RAP_TYPE3 may be used to signal an open GOP that the leading pictures are not used as reference pictures for the decoding process of trailing pictures of the same associated IRAP picture and the random access may be achievable with a desired quality depending on capability of handling the leading pictures in decoding applications.

num_leading_pic_plus1 minus 1, for random_access_type equal to 0, 1 or 2, specifies the number of leading pictures of the associated IRAP picture.

refreshed_region_flag equal to 1 indicates that the slice belong to the refreshed region in the current picture. refreshed_region_flag equal to 0 indicates that the slice may not belong to the refreshed region in the current picture.

gdr_pic_count indicates the number of current and subsequent decoded GDR pictures in output order that precedes the recovery point. gdr_pic_count is a counter and it shall be 1 for the last decoded GDR pictures in output order that precedes the recovery point.

nuh_temporal_id_plus1 minus 1 specifies a temporal identifier for the NAL unit. The value of nuh_temporal_id_plus1 shall not be equal to 0. When nuh_temporal_id_plus1 is not present, it is inferred to be equal to 1 (e.g. it is inferred to be 1 for IDR pictures, DRAP pictures, and GDR pictures).

nuh_reserved_zero_3bits shall be equal to '000'. Other values of nuh_reserved_zero_3bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with values of nuh_reserved_zero_3bits not equal to '000'.

nuh_reserved_zero_4bits shall be equal to '0000'. Other values of nuh_reserved_zero_4bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with values of nuh_reserved_zero_4bits not equal to '0000'.

nuh_reserved_zero_6bits shall be equal to '000000'. Other values of nuh_reserved_zero_6bits may be specified in the future by ITU-T | ISO/IEC. Decoders shall ignore (i.e. remove from the bitstream and discard) NAL units with values of nuh_reserved_zero_6bits not equal to '000000'.

The random access related information signalled in the NAL unit header may be used directly by applications without parsing more syntax elements in the bitstream or an SEI message. An application or a system layer may decide to use them for various random access processing, such as program/channel switching, content splicing and editing etc. The processing result and quality may be “seamless” in terms of the signaled properties, otherwise, though not seamless, it may be dependent on the application and decoding capabilities for qualities within the expectation.

# Reference

- “Versatile Video Coding (Draft 3)”, JVET-L1001-v7.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).