| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0162-v1 |
| --- | --- |

| Title: | Adaptive loop filter with a maximum number of luma filters per slice constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Zhi-Yi Lin, Chen-Yen Lai, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chingyeh.chen, zhi-yi.lin, jenny.lai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a “maximum number of luma filters per slice” constraint to adaptive loop filter (ALF) in order to reduce the storage of filter coefficients in the on-chip memory. In the current ALF design, up to 25 luma filters can be signalled and used in one slice, which requires one on-chip memory with size equal to 25 filters x 13 coefficients per filter x 8 bits per coefficient = 2600 bits. The proposed method is to constrain the number of luma filters per slice. When the maximum number of luma filters is reduced from 25 to 16, the BD-rates for AI, RA, and LB are 0.00%, 0.00%, and 0.00%, respectively and 36% on-chip memory of luma filter coefficients is saved. When the maximum number of luma filters is 12, the BD-rates for AI, RA, and LB are 0.01%, 0.01%, and -0.01%, respectively, and 52% on-chip memory of luma filter coefficients is saved. When the maximum number of luma filters is 8, the BD-rates for AI, RA, and LB are 0.04%, 0.04%, and 0.01%, respectively, and 68% on-chip memory of luma filter coefficients is saved.

# Proposed method

In VTM3.0 [1], ALF utilizes block-based classification to achieve block-level filter adaptation for luma component. One luma picture is divided into 4x4 luma blocks, and one 4x4 luma block is classified into one of 25 groups. For each group, one filter is designed to reduce the compression error, and multiple groups can share one filter. However, in the worst case (no filters are shared across groups), the number of luma filters per slice is 25. This implies that one on-chip memory with size equal to 25x13=325 bytes is required to store filter coefficients of 25 luma filters. In order to reduce the on-chip memory size, one constraint related to the number of luma filters per slice is proposed. When the maximum number of luma filters per slice is reduced from 25 to 16, then the on-chip memory is reduced from 325 to 16x13=208 bytes, which is only 64% of the original one. When the maximum number of luma filters per slice is 12, the on-chip memory size is 156 bytes (52% saving). When the maximum number of luma filters per slice is 8, the on-chip memory size is 104 bytes (68% saving).

# Experimental results

The ALF with the “maximum number of filters per slice” constraint has been implemented on VTM3.0 [1]. The simulations were performed following the JVET common test conditions (CTC) [2]. Table 1, Table 2, and Table 3 show the simulation results when the maximum number of luma filters per slice is set as 16, 12, and 8, respectively.

Table 1.  Simulation results of ALF with up to 16 luma filters per slice.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | -0.01% | -0.01% | 100% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | -0.01% | -0.01% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | 0.05% | 0.05% | 100% | 98% |
| Class B | 0.01% | 0.01% | -0.02% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | -0.01% | -0.01% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.02% | 0.05% | 100% | 100% |
| Class C | 0.00% | 0.01% | 0.03% | 100% | 100% |
| Class E | 0.00% | -0.02% | -0.02% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class D | -0.02% | -0.03% | 0.13% | 101% | 101% |
| Class F | 0.00% | -0.03% | 0.03% | 100% | 100% |

Table 2.  Simulation results of ALF with up to 12 luma filters per slice.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.01% | 100% | 99% |
| Class A2 | 0.02% | -0.02% | -0.02% | 100% | 100% |
| Class B | 0.01% | -0.02% | -0.02% | 100% | 100% |
| Class C | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Class E | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Overall | 0.01% | -0.01% | -0.01% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.01% | -0.02% | -0.02% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | -0.06% | 100% | 100% |
| Class A2 | 0.01% | 0.13% | 0.06% | 100% | 98% |
| Class B | 0.01% | 0.12% | -0.03% | 100% | 100% |
| Class C | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.06% | -0.01% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | -0.01% | 0.04% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.30% | 0.08% | 100% | 101% |
| Class C | -0.01% | 0.09% | 0.07% | 100% | 100% |
| Class E | 0.00% | -0.02% | 0.08% | 100% | 100% |
| Overall | -0.01% | -0.10% | 0.08% | 100% | 100% |
| Class D | -0.01% | 0.01% | -0.21% | 100% | 100% |
| Class F | -0.03% | 0.11% | 0.02% | 101% | 100% |

Table 3.  Simulation results of ALF with up to 8 luma filters per slice.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | -0.03% | 100% | 100% |
| Class A2 | 0.07% | -0.03% | -0.03% | 100% | 101% |
| Class B | 0.03% | -0.04% | -0.05% | 100% | 100% |
| Class C | 0.03% | -0.06% | -0.06% | 100% | 101% |
| Class E | 0.02% | -0.05% | -0.05% | 101% | 101% |
| Overall | 0.04% | -0.05% | -0.05% | 100% | 101% |
| Class D | 0.00% | -0.01% | -0.01% | 101% | 104% |
| Class F | 0.04% | -0.06% | -0.06% | 102% | 104% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | 0.02% | 100% | 103% |
| Class A2 | 0.05% | 0.00% | 0.05% | 100% | 101% |
| Class B | 0.05% | 0.01% | -0.14% | 100% | 102% |
| Class C | 0.03% | 0.05% | -0.05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.00% | -0.04% | 100% | 101% |
| Class D | -0.01% | 0.02% | -0.01% | 100% | 100% |
| Class F | 0.02% | -0.01% | -0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.24% | 0.04% | 100% | 100% |
| Class C | 0.03% | 0.14% | 0.02% | 101% | 102% |
| Class E | -0.06% | -0.29% | -0.05% | 101% | 101% |
| Overall | 0.01% | -0.13% | 0.01% | 101% | 101% |
| Class D | 0.00% | 0.26% | 0.08% | 100% | 100% |
| Class F | 0.08% | -0.28% | 0.16% | 101% | 102% |

# Conclusions

This contribution proposes to add a “maximum number of luma filters per slice” constraint for ALF in order to reduce the on-chip memory requirement. When the maximum number of luma filters per slice is reduced from 25 to 16, the BD-rates for AI, RA, and LB are 0.00%, 0.00%, and 0.00%, respectively, and 36% on-chip memory of luma filter coefficients is saved. When the maximum number of luma filters per slice is 12, the BD-rates for AI, RA, and LB are 0.01%, 0.01%, and -0.01%, respectively, and 52% on-chip memory of luma filter coefficients is saved. When the maximum number of luma filters per slice is 8, the BD-rates for AI, RA, and LB are 0.04%, 0.04%, and 0.01%, respectively, and 68% on-chip memory of luma filter coefficients is saved.

# References

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).