| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0163-v1 |
| --- | --- |

| Title: | Adaptive loop filter with history filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen, Zhi-Yi Lin, Chen-Yen Lai, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chingyeh.chen, zhi-yi.lin, jenny.lai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes history filters in adaptive loop filter (ALF). The concept of history filters is to allow using history filters for the current slice to increase coding efficiency, where the history filters are decoded filters from previously decoded slices. When the maximum number of history filter set is five, where one filter set contains all signalled filters of one slice, BD-rate savings are 0.16% and 0.34% for RA and LB, respectively, and the required memory of history filter set storage is 1660 bytes (5 sets * 25 luma filters per set * 13 coefficients per luma filter * 1 byte per coefficient + 5 sets * 1 chroma filter per set * 7 coefficients per chroma filter * 1 byte per coefficient). To reduce the memory requirement, it is further proposed to apply history filters with the “maximum number of luma filters per slice” constraint in JVET-M0162. When the maximum number of luma filters per slice is reduced from 25 to 16, BD-rate savings are 0.16% and 0.33% for RA and LB, respectively, and the required memory size is 1075 bytes (65% of no constraint). When the maximum number of luma filters per slice is 12, BD-rate savings are 0.15% and 0.33% for RA and LB, respectively, and the required memory size is 815 bytes (49% of no constraint).

# Proposed method

The concept of ALF with history filters is to reuse decoded filter sets from previously decoded slices for the current slice to improve coding efficiency, where one filter set contains all signalled filters of one slice. A cost is an increase of memory to store decoded filter sets. For example, if up to five decoded filters sets are reused, the memory size is 1660 bytes (5 sets * 25 luma filters per set * 13 coefficients per luma filter * 1 byte per coefficient + 5 sets * 1 chroma filter per set * 7 coefficients per chroma filter * 1 byte per coefficient). To reduce the memory requirement, the “maximum number of luma filters per slice” constraint as in JVET-M0162 is combined with history filters. When the maximum number of luma filters per slice is reduced from 25 to 16, the memory size is reduced to 1075 bytes, which is only 65% of no constraint. If we further reduce the maximum number of luma filters per slice to 12, the required memory size becomes 815 bytes, which is only 49% of no constraint.

# Experimental results

The ALF with history filters has been implemented on VTM3.0 [1]. The simulations were performed following the JVET common test conditions (CTC) [2]. Table 1 shows the simulation results of ALF with five history filter sets. Table 2, Table 3, and Table 4 show the simulation results of ALF with five history filter sets combined with the maximum number of luma filters per slice equal to 16, 12, and 8 respectively. The run time numbers are not accurate.

Table 1.  Results of ALF with five history filter sets and up to 25 luma filters per slice.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.20% | -0.06% | 96% | 105% |
| Class A2 | -0.11% | -0.07% | -0.28% | 96% | 105% |
| Class B | -0.14% | -0.15% | -0.18% | 98% | 101% |
| Class C | -0.26% | -0.30% | -0.23% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.10% | -0.19% | 97% | 102% |
| Class D | -0.46% | -0.60% | -0.54% | 96% | 101% |
| Class F | -0.13% | -0.11% | -0.09% | 97% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | -0.35% | -0.16% | 104% | 102% |
| Class C | -0.34% | -0.20% | -0.07% | 99% | 98% |
| Class E | -0.60% | -1.15% | -0.71% | 104% | 105% |
| Overall | -0.34% | -0.50% | -0.27% | 102% | 101% |
| Class D | -0.56% | -0.70% | -0.18% | 104% | 105% |
| Class F | -0.20% | -0.11% | -0.13% | 98% | 98% |

Table 2.  Results of ALF with five history filter sets and up to 16 luma filters per slice.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.19% | -0.04% | 100% | 101% |
| Class A2 | -0.10% | -0.02% | -0.26% | 100% | 106% |
| Class B | -0.15% | -0.13% | -0.18% | 100% | 104% |
| Class C | -0.27% | -0.31% | -0.22% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.09% | -0.18% | 100% | 103% |
| Class D | -0.46% | -0.60% | -0.54% | 100% | 102% |
| Class F | -0.13% | -0.11% | -0.11% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | -0.26% | -0.21% | 100% | 101% |
| Class C | -0.35% | -0.15% | -0.11% | 101% | 108% |
| Class E | -0.59% | -1.13% | -0.69% | 101% | 104% |
| Overall | -0.33% | -0.44% | -0.30% | 101% | 104% |
| Class D | -0.56% | -0.70% | -0.18% | 100% | 103% |
| Class F | -0.25% | -0.04% | -0.51% | 100% | 102% |

Table 3.  Results of ALF with five history filter sets and up to 12 luma filters per slice.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.17% | -0.12% | 100% | 97% |
| Class A2 | -0.10% | 0.10% | -0.24% | 100% | 102% |
| Class B | -0.14% | -0.06% | -0.20% | 100% | 100% |
| Class C | -0.24% | -0.29% | -0.20% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.15% | -0.04% | -0.19% | 100% | 101% |
| Class D | -0.46% | -0.60% | -0.54% | 100% | 102% |
| Class F | -0.11% | -0.11% | -0.07% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | -0.44% | -0.10% | 100% | 101% |
| Class C | -0.35% | -0.09% | -0.03% | 100% | 101% |
| Class E | -0.60% | -1.22% | -0.69% | 101% | 105% |
| Overall | -0.33% | -0.52% | -0.22% | 101% | 102% |
| Class D | -0.56% | -0.70% | -0.18% | 100% | 102% |
| Class F | -0.25% | 0.03% | -0.42% | 101% | 103% |

Table 4.  Results of ALF with five history filter sets and up to eight luma filters per slice.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.10% | -0.08% | 96% | 104% |
| Class A2 | -0.03% | 0.00% | -0.19% | 96% | 104% |
| Class B | -0.09% | -0.09% | -0.28% | 101% | 102% |
| Class C | -0.23% | -0.30% | -0.29% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.09% | -0.22% | 98% | 102% |
| Class D | -0.47% | -0.59% | -0.53% | 98% | 102% |
| Class F | -0.11% | -0.14% | -0.12% | 102% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.10% | -0.40% | -0.12% | 103% | 103% |
| Class C | -0.33% | -0.12% | 0.12% | 100% | 100% |
| Class E | -0.60% | -0.76% | -0.06% | 104% | 105% |
| Overall | -0.30% | -0.40% | -0.02% | 102% | 102% |
| Class D | -0.60% | -0.53% | 0.13% | 105% | 107% |
| Class F | -0.33% | -0.33% | -0.18% | 99% | 104% |

# Conclusions

This contribution combines history filters in ALF with a constraint on maximum number of luma filters per slice. The coding efficiency can be improved (0.2% - 0.3% BD-rate saving), and the complexity increase is very minor in terms of memory size (<1000 bytes).

# References

- VTM3.0: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).