| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0175-v1 |
| --- | --- |

| Title: | CE8-related: Clarification on interaction between CPR and other inter coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Tzu-Der Chuang, Yu-Ling Hsiao, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, peter.chuang, yuling.hsiao, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VTM3.0 encoder, current picture referencing (CPR) for each coding unit (CU) is never tested with combined inter intra prediction (CIIP), merge mode with motion vector difference (MMVD), and subblock modes including subblock-based temporal motion vector prediction (SbTMVP) merge mode, affine merge mode, and affine inter mode. When the reference picture list contains only the current picture and no other reference picture, mh_intra_flag (indicating CIIP on/off of the CU; it is suggested to rename it as ciip_flag), mmvd_flag (indicating MMVD on/off of the CU), merge_subblock_flag (equal to 1 indicating SbTMVP merge or affine merge is on; equal to 0 indicating SbTMVP merge and affine merge are off), and inter_affine_flag (indicating affine inter mode on/off) are always zero and still signalled. In addition, in both VTM3.0 decoder and VVC draft text JVET-L1001-v7.docx, decoding process is not defined. This contribution proposes two methods to deal with this issue. Method 1 is to add a normative encoder-only constraint in the VVC draft text, i.e., conformant encoders shall not enable any of the four flags when CPR is enabled at CU level. Method 2 is to skip signalling and parsing the above four flags when the reference picture list only contains the current picture at encoder and decoder, respectively. Method 1 does not cause any BD-rate change for VTM3.0. Method 2 reportedly causes minor BD-rate impact. It is claimed that Method 2 is a preferred solution than Method 1 in order to reduce the possibility of related illegal bitstreams generated by careless encoders.

# Introduction

CPR is adopted in JVET-L meeting. A constraint limiting reference samples within the current coding tree unit (CTU) is used. In VTM3.0, when the reference picture list only contains the current picture, MHIP, MMVD, and subblock modes including subblock-based merge mode and affine inter mode, are not tested with CPR and thus never enabled at the encoder side, but related syntax elements are still signalled. In addition, in both VTM3.0 decoder and VVC draft text JVET-L1001-v7.docx, decoding process is not defined. In this contribution, two methods are proposed to solve this problem.

# Proposed methods

## Method 1

In Method 1, it is proposed to add a normative encoder-only constraint in the VVC draft text. Conformant encoders shall not enable any of the four flags, mh_intra_flag, mmvd_flag, merge_subblock_flag, and inter_affine_flag, when CPR is enabled at CU level.

## Method 2

In Method 2, it is proposed to skip sending and parsing mh_intra_flag, mmvd_flag, merge_subblock_flag, and inter_affine_flag when the reference picture list only contains the current picture at encoder and decoder, respectively. The four flags are inferred to be zero in this case at the decoder side.

# Experimental results

Method 1 does not need to change VTM3.0 software at all, so it results in no encoding or decoding result changes. Method 2 has been integrated into VTM3.0. All the tests were conducted according to the AI and RA common test conditions [1] except that the anchor enables CPR. Additional tests for class F and TGM sequences are also tested. Table 1 shows the test results.

Table 1.  Results of the proposed syntax redundancy removal in Method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.00% | 100% | 99% |
| Class A2 | -0.01% | -0.01% | 0.02% | 100% | 101% |
| Class B | 0.00% | -0.02% | 0.02% | 100% | 100% |
| Class C | 0.00% | 0.01% | 0.00% | 100% | 105% |
| Class E | 0.00% | 0.02% | -0.06% | 100% | 106% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class D | -0.01% | 0.01% | -0.05% | 100% | 100% |
| Class F (optional) | 0.00% | -0.08% | -0.04% | 101% | 104% |
| TGM | -0.02% | -0.01% | -0.05% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.10% | 0.03% | 100% | 97% |
| Class A2 | 0.01% | 0.08% | -0.04% | 100% | 97% |
| Class B | 0.01% | -0.11% | -0.16% | 100% | 100% |
| Class C | -0.01% | -0.09% | -0.14% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.09% | 100% | 99% |
| Class D | 0.01% | -0.19% | 0.11% | 102% | 100% |
| Class F (optional) | -0.01% | 0.13% | 0.05% | 100% | 100% |
| TGM | 0.02% | -0.03% | 0.02% | 100% | 97% |

# Conclusions

This contribution proposes to remove the signalling of mh_intra_flag, mmvd_flag, merge_subblock_flag, and inter_affine_flag, when the reference picture list only contains the current picture. It is reported that the BD-rates are 0.00%, 0.00%, -0.02% for ClassA/B/C, ClassF, and TGM in AI, respectively. A normative encoder-only constraint is also proposed as another solution to fix the problem of undefined decoding process when CPR is on and any of the four flags is on at CU level.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-L1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).