| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0189-v1 |
| --- | --- |

| Title: | CE10: AMVP mode for triangle prediction (test 10.3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Donggyu Sim | Tel: Email: | +82-2-941-6470 yjahn@digitalinsights.co.kr dgsim@digitalinsights.co.kr |
| Source: | Digital Insights Inc. |  |  |

_____________________________

# Abstract

This contribution provides the simulation results of CE10.3.1. In this test, AMVP mode for triangle prediction is applied. The test 10.3.1 can achieve 0.06% and 0.07% BD-rate gain in random access and low-delay B configuration, respectively.

# Introduction

The proposed prediction method uses triangular partitions, left and right diagonal, for each coding unit (CU). The motivation of the proposed prediction method is that binary or ternary split CUs can help represent arbitrary motion partitions. It means that the binary or ternary split CUs can make rectangular shape blocks which have different block width and height compared with only using quadtree (QT) block structure. Therefore, the proposed triangle prediction unit can reduce signaling overhead with MTT split syntaxes. MERGE mode for triangle prediction was adopted in the previous meeting [1]. This contribution proposes AMVP mode for triangle prediction.

(a) Left diagonal			 (b) Right diagonal

Figure 1. Two triangle prediction in the proposed method

# Proposed method

The proposed AMVP mode for triangle prediction units needs bit overhead to signal motion information for each partition. Also, the motion search process is a huge burden on the encoder side to search both triangle prediction units. To reduce the bit overhead of motion information and complexity, the proposed method applies several constraints as below:

- Reference picture: Only using RefIdx0

- Block size: block width >= 8 && block height >= 8

First of all, AMVP mode for triangle prediction applies the constraint of reference picture selection. The proposed AMVP mode for triangle prediction uses only the first reference picture in reference picture list of both directions. It means that the triangle prediction units coded AMVP mode do not need to signal the reference picture indices.

The second constraint is related to block sizes. In MERGE mode for triangle prediction, it has applied to the constraint of block sizes. The condition of the constraint is that block size (block width*block height) is smaller than 64. In that case, the CU does not use the MERGE mode for triangle prediction unit. In this test, the condition of the second constraint is that block width or block height is smaller than 8.

# Performance evaluation

The proposed methods were evaluated under JVET common test conditions (CTC) [2]. The proposed methods were integrated on VTM-3.0. Table 1 shows BD-rate reduction of the test 10.3.1. In this test, the proposed methods can achieve 0.06% and 0.07% BD-rate reduction in random access and low-delay B configuration, respectively.

Table 1. BD-rate reduction of the test 10.3.1

|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.12% | 0.12% | 121% | 98% |  |  |  |  |  |
| Class A2 | -0.07% | -0.01% | 0.03% | 125% | 100% |  |  |  |  |  |
| Class B | -0.04% | -0.01% | -0.09% | 125% | 102% | -0.04% | -0.14% | 0.12% | 123% | 104% |
| Class C | -0.16% | -0.19% | -0.22% | 121% | 101% | -0.13% | -0.03% | -0.16% | 118% | 101% |
| Class E |  |  |  |  |  | -0.02% | -0.13% | -0.02% | 125% | 106% |
| Overall | -0.06% | -0.03% | -0.06% | 123% | 100% | -0.07% | -0.10% | -0.01% | 122% | 103% |
| Class D | -0.20% | -0.39% | -0.36% | 122% | 102% | -0.15% | 0.34% | -0.03% | 113% | 100% |

Table 2 shows the additional test result using the same constraint of block size with MERGE mode for triangle prediction units. In this test, the proposed methods can achieve 0.08% and 0.10% BD-rate reduction in random access and low-delay B configuration, respectively.

Table 2. BD-rate reduction of the additional test

|  | Over VTM-3.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.08% | 0.08% | 134% | 100% |  |  |  |  |  |
| Class A2 | -0.08% | -0.07% | -0.04% | 132% | 107% |  |  |  |  |  |
| Class B | -0.05% | -0.11% | -0.13% | 130% | 97% | -0.05% | -0.32% | -0.08% | 134% | 89% |
| Class C | -0.22% | -0.31% | -0.24% | 130% | 103% | -0.11% | -0.12% | -0.15% | 123% | 101% |
| Class E |  |  |  |  |  | -0.17% | 0.16% | -0.52% | 111% | 74% |
| Overall | -0.08% | -0.12% | -0.10% | 131% | 101% | -0.10% | -0.13% | -0.21% | 124% | 89% |
| Class D | -0.25% | -0.44% | -0.45% | 126% | 94% | -0.15% | -0.07% | 0.32% | 128% | 111% |

# References

- B. Bross, J. Chen, and S. Liu, “Versatile video coding (Draft 3),” JVET-L1001, 12th JVET meeting, Macau, CN, Oct. 2018.

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th JVET meeting, Macau, CN, Oct. 2018.

# Patent rights declaration(s)

Digital Insights Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).