| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0191 |
| --- | --- |

| Title: | CE3-related: Construction of non-MPM mode list in intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sangguk Cha Geon-Won Lee Geun-Bae Kim Jong-Ki Han | Tel: Email: | +82-10-6414-4338 sgsept29@sju.ac.kr geonwonlee@sju.ac.kr gbkim@sju.ac.kr hjk@sejong.edu |
| Source: | Sejong University |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to construct the non-MPM mode list for each coding block, where the order of modes in the non-MPM list is derived based on the angular modes which are the neighboring blocks of the current coding block. The number of modes in the non-MPM list depends on the angular modes of the neighboring blocks.

If MPM flag is false, the truncated binary code is used to signal the index of mode in the non-MPM list, where the shorter and longer codes are used to signal the modes in the top and bottom parts of the non-MPM list, respectively, when the number of modes in non-MPM list is less than 2n. In this contribution, the modes which are similar to those of the neighboring blocks are placed in the top part of the non-MPM list.

When the proposed method1 is applied, simulation results reportedly show that the average -0.14% and -0.xx% BD-rate change is achieved on top of VTM3.0 for all intra (AI) and random access (RA) configuration under the common test condition. The increments of encoding and decoding complexity are 5% and 2% for AI configuration, x.x% and x.x% for RA configuration.

When the proposed method2 is applied, the average -0.10% and -0.xx% BD-rate change is achieved on top of VTM3.0 for all intra (AI) and random access (RA) configuration. The increments of encoding and decoding complexity are 4% and 2% for AI configuration, x.x% and x.x% for RA configuration.

Introduction

In VTM3.0, Planar, DC and 93 intra angular modes are used to predict the intra prediction block as shown in Figure 1. The number of the available intra prediction modes is always 67 for each block. To encode an intra mode, a most probable mode (MPM) list or a non-MPM list are used. The MPM list is constructed using the intra prediction modes of two neighboring blocks. The size of MPM list is 6.

Figure 1. Intra prediction modes in VTM3.0

Firstly, one MPM flag is signaled to indicate whether current mode is MPM or not. Then, if MPM flag is true, truncated unary coding is used to signal the MPM index, otherwise, the truncated binary coding is used to signal the remaining modes in non-MPM list whose size is 61.

When the truncated binary code is used to signal the index of non-MPM list, if the number m of symbols in the list is 2n-1 < m < 2n, n-1 bits codewords are assigned to symbols whose indexes are 0 ~ 2n-m-1, as shown in Figure 2. For other symbols whose indexes are 2n-m ~ m-1, n bits codewords are assigned. In VTM 3.0, 61 non-MPM modes are sorted in descending order of the mode number. Note that m=61 and n=6 in VTM3.0. The modes of indexes 0, 1, 2 are encoded with 5 bits length codeword and other modes are coded with 6 bits.

Figure 2. The lengths of codewords according to the index in the non-MPM list when 2n-1<m<2n

Proposed Methods

In this contribution, the non-MPM list consists of the selected modes which are close to the angular modes of the neighboring blocks. By selecting some modes from 61 modes, the size (m) of non-MPM list decreases. It results in the increment of the number (2n-m) of modes to be encode with 5 bits. In addition, the modes that are closer to the angular modes of the neighboring blocks are assigned to the smaller indexes in the non-MPM list. The modified structure of the non-MPM reduces the generated bitrates.

Proposed Method1

Figure 3. Neighboring blocks of current coding unit

AL

A

AR

L

BL

In the proposed method1, Neighboring_Angular_Mode (NAM) is searched according to the merge candidate searching order { L  A  AR  BL  AL } until two angular modes are found. The maximum number of Neighboring_Angular_Mode is two. If there is no angular mode in the neighboring blocks, then the modes in the non-MPM list are encoded by VTM3.0 method.

When one Neighboring_Angular_Mode (NAM) is used

When one Neighboring_Angular_Mode is found, the non-MPM list is filled up as follows.

Firstly, NAM is inserted to the first index in the non-MPM list, if NAM is not included in MPM list.

Secondly, the closer modes to NAM are inserted to the smaller indexes of the non-MPM list so that the order of the considered modes is { NAM-1  NAM+1  NAM-2  NAM+2  …  NAM-D NAM+D}, where if the considered mode is included in MPM list, the mode is not inserted in the non-MPM list. D is a threshold to construct the set of the closer modes.

Thirdly, among the modes which are far from NAM, the even modes are inserted to the non-MPM list. The order of the considered modes is { NAM-D-1  NAM+D+1  NAM-D-2  NAM-D+2  … }, where only even modes are inserted to the non-MPM list if the mode is not included in MPM list.

When two Neighboring_Angular_Mode (NAM0, NAM1) are used

When two NAMs are found, the non-MPM list is filled up as follows.

Firstly, two NAMs (NAM0, NAM1) are inserted to the non-MPM list, if those are not included in MPM list.

Secondly, the closer modes to the NAMs are inserted to the smaller indexes of the non-MPM list. The order of the considered modes is { NAM0-1  NAM0+1  NAM1-1  NAM1+1  NAM0-2  NAM0+2  NAM1-2  NAM1+2  …  NAM0-D  NAM0+D  NAM1-D  NAM1+D}, where if the considered mode is included in MPM list, the mode is not inserted in the non-MPM list.

Thirdly, among the modes which are far from NAMs, the even modes are inserted to the non-MPM list. The order of the considered modes is { NAM0-D-1  NAM0+D+1  NAM1-D-1  NAM1+D+1 NAM0-D-2  NAM0+D+2  NAM1-D-2  NAM1+D+2 … }, where only even modes are inserted to the non-MPM list if the mode is not included in MPM list.

Proposed Method2

In the proposed method2, NAMs are searched in the MPM list, whereas NAMs are searched over the neighboring blocks when the proposed method1 is applied. NAMs are searched according to the increasing index of MPM list until two angular modes are found. When the first and second modes in the MPM list are either Planar or DC (i.e.: if the first is DC then the second must be Planar or vise versa), the proposed method2 is not applied. In the proposed method2, because NAMs are included in MPM list, those are not inserted in the non-MPM list. Except for the scheme to compose NAMs and the exclusion of NAMs in the non-MPM list, the algorithm to construct the non-MPM list is equal to that of the proposed method1.

- 3	Experimental results

The proposed methods are implemented on top of VTM-3.0 [1] software. The simulations are performed on Windows 10 using VTM configuration with AI and RA configurations, using the common test condition described in JVET-L1010 [2].

Table 1: Coding performance of the proposed method1 over VTM-3.0 (D=5)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.23% | -0.08% | 105% | 101% |
| Class A2 | -0.10% | -0.07% | -0.12% | 104% | 102% |
| Class B | -0.10% | -0.05% | -0.14% | 104% | 102% |
| Class C | -0.13% | -0.13% | -0.21% | 105% | 102% |
| Class E | -0.17% | -0.19% | -0.07% | 105% | 101% |
| Overall | -0.13% | -0.12% | -0.13% | 104% | 102% |
| Class D | -0.14% | -0.23% | -0.05% | 105% | 112% |
| Class F | -0.25% | -0.32% | -0.24% | 103% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2: Coding performance of the proposed method1 over VTM-3.0 (D=10)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.23% | -0.17% | 105% | 101% |
| Class A2 | -0.10% | -0.06% | -0.14% | 104% | 102% |
| Class B | -0.11% | -0.08% | -0.11% | 105% | 102% |
| Class C | -0.13% | -0.15% | -0.12% | 106% | 105% |
| Class E | -0.23% | -0.26% | -0.19% | 107% | 103% |
| Overall | -0.14% | -0.15% | -0.14% | 105% | 102% |
| Class D | -0.17% | -0.14% | -0.25% | 102% | 111% |
| Class F | -0.21% | -0.24% | -0.09% | 106% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.12% | -0.30% | -0.25% | 101% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3: Coding performance of the proposed method2 over VTM-3.0 (D=5)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.13% | -0.02% | 102% | 101% |
| Class A2 | -0.06% | -0.04% | -0.10% | 102% | 100% |
| Class B | -0.06% | -0.02% | -0.03% | 103% | 101% |
| Class C | -0.08% | -0.05% | -0.18% | 100% | 105% |
| Class E | -0.10% | -0.15% | -0.11% | 104% | 104% |
| Overall | -0.08% | -0.07% | -0.09% | 102% | 102% |
| Class D | -0.14% | -0.07% | 0.03% | 104% | 105% |
| Class F | -0.21% | -0.27% | -0.24% | 103% | 103% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 4: Coding performance of the proposed method2 over VTM-3.0 (D=10)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.13% | -0.06% | 105% | 102% |
| Class A2 | -0.06% | -0.04% | -0.07% | 104% | 101% |
| Class B | -0.06% | -0.07% | -0.08% | 104% | 101% |
| Class C | -0.09% | -0.17% | -0.11% | 103% | 104% |
| Class E | -0.16% | -0.14% | -0.08% | 105% | 105% |
| Overall | -0.10% | -0.11% | -0.08% | 104% | 102% |
| Class D | -0.14% | -0.08% | -0.13% | 104% | 109% |
| Class F | -0.19% | -0.13% | -0.12% | 104% | 107% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

- 4	Conclusion

In this contribution, it is proposed to construct the non-MPM list for each coding unit.

When the proposed method1 is applied, under AI configuration, the increments of the +5% encoding and +2% decoding run time and average -0.14% BD-rate change are reportedly shown on top of VTM3.0. Under RA configuration, the increments of the +0.xx% encoding and +0.xx% decoding run time and average -0.xx% BD-rate change are shown.

When the proposed method2 is applied, under AI configuration, the increments of the +4% encoding and +2% decoding run time and average -0.10% BD-rate change are reportedly shown on top of VTM3.0. Under RA configuration, the increments of the +0.xx% encoding and +0.xx% decoding run time and average -0.xx% BD-rate change are shown.

It is proposed to adopt method1(D=10) in VTM.

- 5	References

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of Joint Video Experts Team (JVET), JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- 6	Patent rights declaration(s)

Sejong University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).