| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0199 |
| --- | --- |

| Title: | CE5: Counter-based probability estimation (CE5.1.8) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | [e.g, Proposal, Information, Draft text, Proposed draft, Report] |  |  |
| Author(s) or Contact(s): | Kiho Choi, Yinji Piao, Seongchon-gil 34, Seocho-gu, Seoul, 06765 Korea | Tel: Email: | kiho14.choi@samsung.com yj1003.piao@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

- Abstract

This contribution presents a technology described in CE 5.1.8 that is a counter-based probability estimation scheme using two-probability update model. Testing results on the proposed method show 0.86%, 0.68%, and 0.53% BD rate savings on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB) configurations, respectively, compared to VTM3.0.

Proposed method

Two-probability update model was proposed in JVET-L0057 [1]. The probability is updated with linear combination for probability estimation that consists of only two summands corresponding N0=16 (short window) and N1=128 (long window):

P0 = (Y >> 4) + P0 – (P0 >> 4)

P1 = (Y >> 7) + P1 – ( P1>> 7)

P = (P0 + P1 + 1) >> 1

where Y = 2^15.

To start probability estimation it is necessary to initialize probability. Different from two window probability update model in JEM, only short window size model is updated until the counter reaches 31. When the counter reaches 31, long window model starts updating with initial value borrowed from short window model as shown in Figure 1. This approach reduces negative effect from wrong initialization and also reduces the complexity due to less update of long window size model.

Figure 1 Example of probability using different initialized value

The probability estimation process is as follows,

- bFastProbUpdate = counter < 31

- counter++

- MPS pass:

  - P0mps = P0 – (P0 >> 4)

  - P1mps = bFastProbUpdate ? P0mps: P1 – (P1 >> 7)

- LPS pass

  - P0lps = P0 + ((2^15 – P0) >> 4)

  - P1lps = bFastProbUpdate ? P0lps : P1 + ((2^15 – P1) >> 7)

  - If(P0 + P1 > 2^15)

    - mps = 1 – mps

    - P1lps = 2^15 – P1lps

    - P0lps = 2^15 – P0lps

- P= (P0 + P1) >> 1

Test result

Test conditions follows the description in CE5 document [2]. VTM3.0 was used for these experiments, as specified in [2][3]. The testing results are shown in Table 1. Testing results on the proposed method show 0.86%, 0.68%, and 0.53% BD rate savings on average for All Intra (AI), Random Access (RA), and Low Delayed B (LDB) configurations, respectively, compared to VTM3.0.

Table 1: Test results of proposed method in VTM3.0

|  | Over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.97% | -1.37% | -1.41% | 108% | 101% |
| Class A2 | -0.91% | -1.48% | -1.50% | 106% | 101% |
| Class B | -0.87% | -1.41% | -1.37% | 107% | 102% |
| Class C | -0.81% | -1.30% | -1.40% | 106% | 104% |
| Class E | -0.76% | -1.24% | -1.15% | 105% | 97% |
| Overall | -0.86% | -1.36% | -1.37% | 106% | 101% |
| Class D | -0.69% | -1.08% | -1.12% | 107% | 104% |
| Class F (optional) | -0.69% | -0.78% | -0.71% | 106% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.74% | -0.88% | -1.08% | 103% | 104% |
| Class A2 | -0.65% | -0.59% | -0.77% | 102% | 101% |
| Class B | -0.71% | -0.88% | -0.65% | 103% | 101% |
| Class C | -0.60% | -1.01% | -0.47% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.68% | -0.86% | -0.71% | 102% | 101% |
| Class D | -0.50% | -0.95% | -0.11% | 101% | 98% |
| Class F (optional) | -0.51% | -0.37% | -0.45% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.68% | -0.92% | 1.17% | 101% | 98% |
| Class C | -0.60% | -0.57% | 0.00% | 100% | 95% |
| Class E | -0.20% | -0.10% | 0.08% | 97% | 88% |
| Overall | -0.53% | -0.60% | 0.51% | 99% | 95% |
| Class D | -0.47% | 0.30% | 1.19% | 100% | 100% |
| Class F (optional) | -0.56% | 0.10% | -0.84% | 101% | 97% |

Conclusion

In this contribution, a technology described in CE 5.1.8 that is a counter-based probability estimation scheme using two-probability update model is proposed. Testing results show good coding performance in all scenarios. It is recommend to adopt the proposed method into the next version of test model of VVC.

Reference

[1] K. Choi, Y. Piao, M. W. Park, K. P. Choi, “CE5: Counter-based probability estimation (Test 5.1.2)”, JVET-L0057, Macao, CN, Oct. 2018

[2] H. Kirchhoffer, A. Said, “Description of Core Experiment 5 (CE5): Arithmetic Coding Engine”, JVET-L1025, Macao, CN, Oct. 2018

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Macao, CN, Oct. 2018

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard.