| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0203-v4 |
| --- | --- |

| Title: | CE3: DM-based chroma intra prediction modes (Test 3.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Narae Choi Min Woo Park Kiho Choi (Samsung) Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | narae1.choi@samsung.com m.w.park@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

In VTM-3.0, chroma intra prediction candidates are composed with four prediction modes, a direct mode (DM) and three linear model (LM) related modes. In this contribution, the four prediction modes are adaptively constructed according to the DM and the block shape. If the DM is angular mode, the candidate list contains two adjacent modes with offset ± 2 instead of the horizontal and vertical modes. The DM was derived same as VTM-3.0 which is derived from the prediction mode of the luma block corresponding to the center position of the chroma block. Chroma mode coding is aligned with BMS-2.0 for the test 3.5.a and VTM-3.0 for the test 3.5.b. The test 3.5.a results reportedly show average –0.01% luma BD-rate and –0.61%, –0.64% Cb and Cr BD-rates, respectively, compared to VTM-3.0 in All Intra configuration. The test 3.5.b results reportedly show average –0.02% luma BD-rate and –0.58%, –0.61% Cb and Cr BD-rates, respectively, compared to VTM-3.0 in All Intra configuration.

Proposed Method

This contribution proposes adaptive construction of chroma prediction candidate list. In the proposed method, four chroma intra mode candidates are constructed with a DM mode. The DM is the prediction mode of the luma block located at the center position of the chroma block as in VTM 3.0. Two conditions are used to construct the chroma candidate list: whether the DM is a non-angular mode; if the DM is a non-angular mode, width and height of current chroma block are compared.

The chroma intra mode candidate list is generated as follows. Let the chroma candidate list as a variable modeList and the modeList[0] is denoted the DM mode.

- List Generation:

modeList[0] = DM mode

If DM mode > DC_IDX

modeList[1] = (modeList[0] == PLANAR_IDX ? DC : PLANAR)

If chroma width > chroma height

modeList[2] = HOR_IDX

modeList[3] = VER_IDX

modeList[4] = 2

Else

modeList[2] = HOR_IDX

modeList[3] = VER_IDX

modeList[4] = 66

Else

modeList[1] = ((modeList[0] + 61 - 1) % 64) + 2   // DM - 2

modeList[2] = (modeList[0] % 64) + 2			// DM + 2

modeList[3] = PLANAR_IDX

modeList[4] = DC_IDX

- Chroma mode coding:

Test 3.5.a uses same coding scheme used in MDMS (multiple DM signaling), which is implemented in BMS-2.0. This scheme uses an additional flag and a context model to indicate DM-based prediction mode list. We have additionally tested 3.5.b with aligned mode signaling part as VTM-3.0. Table I shows comparison of two mode coding schemes.

Table I: Chroma mode coding for Test 3.5

| Chroma intra prediction mode | Binarization for Test 3.5.a | Binarization for Test 3.5.b (same as VTM-3.0) |
| --- | --- | --- |
| DM (modeList[0]) | 10 | 0 |
| LM | 01 | 10 |
| MDLM_L | 0000 | 1110 |
| MDLM_T | 0001 | 1111 |
| modeList[1] | 1100 | 11000 |
| modeList[2] | 1101 | 11001 |
| modeList[3] | 1110 | 11010 |
| modeList[4] | 1111 | 11011 |

Table II shows comparison among Test 3.5.a, b and VTM-3.0. The condition check for derivation process is half compare to the VTM-3.0.

Table II: Comparison of number of operations in the worst case between VTM-3.0 and Test 3.5

|  | VTM-3.0 | Test 3.5.a | Test 3.5.b |
| --- | --- | --- | --- |
| Number of luma DM point access | 1 | 1 | 1 |
| Number of chroma neighbor spatial access | 0 | 0 | 0 |
| Condition check to construct the chroma candidate list | 4 | 2 | 2 |
| Number of context models | 3 | 4 | 3 |

|  | Max number of Luma DM point to access | Max number of neighbors to access | Max layers of if conditions | Max number of comparison operator | Max number of logical operators | Max number of assignment operators | Max number of increments | Max number of bit operation | Number of Context modeling for MPM coding | number of full RDO checks | Has LUT? | LUT size |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VTM3.0 | 1 | 0 | 1 | 5 | 0 | 15 | 4 | 2 | 1 | 5 | N | 0 |
| CE3-3.3.1 | 2 | 0 | 4 | 4 | 2 | 13 | 0 | 4 | 1 | 5 | N | 0 |
| CE3-3.3.2 | 2 | 0 | 3 | 3 | 1 | 12 | 0 | 2 | 1 | 3 | N | 0 |
| CE3-3.4.1 | 2 | 0 | 5 | 5 | 3 | 15 | 0 | 2 | 1 | 6 | N | 0 |
| CE3-3.4.2 | 2 | 0 | 3 | 3 | 1 | 13 | 0 | 2 | 1 | 4 | N | 0 |
| CE3-3.5.a | 1 | 0 | 2 | 2 | 1 | 11 | 0 | 2 | 2 | 5 | N | 0 |
| CE3-3.5.b | 1 | 0 | 2 | 2 | 1 | 11 | 0 | 2 | 1 | 5 | N | 0 |

Experimental Results

The proposed DM-based chroma intra prediction modes is implemented on top of the VTM-3.0 software. The BD-rates are compared to VTM-3.0. Both the ‘All Intra’ and ‘Random Access’ test configurations were used. Table III and Table IV presents the performance of Test 3.5.a and 3.5.b, respectively, which uses different chroma coding schemes. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions.

Table III. Results for the Test 3.5.a

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.40% | -0.54% | 100% | 100% |
| Class A2 | -0.02% | -0.58% | -0.43% | 100% | 101% |
| Class B | 0.01% | -0.53% | -0.58% | 101% | 100% |
| Class C | 0.00% | -0.58% | -0.73% | 100% | 100% |
| Class E | 0.01% | -1.01% | -0.90% | 101% | 100% |
| Overall | -0.01% | -0.61% | -0.64% | 100% | 100% |
| Class D | 0.02% | -0.22% | -0.38% | 101% | 100% |
| Class F | 0.02% | -0.32% | -0.34% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.45% | -0.54% | 101% | 101% |
| Class A2 | 0.04% | -0.72% | -0.38% | 100% | 101% |
| Class B | 0.06% | -0.68% | -1.04% | 100% | 101% |
| Class C | 0.12% | -0.81% | -0.87% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | -0.68% | -0.76% | 100% | 101% |
| Class D | 0.08% | -1.16% | -0.58% | 100% | 100% |
| Class F | 0.02% | -0.35% | -0.31% | 100% | 99% |

Table IV. Results for the Test 3.5.b

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.45% | -0.39% | 100% | 101% |
| Class A2 | -0.02% | -0.50% | -0.45% | 100% | 101% |
| Class B | -0.01% | -0.47% | -0.58% | 100% | 99% |
| Class C | 0.00% | -0.65% | -0.78% | 101% | 99% |
| Class E | -0.02% | -0.90% | -0.81% | 100% | 99% |
| Overall | -0.02% | -0.58% | -0.61% | 100% | 100% |
| Class D | 0.02% | -0.44% | -0.50% | 100% | 100% |
| Class F | 0.03% | -0.32% | -0.45% | 100% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.36% | -0.20% | 101% | 100% |
| Class A2 | -0.03% | -0.36% | -0.19% | 100% | 101% |
| Class B | 0.00% | -0.26% | -0.57% | 100% | 101% |
| Class C | -0.01% | -0.54% | -0.46% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.37% | -0.39% | 100% | 100% |
| Class D | 0.03% | -0.52% | -0.47% | 99% | 99% |
| Class F | 0.03% | -0.32% | -0.28% | 100% | 100% |

Conclusion

In this contribution, it is proposed to construct chroma prediction candidate list using one DM mode. The four prediction modes are derived according to the DM and the block shape. The test 3.5.b results reportedly show average –0.02% luma BD-rate and –0.58%, –0.61% Cb and Cr BD-rates, respectively, compared to VTM-3.0 in All Intra configuration. The proposed chroma candidate list provides better chroma performance in terms of BD-rate with less condition checks compare to VTM-3.0. Therefore, it is suggested to adopt the proposed DM-based chroma intra prediction modes into the next working draft.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).