| 6Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0204 |
| --- | --- |

| Title: | CE2-related: Simplification of ATMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minsoo Park Min Woo Park Seungsoo Jeong Anish Tamse Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | mss.park@samsung.com m.w.park@samsung.com ss00.jeong@samsung.com tamse.anish@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes three methods for simplifying the temporal motion vector derivation process, which is to the temporal corresponding block for obtaining sub-block based motion vectors, in ATMVP, which is known as one of the most complex merge candidates. In VVC, ATMVP needs to access up to four neighbouring blocks to derive the temporal motion vector. To reduce the number of neighbouring block access: 1) using two neighbouring blocks (left and above), 2) using one neighbouring block (left), 3) using one neighbouring block depending on block shape (left or above). The first method reportedly shows 0.00% and 0.01% BD-Rates for RA and LDB, respectively. The second method reportedly shows 0.00% and 0.02% BD-Rates for RA and LDB, respectively. The third method reportedly shows -0.02% and 0.00% BD-rates for RA and LDB, respectively.

Introduction

In the process of ATMVP, four neighbouring blocks are used. The positions of neighbouring blocks are shown as Fig 1.

Figure 1 Position of neighbouring blocks in ATMVP

Left (A1), left-bottom (A0), above (B1), above-right (B0) are the positions. And the order of block access is shown as follows.

The variable availableFlagN is set equal to FALSE, and the following applies: - When availableFlagA1 is equal to 1, the following applies: - availableFlagN is set equal to TRUE, - refIdxLXN is set equal to refIdxLXA1 and mvLXN is set equal to mvLXA1, for X being replaced by 0 and 1. - When availableFlagN is equal to FALSE and availableFlagLB1 is equal to 1, the following applies: - availableFlagN is set equal to TRUE, - refIdxLXN is set equal to refIdxLXB1 and mvLXN is set equal to mvLXB1, for X being replaced by 0 and 1. - When availableFlagN is equal to FALSE and availableFlagB0 is equal to 1, the following applies: - availableFlagN is set equal to TRUE. - refIdxLXN is set equal to refIdxLXB0 and mvLXN is set equal to mvLXB0, for X being replaced by 0 and 1. - When availableFlagN is equal to FALSE and availableFlagA0is equal to 1, the following applies: - availableFlagN is set equal to TRUE. - refIdxLXN is set equal to refIdxLXA0 and mvLXN is set equal to mvLXA0, for X being replaced by 0 and 1.

As shown the process, the order of block access is A1B1 B0 A0 and if one block is available, the process is fished. It means up to four blocks are accessed. To reduce the number of block access in ATMVP, three methods are proposed.

Proposed Method

Method 1: Using two neighbouring blocks

In this method, two neighbouring blocks are used for ATMVP. The order of block access is A1  B1.

Figure 2 Position of neighbouring blocks in method 1

The variable availableFlagN is set equal to FALSE, and the following applies: - When availableFlagA1 is equal to 1, the following applies: - availableFlagN is set equal to TRUE, - refIdxLXN is set equal to refIdxLXA1 and mvLXN is set equal to mvLXA1, for X being replaced by 0 and 1. - When availableFlagN is equal to FALSE and availableFlagLB1 is equal to 1, the following applies: - availableFlagN is set equal to TRUE, - refIdxLXN is set equal to refIdxLXB1 and mvLXN is set equal to mvLXB1, for X being replaced by 0 and 1. - When availableFlagN is equal to FALSE and availableFlagB0 is equal to 1, the following applies: - availableFlagN is set equal to TRUE. - refIdxLXN is set equal to refIdxLXB0 and mvLXN is set equal to mvLXB0, for X being replaced by 0 and 1. - When availableFlagN is equal to FALSE and availableFlagA0is equal to 1, the following applies: - availableFlagN is set equal to TRUE. - refIdxLXN is set equal to refIdxLXA0 and mvLXN is set equal to mvLXA0, for X being replaced by 0 and 1.

Method 2: Using one neighbouring block

In this method, only left block (A1) is used for ATMVP.

Figure 3 Position of neighbouring block in method 2

The variable availableFlagN is set equal to FALSE, and the following applies: - When availableFlagA1 is equal to 1, the following applies: - availableFlagN is set equal to TRUE, - refIdxLXN is set equal to refIdxLXA1 and mvLXN is set equal to mvLXA1, for X being replaced by 0 and 1. - When availableFlagN is equal to FALSE and availableFlagLB1 is equal to 1, the following applies: - availableFlagN is set equal to TRUE, - refIdxLXN is set equal to refIdxLXB1 and mvLXN is set equal to mvLXB1, for X being replaced by 0 and 1. - When availableFlagN is equal to FALSE and availableFlagB0 is equal to 1, the following applies: - availableFlagN is set equal to TRUE. - refIdxLXN is set equal to refIdxLXB0 and mvLXN is set equal to mvLXB0, for X being replaced by 0 and 1. - When availableFlagN is equal to FALSE and availableFlagA0is equal to 1, the following applies: - availableFlagN is set equal to TRUE. - refIdxLXN is set equal to refIdxLXA0 and mvLXN is set equal to mvLXA0, for X being replaced by 0 and 1.

Method 3: Using one neighbouring block depending on block shape

One neighbouring block depending on block shape is used for ATMVP. If width of current CU is less than or equal to height of current CU, left neighbouring block (A1) is used. Otherwise, above neighbouring block (B1) is used.

a. Width = Height               b. Width < Height           c. Width < Height

Figure 4 Position of neighbouring block in method 3

The variable availableFlagN is set equal to FALSE, and the following applies: - If cbWidth is less than or equal to cbHeight and When availableFlagA1 is equal to 1, the following applies: - availableFlagN is set equal to TRUE, - refIdxLXN is set equal to refIdxLXA1 and mvLXN is set equal to mvLXA1, for X being replaced by 0 and 1. - Otherwise, (cbHeight is greather than cbWidth) When availableFlagN is equal to FALSE and availableFlagLB1 is equal to 1, the following applies: - availableFlagN is set equal to TRUE, - refIdxLXN is set equal to refIdxLXB1 and mvLXN is set equal to mvLXB1, for X being replaced by 0 and 1. - When availableFlagN is equal to FALSE and availableFlagB0 is equal to 1, the following applies: - availableFlagN is set equal to TRUE. - refIdxLXN is set equal to refIdxLXB0 and mvLXN is set equal to mvLXB0, for X being replaced by 0 and 1. - When availableFlagN is equal to FALSE and availableFlagA0is equal to 1, the following applies: - availableFlagN is set equal to TRUE. - refIdxLXN is set equal to refIdxLXA0 and mvLXN is set equal to mvLXA0, for X being replaced by 0 and 1.

Experimental Results

The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 7.3 and the AVX2 SIMD instructions. Table 1 shows summary of all results.

Table 1 Summary of all results

|  | Test | BD-Rate (Y) | BD-Rate (U) | BD-Rate (V) | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| RA | Method 1 | 0.00% | 0.01% | 0.03% | 100% | 100% |
|  | Method 2 | 0.00% | -0.01% | 0.01% | 100% | 100% |
|  | Method 3 | -0.02% | -0.01% | -0.04% | 100% | 100% |
| LD | Method 1 | 0.01% | -0.01% | 0.02% | 99% | 99% |
|  | Method 2 | 0.02% | -0.09% | 0.10% | 100% | 100% |
|  | Method 3 | 0.00% | 0.00% | -0.04% | 99% | 99% |

Results for Method 1(Using two neighbouring blocks)

Table 2 results of method 1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.02% | 99% | 100% |
| Class A2 | -0.01% | 0.06% | 0.09% | 99% | 100% |
| Class B | 0.00% | 0.02% | 0.02% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.03% | 100% | 100% |
| Class D | 0.01% | 0.02% | 0.02% | 101% | 101% |
| Class F | 0.00% | 0.01% | 0.00% | 100% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.25% | 0.11% | 98% | 99% |
| Class C | 0.01% | 0.26% | 0.05% | 100% | 99% |
| Class E | 0.02% | 0.04% | -0.16% | 101% | 99% |
| Overall | 0.01% | -0.01% | 0.02% | 99% | 99% |
| Class D | 0.02% | 0.13% | -0.04% | 100% | 98% |
| Class F | -0.01% | 0.09% | -0.51% | 100% | 94% |

Results for method 2 (Using one neighbouring block)

Table 3 results of method 2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.03% | 99% | 99% |
| Class A2 | 0.00% | 0.04% | 0.04% | 99% | 100% |
| Class B | 0.00% | -0.03% | -0.01% | 100% | 99% |
| Class C | 0.02% | -0.01% | -0.01% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.04% | 0.03% | 100% | 100% |
| Class F | -0.01% | -0.02% | -0.01% | 99% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.22% | 0.13% | 98% | 100% |
| Class C | -0.01% | 0.18% | 0.26% | 101% | 100% |
| Class E | 0.06% | -0.25% | -0.16% | 100% | 99% |
| Overall | 0.02% | -0.09% | 0.10% | 100% | 100% |
| Class D | -0.02% | 0.00% | -0.10% | 100% | 102% |
| Class F | -0.01% | -0.03% | -0.49% | 100% | 96% |

Results for method 3 (Using one neighbouring block depending on block shape)

Table 4 results of method 3

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.03% | -0.04% | 100% | 99% |
| Class A2 | -0.03% | -0.05% | 0.03% | 100% | 100% |
| Class B | -0.02% | 0.07% | -0.09% | 100% | 99% |
| Class C | 0.00% | -0.04% | -0.01% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.01% | -0.04% | 100% | 100% |
| Class D | 0.01% | -0.08% | 0.06% | 100% | 101% |
| Class F | -0.03% | -0.04% | -0.08% | 100% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.17% | -0.10% | 98% | 99% |
| Class C | 0.03% | 0.05% | 0.05% | 101% | 100% |
| Class E | -0.03% | 0.20% | -0.07% | 101% | 99% |
| Overall | 0.00% | 0.00% | -0.04% | 99% | 99% |
| Class D | 0.01% | 0.04% | 0.21% | 100% | 98% |
| Class F | 0.14% | 0.01% | 0.34% | 100% | 96% |

Conclusion

In this contribution, simplification methods for ATMVP by reducing the number of block access are proposed. All the proposed methods show no coding performance change. Therefore, it is suggested to adopt proposed methods into the next VVC working draft text.

References

[1] “Versatile Video Coding (Draft 3),” JVET-L1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).