The syntax specification for proposed method is illustrated in bellowing Table with modification highlighted in yellow.

- 7.3.4.6 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] |  |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit |  |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| intra_luma_non_ang_flag[x0][y0] | ae(v) |
| if( intra_luma_non_ang_flag == 0 ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| intra_luma_planar_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| … |  |
| } |  |
| … |  |
| } |  |

- 7.4.5.6 Coding unit semantics

pcm_flag[ x0 ][ y0 ] equal to 1 specifies that the pcm_sample( ) syntax structure is present and the transform_tree( ) syntax structure is not present in the coding unit including the luma coding block at the location ( x0, y0 ). pcm_flag[ x0 ][ y0 ] equal to 0 specifies that pcm_sample( ) syntax structure is not present. When pcm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

The value of pcm_flag[ x0 + i ][ y0 + j ] with i = 1..cbWidth − 1, j = 1..cbHeight − 1 is inferred to be equal to pcm_flag[ x0 ][ y0 ].

pcm_alignment_zero_bit is a bit equal to 0.

intra_luma_non_ang_flag[ x0 ][ y0 ] equal to  1 specifies that the intra prediction mode for luma samples is not angular. intra_luma_non_ang_flag[ x0 ][ y0 ] equal to 0 specifies that the intra prediction mode for luma samples is angular. When intra_luma_non_ang_flag[ x0 ][ y0 ] is not present it is inferred to be equal to 1.

When intra_luma_non_ang_flag[ x0 ][ y0 ] is equal to 1, intra_luma_planar_flag[ x0 ][ y0 ] specify intra prediction mode for luma samples is PLANAR or DC. intra_luma_planar_flag[ x0 ][ y0 ] equal to 1 specifies that the intra prediction mode for luma samples is PLANAR. intra_luma_planar_flag[ x0 ][ y0 ] equal to  0 specifies that the intra prediction mode for luma samples is DC. When intra_luma_planar_flag[ x0 ][ y0 ] is not present it is inferred to be equal to 1.

When intra_luma_non_ang_flag[ x0 ][ y0 ] is equal to 0 and intra_luma_mpm_flag[ x0 ][ y0 ] is equal to 1, intra_luma_ref_idx[ x0 ][ y0 ] specifies the intra prediction reference line index IntraLumaRefLineIdx[ x ][ y ] for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1 as specified in Table 7-6.

When intra_luma_ref_idx[ x0 ][ y0 ] is not present it is inferred to be equal to 0.

Table 7-6 – Specification of IntraLumaRefLineIdx[ x ][ y ] based on intra_luma_ref_idx[ x0 ][ y0 ].

| intra_luma_ref_idx[ x0 ][ y0 ] | IntraLumaRefLineIdx[ x ][ y ] x = x0..x0 + cbWidth − 1 y = y0..y0 + cbHeight − 1 |
| --- | --- |
| 0 | 0 |
| 1 | 1 |
| 2 | 3 |

When intra_luma_non_ang_flag[ x0 ][ y0 ] is equal to 0, the syntax elements intra_luma_mpm_flag[ x0 ][ y0 ], intra_luma_mpm_idx[ x0 ][ y0 ] and intra_luma_mpm_remainder[ x0 ][ y0 ] specify the angular intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0 , y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When intra_luma_mpm_flag[ x0 ][ y0 ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted coding unit according to clause 8.2.2.

When intra_luma_mpm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 1.

intra_chroma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode for chroma samples. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

### 8.2.2 Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and mh_intra_flag[ xNbX ][ yNbX ] is not equal to 1.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..5 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-4)

candModeList[ 1 ] = INTRA_PLANAR	(8-5)

candModeList[ 2 ] = INTRA_DC		(8-6)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-7)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-8)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-9)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-10)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-11)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-12)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-13)

candModeList[ 4 ] = 2 + ( candIntraPredModeA % 64 )	(8-14)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 59 ) % 64 )	(8-15)

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-16)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-17)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-18)

candModeList[ 1 ] = candIntraPredModeB	(8-19)

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 2 ] = INTRA_PLANAR	(8-20)

candModeList[ 3 ] = INTRA_DC		(8-21)

  - If maxAB − minAB is in the range of 2 to 62, inclusive, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-22)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-23)

  - Otherwise, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-24)

candModeList[ 5 ] = 2 + ( ( maxAB ) % 64 )	(8-25)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

  - If maxAB − minAB is equal to 1, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-26)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-27)

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-28)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-29)

  - Otherwise if maxAB − minAB is equal to 2, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-30)

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-31)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-32)

candModeList[ 5 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-33)

  - Otherwise if maxAB − minAB is greater than 61, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-34)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-35)

candModeList[ 4 ] = 2 + ( minAB % 64 )	(8-36)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-37)

  - Otherwise, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-38)

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-39)

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-40)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-41)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-42)

candModeList[ 1 ] = candIntraPredModeB	(8-43)

candModeList[ 2 ] = 1 − minAB		(8-44)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-45)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-46)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-47)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = maxAB		(8-48)

candModeList[ 1 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-49)

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-50)

candModeList[ 3 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-51)

candModeList[ 4 ] = 2 + ( maxAB % 64 )	(8-52)

candModeList[ 5 ] = 2 + ( ( maxAB + 59 ) % 64 )	(8-53)

- Otherwise, the following applies:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-54)

candModeList[ 1 ] = ( candModeList[0]  = =  INTRA_PLANAR )  ?  INTRA_DC  :  	(8-55) 
		INTRA_PLANAR

candModeList[ 2 ] = INTRA_ANGULAR50	(8-56)

candModeList[ 3 ] = INTRA_ANGULAR18	(8-57)

candModeList[ 4 ] = INTRA_ANGULAR46	(8-58)

candModeList[ 5 ] = INTRA_ANGULAR54	(8-59)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = INTRA_ANGULAR50	(8-60)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-61)

candModeList[ 2 ] = INTRA_ANGULAR2	(8-62)

candModeList[ 3 ] = INTRA_ANGULAR34	(8-63)

candModeList[ 4 ] = INTRA_ANGULAR66	(8-64)

candModeList[ 5 ] = INTRA_ANGULAR26	(8-65)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candModeList[ i ] is greater than candModeList[ j ] for i = 0..4 and for each i, j = ( i + 1 )..5, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-66)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to ( intra_luma_mpm_remainder[ xCb ][ yCb ] + 2 ).

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

| Table 9-4 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| coding_unit( ) | cu_skip_flag[ ][ ] | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_ref_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | intra_luma_non_ang_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_planar_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_mpm_idx[ ][ ] | TR | cMax = 5, cRiceParam = 0 |
|  | intra_luma_mpm_remainder[ ][ ] | TB | cMax = 60 58 |
|  | intra_chroma_pred_mode[ ][ ] | 9.5.3.7 | - |
|  | merge_flag[ ][ ] | FL | cMax = 1 |
|  | inter_pred_idc[ x0 ][ y0 ] | 9.5.3.8 | cbWidth, cbHeight |
| merge_data( ) | mmvd_flag[ ][ ] | FL | cMax = 1 |
|  | mmvd_merge_flag[ ][ ] | FL | cMax = 1 |
|  | mmvd_distance_idx[ ][ ] | TR | cMax = 7, cRiceParam = 0 |
|  | mmvd_direction_idx[ ][ ] | FL | cMax = 3 |
|  | mh_intra_flag[ ][ ] | FL | cMax = 1 |
|  | mh_intra_luma_vert_flag[ ][ ] | FL | cMax = 1 |
|  | mh_intra_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | mh_intra_luma_mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | merge_subblock_flag[ ][ ] | FL | cMax = 1 |
|  | merge_subblock_idx[ ][ ] | TR | cMax = MaxNumSubblockMergeCand − 1, cRiceParam = 0 |

| Table 9-10 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_slice_flag | terminate | na | na | na | na | na |
| alf_ctb_flag[ ][ ][ ] | 0..8 (clause 9.5.4.2.2) | na | na | na | na | na |
| sao_merge_left_flag | 0 | na | na | na | na | na |
| sao_merge_up_flag | 0 | na | na | na | na | na |
| sao_type_idx_luma | 0 | bypass | na | na | na | na |
| sao_type_idx_chroma | 0 | bypass | na | na | na | na |
| .. | .. | .. | .. | .. | .. | .. |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| intra_luma_ref_idx[ ][ ] | 0 | 1 | 2 | na | na | na |
| intra_luma_non_ang_flag[ ][ ] | 0 | na | na | na | na | na |
| intra_luma_planar_flag[ ][ ] | 0 | na | na | na | na | na |
| intra_luma_mpm_flag[ ][ ] | 0 | na | na | na | na | na |
| intra_luma_mpm_idx[ ][ ] | Bypass | bypass | bypass | bypass | bypass | na |
| intra_luma_mpm_remainder[ ][ ] | Bypass | bypass | bypass | bypass | bypass | bypass |
| .. | .. | .. | .. | .. | .. | .. |
| merge_idx[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| mh_intra_flag[ ][ ] | 0 | na | na | na | na | na |
| mh_intra_luma_vert_flag[ ][ ] | bypass | na | na | na | na | na |
| mh_intra_luma_mpm_flag[ ][ ] | 0 | na | na | na | na | na |
| mh_intra_luma_mpm_idx[ ][ ] | Bypass | bypass | na | na | na | na |
| … | … | … | … | … | … | … |