| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0210_v2 |
| --- | --- |

| Title: | Non-CE3: Intra prediction information coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Yao, Jianqing Zhu, Wenting Cai, Kimihiko Kazui 601,Yongda International Tower, 2277 Longyang Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5809 yaojie@cn.fujitsu.com zhujianqing@cn.fujitsu.com caiwenting@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

This contribution proposes a new Intra prediction information coding method in which a syntax element,  intra_luma_non_ang_flag, is signaled at first for intra block to determine whether IntraMode is angular or non angular. For non_angular mode, intra_luma_planar_flag is signaled to indicate whether IntraMode is planar or DC. For angular mode, syntax elements intra_luma_ref_idx, intra_luma_mpm_flag, intra_luma_mpm_idx and intra_luma_mpm_remainder are signaled as in VTM-3.0. The mpm list is only applied for angular mode thus simplify MPM list derivation process. The experimental results reportedly show AI 0.3%, RA 0.12% bitrate savings with AI 109%/100%, RA 106%/100% encoding/decoding time over VTM-3.0.

Introduction

Multiple reference line (MRL) intra prediction was adopt in VTM-3.0[1] which proposed to use more reference lines for intra prediction[2]. The index of selected reference line (intra_luma_ref_idx) is signaled to generate intra predictor. Figure 1 shows the intra prediction information coding process. The intra_luma_ref_idx is signaled before intra prediction modes. If reference line index is non-zero, Planar and DC modes are excluded from intra prediction modes and only include additional reference line modes in MPM list and only signal mpm index (intra_luma_mpm_idx) without remainder. Otherwise signal intra_luma_mpm_flag, intra_luma_mpm_idx and intra_luma_mpm_remainder.

parse intra_luma_ref_idx

if (intra_luma_ref_idx > 0)

parse intra_luma_mpm_idx

IntraMode = mpm1[intra_luma_mpm_idx] // MPM without Planar&DC

else

parse intra_luma_mpm_flag

if (intra_luma_mpm_flag)

parse intra_luma_mpm_idx

IntraMode = mpm2[intra_luma_mpm_idx] // MPM with Planar&DC

else

Parse intra_luma_mpm_remainder

IntraMode = intra_luma_mpm_remainder

Figure 1. intra prediction information coding process in VTM-3.0

In this coding process intra_luma_ref_idx is always signaled, even for Planar and DC which are excluded from multiple reference line intra prediction. Also two different MPM list derivation schemes (with or without Planar&DC) are applied.

This contribution proposes a new Intra prediction information coding process to solve above problems.

Proposed method

A flag, intra_luma_non_ang_flag, is proposed which indicates whether IntraMode is angular or non angular. The coding process is illustrated in Figure2.

The intra_luma_non_ang_flag is signaled at first for intra block and can take value of 0 or 1.

If intra_luma_non_ang_flag is equal to 1, it indicates IntraMode is non_angular mode. Then a intra_luma_planar_flag is signaled to indicates whether IntraMode is planar or DC. If intra_luma_planar_flag is equal to 1, it indicates IntraMode is Planar mode. Otherwise, it indicates IntraMode is DC mode.

Otherwise (if intra_luma_non_ang_flag is equal to 0), it indicates IntraMode is angular mode. Then signal intra_luma_mpm_flag. If intra_luma_mpm_flag is equal to 1, it indicates IntraMode is in MPM list. Then further signal intra_luma_mpm_idx and intra_luma_ref_idx. Else if intra_luma_mpm_flag is equal to 0, signal intra_luma_mpm_remainder. In this condition, only one MPM list(without Planar&DC) derivation process is applied since Planar and DC are excluded.

The intra_luma_non_ang_flag syntax is only applied on Luma channel. Both intra_luma_non_ang_flag and intra_luma_planar_flag are context-adaptive arithmetic entropy-coded.

parse intra_luma_non_ang_flag

if(intra_luma_non_ang_flag == 1)

parse intra_luma_planar_flag

IntraMode = (intra_luma_planar_flag == 1) ? Planar : DC

else

parse intra_luma_mpm_flag

if (intra_luma_mpm_flag)

parse intra_luma_mpm_idx, intra_luma_ref_idx

IntraMode = mpm1[intra_luma_mpm_idx] // MPM without Planar&DC

else

Parse intra_luma_mpm_remainder

IntraMode = intra_luma_mpm_remainder

Figure 2. intra prediction information coding process for proposed method

Add non angular modes into RD check list

In VTM-3.0, modes from neighbour CUs will be added into RD check list which is embedded in MPM list derivation process. In the proposed method, since the MPM list derivation process comprising only angular modes, we propose to simply add non angular modes(Planar&DC) into RD check list which can achieve gains with reasonable increase on encoding time.

Syntax specification for proposed method

The syntax specification for proposed method is illustrated in bellowing Table with modification highlighted in yellow.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] |  |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit |  |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| intra_luma_non_ang_flag[x0][y0] | ae(v) |
| if( intra_luma_non_ang_flag == 0 ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| intra_luma_planar_flag[x0][y0] | ae(v) |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| … |  |
| } |  |
| … |  |
| } |  |

Experimental Results

The proposed method is implemented on top of VTM-3.0, and evaluated in VTM configuration according to the common test conditions [3]. Three tests are reported with different number of MPM candidates added to RD-check list.

- Test 1: Proposed intra prediction information coding method while RD check list is same as that in VTM-3.0.

- Test 2: Proposed intra prediction information coding method while Planar and DC are always added to the Rd Check list.

- Test 3: VTM3.0 intra prediction information coding method while Planar and DC are always added to the Rd Check list.

Test 1

Anchor is VTM-3.0. Test is the proposed method while RD-check is same as that in VTM-3.0.

Table 1. Experiment results of Test 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.28% | -0.15% | 99% | 100% |
| Class A2 | 0.00% | 0.03% | 0.00% | 98% | 100% |
| Class B | -0.02% | -0.06% | -0.02% | 104% | 100% |
| Class C | 0.09% | 0.08% | -0.06% | 106% | 101% |
| Class E | 0.13% | 0.05% | 0.17% | 106% | 100% |
| Overall | 0.00% | -0.03% | -0.01% | 103% | 100% |
| Class D | 0.11% | 0.18% | 0.07% | 109% | 100% |
| Class F | 0.28% | 0.17% | 0.29% | 122% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.17% | 0.04% | 104% | 99% |
| Class A2 | 0.00% | 0.25% | 0.20% | 103% | 99% |
| Class B | -0.02% | 0.31% | -0.05% | 102% | 99% |
| Class C | 0.02% | -0.23% | 0.15% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.06% | 0.07% | 103% | 99% |
| Class D | 0.04% | -0.10% | -0.01% | 103% | 100% |
| Class F | 0.11% | 0.12% | 0.20% | 119% | 102% |

Test 2

Anchor is VTM-3.0. Test is the proposed method while Planar and DC are always added to the Rd Check list.

Table 2. Experiment results of Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.53% | -0.52% | -0.36% | 105% | 100% |
| Class A2 | -0.24% | -0.14% | -0.16% | 105% | 100% |
| Class B | -0.30% | -0.17% | -0.18% | 110% | 100% |
| Class C | -0.23% | -0.15% | -0.23% | 111% | 100% |
| Class E | -0.22% | 0.02% | 0.04% | 114% | 100% |
| Overall | -0.30% | -0.19% | -0.18% | 109% | 100% |
| Class D | -0.22% | -0.13% | 0.06% | 112% | 100% |
| Class F | -0.02% | 0.03% | 0.10% | 132% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.26% | -0.17% | 104% | 100% |
| Class A2 | -0.11% | 0.03% | 0.05% | 102% | 99% |
| Class B | -0.14% | 0.04% | -0.12% | 104% | 99% |
| Class C | -0.08% | -0.25% | -0.02% | 114% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.10% | -0.07% | 106% | 100% |
| Class D | -0.09% | -0.59% | -0.14% | 156% | 96% |
| Class F | 0.01% | -0.03% | 0.03% | 156% | 100% |

Test 3

Anchor is VTM-3.0. Test is VTM-3.0 intra prediction information coding method while Planar and DC are always added to the Rd Check list.

Table 3. Experiment results of Test 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.31% | -0.24% | -0.18% | 108% | 100% |
| Class A2 | -0.23% | -0.13% | -0.15% | 108% | 100% |
| Class B | -0.25% | -0.14% | -0.07% | 111% | 100% |
| Class C | -0.25% | -0.13% | -0.23% | 112% | 100% |
| Class E | -0.31% | -0.16% | -0.11% | 116% | 100% |
| Overall | -0.26% | -0.16% | -0.14% | 111% | 100% |
| Class D | -0.23% | -0.05% | -0.05% | 105% | 99% |
| Class F | -0.28% | -0.17% | -0.08% | 137% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.15% | -0.04% | 105% | 100% |
| Class A2 | -0.11% | 0.04% | 0.01% | 103% | 100% |
| Class B | -0.11% | -0.06% | -0.09% | 105% | 100% |
| Class C | -0.08% | -0.06% | -0.04% | 110% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.06% | -0.04% | 106% | 100% |
| Class D | -0.11% | -0.55% | 0.01% | 118% | 96% |
| Class F | -0.16% | -0.14% | -0.02% | 135% | 101% |

Conclusions

This contribution proposes a new Intra prediction information coding method in which intra_luma_ref_idx coding is skipped when IntraMode is Planar or DC and the number of MPM list derivation schemes is reduced. Thus save coding bits and simplify the coding process. The experimental results report AI 0.00%(0.23% for Class A1), RA 0.01% bitrate savings based on the same number of RD-checks as VTM-3.0 and AI 0.3%, RA 0.12% bitrate savings without increase of decoding running time based on number of RD-checks modified. It is proposed to adopt this proposed method with RD-checks modified as tested in Test 2.

References

- J. Chen, Y. Ye and S.H. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3),” JVET-L1002, Oct 2018.

- B. Bross, L. Zhao, Y.J. Chang, P.H Lin and etc, “CE3: Multiple reference line intra prediction (Test 1.1.1, 1.1.2, 1.1.3 and 1.1.4),” JVET-L0283, Oct 2018.

- F. Bossen, J. Boyce, K. Suhring , X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-L1010, Oct 2018.

Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).