| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0223-v2 |
| --- | --- |

| Title: | Non-CE9: Co-existence analysis for DMVR with BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sriram Sethuraman | Email: | sriram.sethuraman@ittiam.com |
| Source: | Ittiam Systems Pvt. Ltd. |  |  |

_____________________________

# Abstract

BDOF was adopted into VTM at the last meeting and offered 1.17% luma average BDRATE gain over VTM3.0 and smaller gains in chroma as it does not apply to chroma. DMVR which relies on symmetric bilateral matching of the bi-predictive patches is shown to provide an additional 1.13% luma average BDRATE gain (with similar gains in chroma) over VTM3.0 in the CE9.2 results. Hence, these two decoder-side inter prediction refinement tools can offer a total of 2.30% average BDRATE gain with higher gains for UHD content. The method proposed in CE9.2 is one architecture for co-existence of DMVR and BDOF where final MC has a dependency on DMVR, and BDOF has a dependency on final MC samples which create timing issues for hardware designs.

In this proposal, an alternative architecture is proposed for co-existence of DMVR and BDOF wherein the optical flow vectors are solved using the same interpolation performed for DMVR. However, the final correction is applied using these computed optical flow vectors and the gradients computed from final MC samples, with only the non-available boundary sample gradients coming from the interpolated samples used for refinement. This improves concurrency between flow computation and final MC and in conjunction with DMVR improvements suggested in JVET-M0148 can improve the overall timing for bi-prediction involving both DMVR and BDOF. However, this new architecture needs to forego the error surface based sub-pixel delta-MV for performing the final MC for luma. With flow vector calculations using slightly different interpolated samples, this method results in a 0.15% drop in BDRATE, implying that the combination of DMVR and BDOF still offer an average BDRATE gain of -2.15%, -1.50%, -1.62% over VTM3.0. This proposal also mandates a normative sub-PU size at which BDOF applies boundary sample and sample gradient handling is performed even when a CU does not perform DMVR in order to keep the internal memory needs and MC compute needs at the same level as with the case when BDOF is applied to CUs that employ DMVR.

# Introduction

Even after the addition of several coding tools into merge-mode (such as MMVD, inter-intra multi-hypothesis prediction, and affine improvements) and with BDOF enabled also, CE9.2.1 results indicate that DMVR can offer an average luma BDRATE gain of 1.13% with similar gains in chroma as well. CE9.2.1 base implementation provides a straight-forward method for co-existence of DMVR with BDOF, where DMVR is first performed on sub-PUs of a CU. Based on the sub-pixel accurate refinement delta-MV for the sub-PU, final motion compensation is performed. For each 4x4 block of luma samples, BDOF is applied if the CU-mode supports BDOF. It is possible to have a row-level pipeline between interpolation for refinement and the cost evaluations. Methods to simplify the interpolation for refinement and also to facilitate cost evaluations at all refinement positions in parallel without increasing the gate count are proposed in JVET-M0148. With these, timing needs of DMVR can be shrunk considerably while keeping the gate count reasonable. However, final MC for the sub-PU can start only after the sub-pixel accurate delta-MV is available from DMVR. MC itself has the lead time to produce up to 8 horizontally interpolated rows before one vertically interpolated output can be produced for the case of the refined motion vector having both horizontal and vertical offsets. Thus, the 4x4 level operations required for computing a BDOF vector for a 4x4 block of luma samples will have to wait till the final MC output samples start becoming available. Thus, the overall latency for a CU involving DMVR, MC, and BDOF can force the processing to be pipelined across sub-PUs wherein while DMVR works on a given sub-PU, the MC and BDOF can happen for a previous sub-PU. Such a sub-PU level pipeline will double the internal memory requirements that are incurred to avoid accessing a reference pre-fetch cache more than once for the same set of integer grid samples. Also, the current sub-PU DMVR increases the average decoding time by forcing the final MC and BDOF to happen at a sub-PU level.

In this proposal, an alternate method is proposed that can further improve the timing by allowing the flow vector computations to start even before the final MC output samples become available. Secondly, by avoiding the sub-pixel accurate delta-MV from being used for luma, the refinements from DMVR across sub-PUs stay on the same integer-distance grid of samples. Hence, software and hardware implementations have the option of performing MC without waiting for the delta-MVs to be known (at the expense of some additional work). In section 2, the proposed new architecture for co-existence of DMVR with BDOF is presented. The BDRATE impact with the proposed changes is shown in Section 3.

# Proposed Implementation

The primary change proposed is to perform the flow vector calculations using the same interpolation that is employed for DMVR. In other words, the sample differences and sample gradient sums in the horizontal and vertical directions that are required for computing the flow vector for a 4x4 block of samples using up to 8x8 block of samples in L0 and L1 are computed using interpolated samples derived using bilinear interpolation rather than 2-D separable DCTIF. In order to re-use the same interpolated samples, the sub-pixel accurate parametric error surface based delta-MV from DMVR is not used in this case for luma final MC. However, this delta-MV is used to update the refined MV for the sub-PU and also for chroma MC.

The advantages of the proposed changes are the following:

- The flow vector computation steps can start without waiting for the DCTIF based outputs to become available (which can happen only after 8 rows of horizontal interpolation followed by one vertical interpolation is completed in CUs having fractional MV offsets in both directions).

- The boundary gradient computation overheads and boundary sample handling overheads in BDOF are replaced by a regular bilinear interpolation performed for both DMVR and BDOF at the CU level where the operations can be performed well in SIMD when compared to working on one column of left or right boundary samples.

- The final MC can be performed at a CU level producing CU_w + 4) x (CU_h+4) samples from which the appropriate sub-PU MC data can be extracted as soon as the integer distance delta-MV from DMVR is known. In software implementations, this reduces the average decoding time compared to performing MC at sub-PU level.

- The use of parametric error surface based sub-pixel delta-MV to update the refined MV for the sub-PU benefits SbTMVP/TMVP. Also, since BDOF does not provide any benefit currently to chroma, the use of this refined MV for chroma final MC also improves chroma BDRATEs.

However, the final flow vector and gradient based correction is still computed using the final MC output sample gradients and their differences. Only the non-available boundary gradients based on final MC output samples are obtained from the gradients computed using bilinear interpolated samples.

With the improved concurrency between interpolation for refinement and cost evaluations for DMVR offered by JVET-M0148, and the improved concurrency between optical flow vector estimation and priming the final MC output with horizontal interpolations, the overall timing for inter bi-prediction involving DMVR, MC, and BDOF is improved.

The BDRATE impact of foregoing the parametric error surface based sub-pixel accurate delta-MV for luma and the impact of using bilinear interpolated sample values and sample gradient values for flow vector calculations is shown in the next section.

DMVR by itself requires only 5 line buffers of (sub-PU-w + 4) for each reference to perform the cost calculations for the different vertical delta-MV values within the refinement range of +/-2. As each line buffer is produced, sample differences and sample gradient sums can be computed for the different 4x4s that require the interpolated samples. However, the accumulation for a given 4x4 block can happen only after knowing the integer distance delta-MV from DMVR for a sub-PU. This would necessitate either buffering the entire refinement range of interpolated samples in internal memory or buffering up the sample differences and sample gradient sums in internal memory till accumulation can happen. In addition, the boundary gradient differences (horizontal gradient differences at the left and right sub-PU boundary and the vertical gradient differences at the top and bottom sub-PU boundary) will need to be kept in internal memory till these can be utilized for computing the correction term. However, these internal memory increases will be smaller than the memory increase if a sub-PU pipeline between DMVR and MC/BDOF is set up.

Also, in the new architecture, from the use of boundary gradients at a sub-PU level, a sub-PU size is made normative even in CUs that do not employ DMVR. In this proposal, the sub-PU size determination is kept the same as that of the sub-PU size determination for DMVR.

# Results

In this section, the proposed changes are made to CE9.2.1(g) configuration and the impact on BDRATE is measured against VTM3.0.

Compared to the CE9.2.1(g) result of -1.13%, -1.33%, -1.44%, the proposed changes result in an average BDRATE drop of 0.15%, 0.14%, 0.14% due to not using the sub-pixel accurate delta-MV from parametric error surface for luma final MC and due to using bilinear MC for obtaining the optical flow estimate for each 4x4 block of samples.

Due to implementation still being at a sub-PU level and due to the increased requirement of performing bilinear interpolation for MMVD and inter coded CUs, the encoding and decoding time ratios have increased in these results. A more optimal CU-level MC and BDOF product-term calculations is underway with which the EncT ratio shall become comparable with that of CE9.2.1g while the DecT ratio is expected to be around 111% with this architecture.

# Conclusion

To improve the timing for the co-existence of DMVR with BDOF for a given CU, an alternative approach of using the same interpolation performed for DMVR to be used for computing the 4x4 level optical flow vector also. The corrections are performed using a combination of the final MC and the MC done for refinement. The concurrency is significantly improved. However, due to the disabling of the sub-pixel level delta-MV obtained from 2-D parametric error surface in DMVR for the final MC of luma, a small drop in average BDRATE (of 0.15%, 0.14%, 0.14%) is observed with this architecture.

# Patent rights declaration(s)

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).