| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0225-v2 |
| --- | --- |

| Title: | AHG8: On wrap around motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Weiwei Feng Shan Liu | Tel: Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes the following changes on the wrap-around motion compensation:

- JVET-M0451 proposes no_ref_wraparound_constraint_flag. JVET-M0101 proposes general_non_packed_constraint_flag. It is proposed in this contribution that sps_ref_wraparound_enabled_flag shall be equal to 0, when general_non_packed_contraint_flag is equal to 1 as well as no_ref_wraparound_constraint_flag is equal to 1.

2.  The range of sps_ref_wraparound_offset is restricted. sps_ref_wraparound_offset shall be in the range of pic_width_in_luma_samples / 2 to pic_width_in_luma_samples, inclusive.

# Problem statement

The horizontal wrap-around motion compensation (JVET-L0231) was adopted at the Macau meeting. The wrap-around motion compensation can reduce some visual artifact at the seam line in which the left boundary and the right one meet for 360 rendering. It has a marginal BD-rate saving (-0.22%), only when the padded equirectangular projection (PERP) is used. There is no report to provide a positive RD gain with normal video sequences. If the wrap-around motion compensation is allowed for normal video sequences without a restriction, it is asserted that undesired visual artifacts might appear at boundary by copying and using unconnected reverse-side pixel data for motion compensation.

Another problem is that there is no appropriate range of sps_ref_wraparound_offset in the current semantics. The allowed value if greater than 0 and smaller than a picture width  (pic_width_in_luma_samples) in the current VVC WD. If the value of sps_ref_wraparound_offset is not greater than pic_width_in_luma_samples/2 - 1, the same pixel values are circularly repeated for the wrap-around motion compensation. It is asserted that the circularly repeated motion compensation is not intended by the original proposal.

# Proposal

In order to resolve the above problems, the following changes are proposed:

- JVET-M0451 proposes no_ref_wraparound_constraint_flag. JVET-M0101 proposes general_non_packed_constraint_flag. It is proposed in this contribution that sps_ref_wraparound_enabled_flag shall be equal to 0, when general_non_packed_contraint_flag is equal to 1 as well as no_ref_wraparound_constraint_flag is equal to 1.

- The value of sps_ref_wraparound_offset is restricted. sps_ref_wraparound_offset shall be greater than pic_width_in_luma_samples / 2 - 1, and shall not be greater than pic_width_in_luma_samples.

# Proposed Spec Text.

| profile_level( maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| … |  |
| general_non_packed_constraint_flag | u(1) |
| … |  |
| no_ref_wraparound_constraint_flag | u(1) |
| … |  |
| } |  |

general_non_packed_constraint_flag equal to 1 indicates that the cropped output pictures of the decoded bitstream are suitable to be displayed. general_non_packed_constraint_flag equal to 0 indicates that the cropped output pictures of the decoded bitstream may require further processing to be suitable for displaying.

no_ref_wraparound_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_ref_wraparound_enabled_flag shall be equal to 0. no_ref_wraparound_constraint_flag equal to 0 does not impose a constraint.

general_non_packed_constraint_flag equal to 1 indicates that the cropped output pictures of the decoded bitstream are suitable to be displayed. general_non_packed_constraint_flag equal to 0 indicates that the cropped output pictures of the decoded bitstream may require further processing to be suitable for displaying.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| profile_level( maxNumSubLayersMinus1 ) |  |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset | ue(v) |
| ... |  |
| } |  |

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. It is a requirement of bitstream conformance that sps_ref_wraparound_enabled_flag is equal to 0, when no_ref_wraparound_constraint_flag is equal to 1 or general_non_packed_contraint_flag is equal to 1.

sps_ref_wraparound_offset specifies the offset used for computing the horizontal wrap-around position in luma samples. sps_ref_wraparound_offset shall be in the range of pic_width_in_luma_samples / 2 to pic_width_in_luma_samples, inclusive, and shall be an integer multiple of MinCbSizeY.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).