| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0238-v1 |
| --- | --- |

| Title: | Non-CE3: Modification of PDPC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Hahyun Lee Sung-Chang Lim Jungwon Kang Hui Yong Kim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr hanilee@etri.re.kr sclim@etri.re.kr jungwon@etri.re.kr hykim5@etri.re.kr |
| Source: | ETRI (Electronics and Telecommunications Research Institute) |  |  |

_____________________________

- Abstract

This contribution presents modification of position dependent intra prediction combination (PDPC). It includes two modifications which are large block size restriction and removal of interpolation. It is reported that the average luma BD-rate of the block size restriction and the removal of interpolation for AI case is -0.01% and 0.01% compared to VTM 3.0, respectively

- Algorithm description

The position dependent intra prediction combination (PDPC) is applied to intra predicted samples for all block sizes when an intra prediction mode is one of DC, Planar, horizontal/vertical, an angular mode which is less than mode 11, and an angular mode which is greater than mode 57.

In this contribution, two modifications of PDPC such as block size restriction and removal of interpolation are proposed to reduce complexity.

  - Block size restriction

The PDPC applies filters to intra predicted samples using reference samples with weights. Therefore, each predicted sample is refined by applying weight value according to distance from reference sample. However, the PDPC may not perform efficiently for large blocks since the predicted samples are similar to the reference samples. It could reduce complexity without loss by restricting PDPC on large blocks, so the proposed method restricts PDPC when block width or height is 64. As a result, the maximum block size for PDPC is 32x32.

  - Removal of interpolation

For intra angular prediction, 4-tap interpolation filter is applied to reference samples to generated prediction samples. For example, above four reference samples are interpolated for prediction when a vertical angular prediction mode is used as depicted in Fig. 1. In addition, a bilinear interpolation filter is applied to left two reference samples in PDPC when an intra prediction mode is larger than or equal to INTRA_ANGULAR58. Likewise, when an intra prediction mode is less than or equal to INTRA_ANGULAR10 a bilinear filter is applied to above two reference samples for PDPC. To reduce computational complexity, a nearest integer reference sample is used for PDPC rather than applying the bilinear interpolation filter.

Figure 1. Interpolation for intra prediction and PDPC in VTM 3.0

Experimental results

Experiments were performed based on VTM 3.0 software [1] under JVET common test condition [2].

Table 1 and Table 2 show experimental results of the each proposed method compared to VTM 3.0.

Table 1. Results of block size restriction over VTM 3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.03% | -0.01% | 100% | 98% |
| Class A2 | 0.01% | 0.00% | -0.02% | 100% | 99% |
| Class B | 0.00% | -0.03% | 0.02% | 100% | 100% |
| Class C | -0.01% | 0.03% | 0.03% | 100% | 100% |
| Class E | -0.03% | -0.06% | 0.10% | 100% | 99% |
| Overall | -0.01% | -0.02% | 0.02% | 100% | 99% |
| Class D | 0.00% | 0.06% | 0.03% | 100% | 100% |
| Class F (optional) | 0.01% | 0.02% | 0.05% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | 0.19% | 99% | 100% |
| Class A2 | 0.00% | 0.23% | 0.23% | 100% | 100% |
| Class B | -0.01% | 0.06% | 0.03% | 100% | 100% |
| Class C | 0.00% | -0.03% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.08% | 0.10% | 100% | 100% |
| Class D | 0.00% | -0.03% | 0.03% | 100% | 100% |
| Class F (optional) | 0.01% | 0.06% | 0.06% | 100% | 100% |

Table 2. Results of removal of interpolation over VTM 3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.09% | 0.05% | 100% | 100% |
| Class A2 | 0.02% | -0.02% | 0.02% | 100% | 100% |
| Class B | 0.03% | 0.01% | -0.02% | 100% | 100% |
| Class C | 0.00% | 0.01% | -0.06% | 100% | 100% |
| Class E | 0.02% | 0.04% | 0.08% | 100% | 100% |
| Overall | 0.01% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.01% | 0.03% | -0.07% | 100% | 100% |
| Class F (optional) | -0.01% | 0.01% | 0.10% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.00% | 0.11% | 100% | 100% |
| Class A2 | -0.01% | 0.05% | 0.17% | 100% | 100% |
| Class B | -0.02% | 0.03% | -0.08% | 100% | 100% |
| Class C | 0.03% | -0.06% | 0.14% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.07% | 100% | 100% |
| Class D | 0.00% | -0.21% | 0.06% | 100% | 100% |
| Class F (optional) | 0.02% | 0.04% | 0.08% | 100% | 100% |

Conclusions

This contribution reports two simplifications of PDPC to reduce complexity. The average luma BD-rates and encoding/decoding time are almost same when all the proposed methods are applied. We suggest the proposed methods to be adopted into VTM 4.0.

Reference

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, November 2018.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex - Decoding process (Based on JVET-L1001-v7)

Block size restriction

8.2.4.2.1	General intra sample prediction

…

When refIdx is equal to 0, nTbW is less than MaxTbSizeY, nTbH is less than MaxTbSizeY, and one of the following conditions is true, the position-dependent prediction sample filtering process specified in clause 8.2.4.2.9 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the output is the modified predicted sample array predSamples:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is equal to INTRA_ANGULAR18

- predModeIntra is equal to INTRA_ANGULAR50

- predModeIntra is less than or equal to INTRA_ANGULAR10

- predModeIntra is greater than or equal to INTRA_ANGULAR58

Removal of interpolation

8.2.4.2.9	Position-dependent intra prediction sample filtering process

Inputs to this process are:

…

- Otherwise, if predModeIntra is less than or equal to INTRA_ANGULAR10, the following ordered steps apply:

- The variables dXPos[ y ], dXFrac[ y ], dXInt[ y ] and dX[ x ][ y ] are derived as follows using invAngle as specified in clause 8.2.4.2.7 depending on intraPredMode:

dXPos[ y ] =	( ( y + 1 ) * invAngle + 2 )  >>  2
dXFrac[ y ] =	dXPos[ y ] & 63		(8-173)
dXInt[ y ] =	dXPos [ y ]  >>  6
dX[ x ][ y ] =	x + dXInt[ y ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ x ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = 0		(8-174)

refT[ x ][ y ] = ( dX[ x ][ y ] < refW − 1 )  ?  ( ( 64 − dXFrac[ y ] ) * mainRef[ dX[ x ][ y ] ] + 
	dXFrac[ y ] * mainRef[ dX[ x ][ y ] + 1 ] + 32 )  >>  6	(8-175)
	mainRef[ dX[ x ][ y ] + dXFrac[ y ] >> 5 ]  :  0

wT[ y ] = ( dX[ x ][ y ] < refW − 1 )  ?  32  >>  ( ( y  <<  1 )  >>  nScale )  :  0	(8-176)

wL[ x ] = 0		(8-177)

wTL[ x ][ y ] = 0		(8-178)

- Otherwise, if predModeIntra is greater than or equal to INTRA_ANGULAR58, the following ordered steps apply:

- The variables dYPos[ x ], dYFrac[ x ], dYInt[ x ] and dY[ x ][ y ] are derived as follows using invAngle as specified in clause 8.2.4.2.7 depending on intraPredMode:

dYPos[ x ] =	( ( x + 1 ) * invAngle + 2 )  >>  2
dYFrac[ x ] =	dYPos[ x ] & 63		(8-179)
dYInt[ x ] =	dYPos[ x ]  >>  6
dY[ x ][ y ] =	y + dYInt[ x ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ x ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = ( dY[ x ][ y ] < refH − 1 )  ?  ( ( 64 − dYFrac[ x ] ) * sideRef[ dY[ x ][ y ] ] + 
	dYFrac[ x ] * sideRef[ dY[ x ][ y ] + 1 ] + 32 )  >>  6	(8-180)
	sideRef[ dY[ x ][ y ] + dYFrac[ x ] >> 5 ]  :  0

refT[ x ][ y ] = 0		(8-181)

wT[ y ] = 0		(8-182)

wL[ x ] = ( dY[ x ][ y ] < refH − 1 )  ?  32  >>  ( ( x  <<  1 )  >>  nScale )  :  0	(8-183)

wTL[ x ][ y ] = 0