| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0252 |
| --- | --- |

| Title: | CE3-1.3: Harmonization of Linear interpolation intra prediction (LIP) with Multiple reference line prediction (MRL) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Jangwon Choi Jungah Choi Sunmi Yoo Ling Li Jaehyun Lim 19 Yangjaedaero-11gil, Seocho-gu Seoul 06772, South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 jin78.heo@lge.com jangwon84.choi@lge.com jungah.choi@lge.com sunmi.yoo@lge.com l.li@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution describes CE the results of CE3-1.3 on linear interpolation intra prediction (LIP) [1]. In this contribution, we harmonized LIP with multiple reference line prediction (MRL) [2]. Simulation results for this test reportedly showed average luma BD-rate savings of 0.08% and 0.04% compared to VTM3.0-AI and VTM3.0-RA, respectively.

Proposed method

Linear interpolation intra prediction (LIP) is applied to all intra prediction modes except Planar and DC modes. LIP computes a linear combination of the opposite side reference samples with weights dependent on the position within the block. This test applies LIP to MPM candidate lists, LIP flag is signaled after MPM flag. LIP is only applied to the nearest reference lines, i.e., reference index 0 and wide angle intra prediction (WAIP) is disabled when LIP is on. The following aspects cover the summary of LIP process.

- Intra prediction modes:

  - All intra prediction modes excerpt Planar and DC

- LIP flag signalling:

  - LIP flag parsing after MPM flag – since LIP is applied to MPMs, LIP flag is parsed when MPM flag is true. If mrl_idx > 0, skip the LIP flag parsing process.

- Restrictions

  - MRL, i.e., disable LIP for additional reference line

  - WAIP, i.e., disable WAIP for LIP

Experimental results

This CE test has been implemented based on VTM3.0 reference software and experimentally evaluated for VTM test according to common test conditions defined in JVET-L1010 [3].

Table 1 Results of CE3-1.3 over VTM3.0-AI

Table 2 Results of CE3-1.3 over VTM3.0-RA

Reference

- J. Heo, et al., “CE3-4.1: Harmonization of Linear interpolation intra prediction (LIP) with Simplified position dependent intra prediction combination (PDPC) and wide-angle intra prediction (WAIP),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0131, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- B. Bross, et al., “CE3: Multiple reference line intra prediction (Test 1.1.1, 1.1.2, 1.1.3 and 1.1.4),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0283, 12th meeting, Macao, CN, 3-12 Oct. 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L1010, 12th meeting, Macao, CN, 3-12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).