| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0253 |
| --- | --- |

| Title: | Non-CE8: Hash-based Motion Search |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Jiahao Li*, Kai Zhang, Li Zhang, Hongbin Liu, Yue Wang, Pengwei Zhao, Dingkun Hong, Ruiqin Xiong* 8910 University Center Lane San Diego, CA, U.S.A. 92122 *Peking University | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. Peking University |  |  |

_____________________________

# Abstract

This contribution presents a hash-based motion search scheme for the VVC test model. For 4x4 to 64x64 square blocks and 4x8, 8x4 blocks, a hash table is generated for each reference picture. A hash-based block matching is then performed before existing motion estimation for those block sizes. When a block finds its match, the following motion estimation can be skipped. For class SCC sequences, experimental results report 9.6% and 18.87% coding gain for RA and LD_B, with encoding time reduced to 89% and 88%. When CPR is on, 7.81% and 14.91% coding gain is reported, with encoding time reduced to 91% and 89%.

By using an adaptive scheme to determine whether hash search is used or not, encoding time for class A-E can be kept similar.

# Introduction

During the development of HEVC SCC, it was asserted that for screen contents, hash-based motion search can perform effectively to find a block’s match in the reference picture and overcome some shortcoming of conventional motion estimation. The advantages of hash-based motion search include

- The search range can be full picture. The complexity of full picture search is similar to search within a given region.

- Hash-based search can be complemental to conventional motion search, especially for screen contents, conventional fast motion search may not work well.

- When hash-based search finds match, the following motion estimation can be skipped to save encoding time.

Based on the above reasons, this contribution proposes a hash-based motion search scheme for VTM and presents its performance.

# Hash-based Motion Search

In the scheme, hash-based motion search is performed for 4x4, 4x8, 8x4, 8x8, 16x16, 32x32 and 64x64 blocks. For each reference picture, hash tables corresponding to those block sizes are generated. For a block with hash tables, hash based block match is performed. When a hash match is found, the following checks can be early terminated.

## Hash table generation

To enable hash-based search for different block sizes, corresponding hash tables are generated. To save computations, we take a bottom-to-up manner to speed up the hash table generation. The hash table generation is built as follows:

- For 2x2 block, the block hash value is calculated directly from the original pixels (for YUV 4:2:0, only luma component is utilized). The CRC (Cyclic Redundancy Check) value is used as the hash value.

- For 4x4, 8x8, 16x16, 32x32 and 64x64 square blocks, the hash value of the current block is the CRC value calculated from the CRC values of its four sub blocks.

- For 4x8 and 8x4 blocks, the hash value of the current block is the CRC value calculated from the CRC values of its two sub 4x4 blocks.

To avoid the hash conflicts, we take the structure of inverted index table. To control the length of hash table, some hash values will not be added into the table if their corresponding blocks satisfy the following conditions:

- There is only one-pixel value in every row or column.

- The horizontal (vertical) position is not integral multiples of block width (height).

## Early termination

For inter coding, the hash-based motion search is performed before all coding modes. To accelerate the encoding process, the other coding modes (except the skip part of ETM_MERGE_SKIP, ETM_AFFINE, ETM_MERGE_TRIANGLE modes) and finer-grained block splitting are skipped if the following conditions are satisfied:

- Current block size is 64x64.

- Hash match is found in reference frame.

- The QP of reference frame is not larger than that of current frame.

In addition, for all block sizes hash search supported, the hash-based motion vector is checked for the start point initialization in the normal integral motion search. If the hash-based motion vector exists, the fractional motion estimation is skipped.

In a recent implementation, for each sequence, we determine if the proposed hash search is applied or not. By doing so, the encoding time for camera captured contents can be kept similar.

# Experimental Results

The proposed hash-based motion search is tested on VTM 3.0 (commit SHA-1: 3b48a28984134dc696daf28

-822985fb4f83d1c30). We test the results with and without CRP enabling.

Table I and II show the performance with hash-based search always applied. It brings a constant cost to the encoder even for camera-capture contents.

Table II and IV show the performance with adaptive hash-based search, in which we perform a pre-analysis to determine if hash-based search is applied or not. By doing so, encoding of camera-capture contents can be kept similar to the current encoding.

Table I. Inter Hash-Based Motion Search - CPR Off

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.03% | 104% | 100% |
| Class A2 | 0.00% | 0.05% | -0.02% | 104% | 100% |
| Class B | 0.00% | 0.01% | 0.01% | 104% | 100% |
| Class C | 0.00% | 0.01% | 0.00% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 104% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class F (optional) | -0.12% | -0.18% | -0.19% | 100% | 101% |
| Class SCC 1080p | -9.60% | -8.93% | -8.78% | 89% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.01% | 0.04% | 105% | 101% |
| Class C | -0.01% | 0.00% | -0.01% | 104% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 111% | 102% |
| Overall | 0.00% | 0.01% | 0.01% | 106% | 100% |
| Class D | 0.00% | -0.02% | 0.09% | 104% | 102% |
| Class F (optional) | -1.79% | -2.10% | -1.31% | 102% | 101% |
| Class SCC 1080p | -18.87% | -17.45% | -17.14% | 88% | 96% |

Table II. Inter Hash-Based Motion Search - CPR On

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.05% | 104% | 101% |
| Class A2 | 0.00% | 0.04% | 0.00% | 103% | 99% |
| Class B | 0.00% | 0.02% | 0.02% | 103% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.02% | 103% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Class F (optional) | -0.08% | -0.11% | -0.21% | 101% | 100% |
| Class SCC 1080p | -7.81% | -7.32% | -7.12% | 91% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.16% | -0.04% | 106% | 101% |
| Class C | -0.01% | 0.00% | 0.05% | 104% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 111% | 100% |
| Overall | 0.00% | 0.07% | 0.00% | 106% | 100% |
| Class D | 0.01% | 0.11% | -0.12% | 104% | 99% |
| Class F (optional) | -1.90% | -1.26% | -2.00% | 102% | 98% |
| Class SCC 1080p | -14.91% | -13.59% | -13.26% | 89% | 94% |

Table III. Adaptive Inter Hash-Based Motion Search - CPR Off

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F (optional) | -0.15% | -0.21% | -0.23% | 98% | 100% |
| Class SCC 1080p | -9.60% | -8.93% | -8.78% | 88% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class F (optional) | -1.76% | -2.07% | -1.41% | 101% | 101% |
| Class SCC 1080p | -18.87% | -17.45% | -17.14% | 88% | 95% |

Table IV. Adaptive Inter Hash-Based Motion Search - CPR On

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class F (optional) | -0.09% | -0.15% | -0.16% | 99% | 100% |
| Class SCC 1080p | -7.81% | -7.34% | -7.13% | 91% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 95% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | -1.88% | -1.26% | -1.92% | 103% | 99% |
| Class SCC 1080p | -15.01% | -13.57% | -13.24% | 88% | 93% |

# Conclusions

This contribution presents a hash based motion search method. With the proposed method, it could bring up to 14.91% coding gains with 10% encoding time saving. It is recommended to adopt this to the next release of VTM software.

# Patent rights declaration(s)

Bytedance Inc. and Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).