| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0265 |
| --- | --- |

| Title: | AHG16: Clean-up on MV Rounding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In VTM-3.0, MV averaging is performed by pair-wise merge candidate, triangular prediction and affine prediction for chroma components with different ways of rounding. It is proposed to unify the MV rounding operation to be the same as MV rounding in affine MV derivation. In addition, it is proposed not to clip MVs for the luma component, before calculating the MV for chroma components in affine prediction, to align the VTM software and the working draft. Simulation results reportedly show 0.00% BD-rate change under Random Access (RA) configurations.

# Introduction

In VTM-3.0, MV averaging is performed by inter-intra combined prediction, triangular prediction and affine prediction for chroma components. After getting the sum S, the average value M is calculated in different ways of rounding:

Pairwise: M = S/2;

Triangular: M = (S + 1) >> 1;

MV for chroma in affine: M = (S + 2) >> 2;

But in the derivation of affine model, MV rounding with a right shift F is defined as:

Offset = 1 << (F-1);

M= S >= 0 ? (S + Offset) >> F: -((-S + Offset) >> F).

Besides, the MVs for the luma component are clipped before calculating the MV for chroma components in affine prediction in VTM-3, which is not aligned with the working draft.

# Proposed methods

It is proposed to unify all the MV rounding to be the same as that defined in the affine model derivation. Besides, it is proposed to remove MV clipping before calculating the MV for chroma components in affine prediction to align the VTM software and the working draft.

# Experimental Results

The proposed clean-up has been implemented on top of VTM-3.0. Results of RA and LB configuration are summarized in Table 1. The simulations were performed following the JVET CTC [1]. More detail results can be found in the attached spreadsheets.

Table 1: Results for the MV rounding clean-up

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.11% | 0.02% | 99% | 99% |  |  |  |  |  |
| Class A2 | -0.03% | 0.13% | 0.12% | 100% | 100% |  |  |  |  |  |
| Class B | 0.01% | 0.06% | 0.03% | 100% | 96% | 0.05% | -0.11% | -0.05% | 100% | 100% |
| Class C | 0.01% | 0.07% | 0.16% | 101% | 98% | 0.02% | 0.45% | 0.28% | 101% | 102% |
| Class E |  |  |  |  |  | 0.06% | 0.50% | 0.50% | 100% | 98% |
| Overall | 0.00% | 0.04% | 0.08% | 100% | 98% | 0.04% | 0.23% | 0.20% | 100% | 100% |
| Class D | 0.02% | -0.06% | -0.07% | 101% | 91% | 0.00% | -0.48% | 0.28% | 100% | 97% |
| Class F | -0.01% | 0.08% | 0.06% | 100% | 97% | -0.09% | 0.01% | 0.11% | 101% | 92% |

# Conclusion

This contribution proposes to unify all the MV rounding process and align the SW with the working draft for the sub-block chroma affine MV derivation. Both aspects are beneficial for hardware/software implementation with almost identical performance. It is recommended to adopt these two aspects to the next version of VVC.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-L1010, “JVET common test conditions and software reference configurations for SDR video”, Oct. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).