| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0273 |
| --- | --- |

| Title: | CE2-related: Early awareness of accessing temporal blocks in sub-block merge list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Kai Zhang, Hongbin Liu, Jizheng Xu, Yue Wang, Pengwei Zhao, Dingkun Hong 8910 University Center Ln, San Diego, CA, U.S.A. 92122 | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com zhangkai.video@bytedance.com liuhongbin.01@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC design, a sub-block merge list is constructed in a way that both ATMVP candidate and affine candidates are inserted in order. For both the ATMVP candidate and affine candidates, they rely on temporal motion information from collocated picture but may be from different locations. In addition, for some cases, the ATMVP candidate may be unavailable. This contribution proposes three modifications: 1) Only 1 spatial neighboring block, instead of 4, is checked to derive initial MV; 2) ATMVP candidate is always available; 2) remove TMVP in affine candidate construction process. With the proposed method, the decoder could immediately know whether loading temporal motion information is needed after paring the sub-block merge candidate index. In addition, the decoder is able to skip the ATMVP candidate construction process based on the decoded candidate index. Simulation results reportedly show that the proposed methods achieves 0.02% and 0.09% BD rate reduction for RA Main10 and LDB Main 10 configurations, respectively.

# Introduction

In the 3rd VVC meeting, it is decided to put both subblock-based temporal motion vector predictors (a.k.a., ATMVP) and affine merge candidates to the same merge list, named sub-block merge list. An index to the sub-block merge list may be signalled to indicate which candidate is selected. More specifically, the sub-block merge list is constructed with the following sequential of steps:

- Step 1: ATMVP candidate derivation

  - Step 1.1: obtain initial MV

Get the motion information from the first block in scan order (i.e., block 1-4 in Figure 1) that is an inter block. If any of the two MVs points to the collocated picture, the initial MV is set to this MV. Otherwise, zero motion vector is set as the initial MV.

  - Step 1.2: obtain default ATMVP candidate

Identify a temporal block with the initial MV and center position of current block. If the motion information of the temporal block is unavailable (e.g., intra coded), ATMVP candidate is set as unavailable, go to step 2. Otherwise, use the temporal motion information of the temporal block to derive the default ATMVP candidate.

  - Step 1.3: obtain sub-block motion candidates

For each sub-block, locate a corresponding temporal block, use the motion information of the corresponding temporal block to derive sub-block’s motion information. If it is unavailable, use the default ATMVP candidate as the sub-block’s motion information.

- Step 2: Affine merge candidates insertion

  - Step 2.1: Insertion of inherited affine motion predictors

  - Step 2.2: Insertion of constructed affine candidates

Derived from the spatial neighbors and temporal neighboring block (block 6 in Figure 1).

  - Step 2.3: Insertion of default motion candidates

Figure 1. Position of the neighboring blocks

# Problem statements

From the descriptions mentioned aforementioned, the current design may have the following problems:

- In the worst case, the number of accesses to spatial neighboring blocks is 4 to derive the initial MV used by ATMVP.

- Both ATMVP and affine merge candidates need to locate motion information from temporal blocks but located at different locations.

- Based on the decoded sub-block merge index, it is impossible to tell whether it is ATMVP or affine merge candidates since ATMVP could be available. Therefore, the decoder has to always invoke the ATMVP process.

# Proposed methods

To solve the problems mentioned above, three modifications are proposed in this contribution:

- Only check the left spatial neighboring block (i.e., block 1 in Figure 1) to derive the initial MV used in ATMVP process;

- ATMVP candidate is always set available. When the default ATMVP candidate is unavailable (the temporal motion information of step 1.2 is unavailable), the initial MV and collocated picture index is set as the temporal motion information of the temporal block. It is used to derive default ATMVP candidate.

- The temporal block checked for constructed affine candidates is removed.

With all the three proposed modifications, after decoding a sub-block merge index, the decoder could immediately know whether ATMVP candidate should be constructed or not. Since ATMVP candidate is always available, if the decoded index (i.e., 0) denotes an ATMVP candidate, temporal information needs to be accessed. If the decoded index denotes an ATMVP candidate, the ATMVP candidate construction process could be skipped and only spatial neighboring blocks need to be utilized.

# Experimental Results

The proposed methods are implemented on VTM-3.0 [2]. Three sets of simulations are performed following the JVET CTC [3]. The three sets are listed as follows:

- Test #1: ATMVP changes (first two proposed changes)

- Test #2: modification from affine (the 3rd proposed change)

- Test #3: all three modifications

Results of Random Access (RA) configuration and Low-delay B (LB) configuration are summarized in the Table 1. More detail results can be found in the attached spreadsheets.

Table 1. Results of Test #1 compared to VTM-3.0

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.01% | 101% | 99% |  |  |  |  |  |
| Class A2 | -0.05% | 0.03% | 0.07% | 101% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.04% | -0.04% | 100% | 99% | 0.02% | -0.17% | 0.25% | 101% | 99% |
| Class C | 0.02% | 0.02% | 0.02% | 99% | 98% | 0.02% | 0.05% | 0.20% | 100% | 101% |
| Class E |  |  |  |  |  | 0.06% | 0.16% | -0.12% | 99% | 100% |
| Overall | 0.00% | 0.02% | 0.01% | 100% | 99% | 0.03% | -0.01% | 0.14% | 100% | 100% |
| Class D | 0.04% | -0.02% | 0.02% | 102% | 103% | -0.02% | -0.29% | 0.26% | 99% | 98% |
| Class F | -0.03% | -0.06% | -0.06% | 102% | 101% | -0.05% | 0.14% | -0.15% | 101% | 100% |

Table 2. Results of Test #2 compared to VTM-3.0

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.04% | 0.03% | 100% | 99% |  |  |  |  |  |
| Class A2 | 0.01% | 0.01% | 0.11% | 100% | 100% |  |  |  |  |  |
| Class B | 0.01% | 0.07% | -0.03% | 100% | 100% | 0.08% | -0.28% | -0.14% | 100% | 99% |
| Class C | 0.02% | -0.03% | 0.00% | 100% | 100% | 0.09% | 0.26% | -0.02% | 99% | 102% |
| Class E |  |  |  |  |  | 0.11% | 0.10% | 0.13% | 101% | 101% |
| Overall | 0.01% | 0.01% | 0.02% | 100% | 100% | 0.09% | 0.00% | -0.04% | 100% | 101% |
| Class D | 0.02% | -0.03% | 0.03% | 99% | 101% | 0.10% | 0.07% | -0.39% | 100% | 100% |
| Class F | 0.03% | -0.01% | 0.04% | 100% | 100% | 0.11% | 0.29% | -0.02% | 100% | 101% |

Table 3. Results of Test #3 compared to VTM-3.0

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.01% | 0.04% | 101% | 99% |  |  |  |  |  |
| Class A2 | -0.03% | 0.05% | 0.06% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | -0.01% | -0.03% | 100% | 99% | 0.08% | -0.27% | -0.10% | 101% | 98% |
| Class C | 0.06% | 0.08% | 0.05% | 102% | 101% | 0.07% | 0.30% | 0.00% | 100% | 101% |
| Class E |  |  |  |  |  | 0.13% | 0.24% | -0.04% | 101% | 101% |
| Overall | 0.02% | 0.03% | 0.03% | 101% | 100% | 0.09% | 0.05% | -0.05% | 101% | 100% |
| Class D | 0.06% | 0.02% | -0.02% | 101% | 103% | 0.13% | 0.41% | -0.01% | 101% | 98% |
| Class F | -0.02% | -0.02% | -0.05% | 102% | 101% | -0.01% | 0.27% | 0.08% | 102% | 102% |

# Conclusions

In this contribution, three modifications to the sub-block merge candidate list are proposed. With the proposed methods, the decoder is able to directly decide whether to reconstruct ATMVP or affine candidates. Meanwhile, the decoder is able to decide whether to load temporal information according to the decoded index. It is recommended to consider adoption of the proposed methods to the next version of VTM.

# Reference

- “CE4: History-based Motion Vector Prediction (Test 4.4.7) ”, L. Zhang, K. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong, Doc. JVET-L0266, Oct. 2018.

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, Doc. JVET-J1010, April 2018.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).