| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0274 |
| --- | --- |

| Title: | CE3-related: Modified linear model derivation for CCLM modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Wang, Kai Zhang, Li Zhang, Hongbin Liu, Jizheng Xu, Shiqi Wang (Bytedance) Jurun Li, Shanshe Wang, Wen Gao (Peking University) | Tel: Email: | +1-858-380-8522 wangmeng.kids@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. and Peking University |  |  |

_____________________________

# Abstract

In VTM-3.0, the linear model for CCLM modes are derived by a max-min method. 128 neighbouring luma samples are down-sampled and 127 comparisons are performed on them to find the minimum and maximum values for a 64×64 chroma block. Besides, a lookup table with 2K bytes are stored on ROM to mimic the division operation. In this contribution, it is proposed that four neighbouring luma samples are down-sampled and three comparisons are performed on them for a 64×64 chroma block. And the lookup table size is reduced to be 128 bytes. Simulation results reportedly show 0.03% and 0.01% BD-rate loss under All Intra (AI) and Random Access (RA) configurations, respectively.

# Introduction

In VTM-3.0, the linear model for CCLM modes are derived by a max-min method. With LM mode, the minimum and maximum down-sampled neighbouring luma samples denoted as xA and xB are found by performing (W+H-1) comparisons on all down-sampled neighbouring luma samples. Suppose their corresponding chroma sample values are yA and yB, then the linear model parameters α and β are obtained according to the following equation:β= ∑Cn-α·∑LnN

.

A lookup table with 2K bytes are stored on ROM to mimic the division operation.

With LM-A or LM-L modes, xA and xB are found by performing 2W or 2H comparisons on all extended above or left down-sampled neighbouring luma samples.

# Proposed methods

It is proposed that only 4 neighbouring luma samples are down-sampled and searched to find smaller two values denoted as x0A and x1A, and larger two values denoted as x0B and x1B. Their corresponding chroma sample values are denoted as y0A, y1A, y0B and y1B. Then xA, xB, yA and yB are derived as:

xA=(x0A + x1A +1)>>1; xB=(x0B + x1B +1)>>1; yA=(y0A + y1A +1)>>1; yB=(y0B + y1B +1)>>1;

Then α and β are obtained according to the same equation as in VTM-3.0. Since we do not need to further compare x0A and x1A, or x0B and x1B, at most three comparisons are required.

Selection of down-sampled samples are defined as follows:

–	If it is normal LM mode, and both left and above neighboring blocks are available, the four down-sampled luma samples are fetched from locations [-1, 0], [-1, H - max(1, H/W)], [0, -1] and [W- max(1, W/H), -1].

–	Otherwise, four samples with equal distance are chosen, such as [0, -1], [0, X/4], [0, 2X/4], [0, 3X/4] wherein X indicates the maximumly allowed number of accessed samples (W, or 2W); or [-1, 0], [-1, Y/4], [-1, 2Y/4], [-1, 3Y/4].

Besides, the lookup table size is reduced to be with 128 bytes.

Table 1 summarizes the complexity comparison between VTM-3.0 and the proposed method to derive the CCLM parameter derivation process for a 64×64 chroma block.

Table 1: Complexity comparison for CCLM parameter derivation on a 64×64 chroma block.

|  | Luma sample down-sampling | Comparison between luma samples | Lookup table Size |
| --- | --- | --- | --- |
| VTM-3.0 | 128 | 127 | 2K bytes |
| Proposed | 4 | 3 | 128 bytes |
| Proposed/VTM-3.0 | 3.125% | 2.36% | 6.250% |

# Experimental Results

The proposed methods have been implemented on top of VTM-3.0. Three tests are listed as follows:

- Test #1: CCLM with four down-sampled luma samples.

- Test #2: LUT simplification.

- Test #3: #1+#2

The simulations were performed following the JVET CTC [1]. Table 2-Table 4 summarize the results for Test #1 – Test #3, respectively. More detail results can be found in the attached spreadsheets.

Table 2: Results for Test #1

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.12% | -0.05% | 99% | 95% |
| Class A2 | 0.07% | 0.28% | 0.17% | 101% | 98% |
| Class B | 0.03% | -0.33% | -0.27% | 100% | 98% |
| Class C | 0.04% | -0.14% | -0.06% | 97% | 95% |
| Class E | 0.03% | 0.19% | 0.22% | 101% | 96% |
| Overall | 0.04% | -0.02% | -0.03% | 100% | 97% |
| Class D | 0.06% | 0.12% | 0.35% | 98% | 95% |
| Class F | 0.06% | 0.22% | 0.37% | 98% | 96% |

Table 3: Results for Test #2

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.21% | -0.05% | 99% | 99% |
| Class A2 | 0.01% | -0.03% | 0.02% | 101% | 100% |
| Class B | 0.00% | 0.00% | -0.02% | 100% | 99% |
| Class C | 0.00% | 0.01% | -0.13% | 98% | 96% |
| Class E | -0.01% | 0.09% | 0.09% | 101% | 99% |
| Overall | 0.00% | -0.02% | -0.03% | 100% | 99% |
| Class D | 0.00% | 0.01% | -0.03% | 100% | 98% |
| Class F | -0.01% | 0.11% | 0.12% | 99% | 99% |

Table 4: Results for Test #3

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | -0.18% | 98% | 95% | 0.00% | 0.01% | 0.12% | 99% | 99% |
| Class A2 | 0.07% | 0.28% | 0.19% | 100% | 98% | 0.01% | 0.38% | 0.25% | 99% | 100% |
| Class B | 0.04% | -0.27% | -0.26% | 99% | 98% | 0.02% | -0.26% | -0.36% | 99% | 100% |
| Class C | 0.01% | -0.05% | -0.01% | 98% | 97% | 0.01% | -0.10% | 0.18% | 98% | 94% |
| Class E | 0.04% | 0.07% | 0.26% | 99% | 97% |  |  |  |  |  |
| Overall | 0.03% | -0.04% | -0.03% | 99% | 97% | 0.01% | -0.03% | 0.00% | 99% | 98% |
| Class D | 0.05% | 0.21% | 0.33% | 98% | 97% | 0.04% | -0.29% | 0.12% | 99% | 98% |
| Class F | 0.08% | 0.22% | 0.42% | 98% | 98% | 0.07% | 0.24% | 0.33% | 98% | 96% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-L1010, “JVET common test conditions and software reference configurations for SDR video”, Oct. 2018.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking Unviersity may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).