| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0277 |
| --- | --- |

| Title: | Non-CE: Fixes of enabling pcm_loop_filter_disabled_flag with PCM mode signalling under dual tree partition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Kai Zhang, Jizheng Xu, Hongbin Liu, Yue Wang, Pengwei Zhao, Dingkun Hong | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VVC, PCM mode flags for luma and chroma components are signalled separately when dual tree partition is enabled. However, when the pcm_loop_filter_disabled_flag is enabled, determination of enabling/disabling SAO filtering process for chroma blocks is wrongly based on the signalled PCM mode flag for luma blocks. In addition, ALF is performed even when pcm_loop_filter_disabled_flag is true. In this contribution, these two issues are fixed wherein whether to filter a block is dependent on the signalled PCM mode flag for the associated color component. Simulation results reportedly show that there is no performance changes under common test conditions.

# Introduction

PCM mode under dual tree partition was adopted in the 3rd VVC meeting [1]. When dual tree partition is enabled, PCM mode flag is signalled separately for luma and chroma blocks. That is, for a luma block and its corresponding chroma blocks, they may be coded with different PCM mode flags. Therefore, when pcm_loop_filter_disabled_flag is set to true, the control of disabling filter processes for luma and chroma blocks shall be dependent on the associated PCM mode flags.

However, in the current reference software and VVC specification, there are two issues:

- For the SAO process, the current reference software disable SAO for both luma and chroma blocks based on the signalled luma PCM mode flag.

- For the ALF process, filtering process is still invoked when pcm_loop_filter_disabled_flag is set to true.

# Proposed methods

To fix the issues mentioned above, it is proposed to disabling filtering processes (SAO, ALF) for a block based on the signalled PCM mode flag for the associated color component.

Proposed syntax changes could be found in the attached document.

# Experimental Results

The proposed method is implemented on VTM-3.0 [3]. According to the the JVET CTC [4], no performance changes are observed.

# Conclusions

This contribution proposes to fix the issues of disabling filtering processes for a block based on the signalled PCM mode flag for the associated color component. It is recommended to consider adoption of the proposed method to the next version of VTM.

# Reference

- “PCM mode with dual tree partition”, Y. Sun, J. An, J. Lou, Doc. JVET-L0209, Oct. 2018.

- “JVET common test conditions and software reference configurations”, J. Boyce, K. Suehring, X. Li, V. Seregin, Doc. JVET-J1010, April 2018.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).