| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0277 |
| --- | --- |

| Title: | Non-CE: Fixes of enabling pcm_loop_filter_disabled_flag with PCM mode signalling under dual tree partition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang, Kai Zhang, Jizheng Xu, Hongbin Liu, Yue Wang, Pengwei Zhao, Dingkun Hong | Tel: Email: | 1-858-999-7093 lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document presents the suggested specification changes.

# Proposed specification changes

The proposed changes on top of JVET-L1001_v7 are highlighted in grey. The deleted parts are marked with strikethrough.

7.3.4.6	Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| pcm_luma_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_luma_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_luma_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| pcm_chroma_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_chroma_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_chroma_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } |  |
| } |  |
| if( !pcmFlag[ x0 ][ y0 ]) { |  |
| if( !pcm_flag[ x0 ][ y0 ]) { |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| … |  |
| } |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |
| } |  |

pcm_enabled_flag equal to 0 specifies that PCM-related syntax (pcm_sample_bit_depth_luma_minus1, pcm_sample_‌bit_depth_chroma_minus1, log2_min_pcm_luma_coding_block_size_minus3, log2_diff_max_min_pcm_luma_‌coding_block_size, pcm_loop_filter_disabled_flag, pcm_luma_flag, pcm_chroma_flag, pcm_luma_alignment_zero_bit, pcm_chroma_alignment_zero_bit syntax elements and pcm_sample( ) syntax structure) is not present in the CVS.

pcm_luma_flag[ x0 ][ y0 ] equal to 1 specifies that the pcm_sample( ) syntax structure is present and the transform_tree( ) syntax structure is not present in the coding unit including the luma coding block at the location ( x0, y0 ). pcm_ luma_flag[ x0 ][ y0 ] equal to 0 specifies that pcm_sample( ) syntax structure is not present. When pcm_ luma_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

The value of pcm_flagpcmFlag[ x0 + i ][ y0 + j ] with i = 1..cbWidth − 1, j = 1..cbHeight − 1 is inferred to be equal to pcm_flag[ x0 ][ y0 ].

pcm_ luma_alignment_zero_bit is a bit equal to 0.

pcm_chroma_flag[ x0 ][ y0 ] equal to 1 specifies that the pcm_sample( ) syntax structure is present and the transform_tree( ) syntax structure is not present in the coding unit including the chroma coding block at the location ( x0, y0 ). pcm_chroma_flag[ x0 ][ y0 ] equal to 0 specifies that pcm_sample( ) syntax structure is not present. When pcm_chroma_flag[ x0 ][ y0 ] is not present and treeType is equal to SINGLE_TREE, it is inferred to be equal to 0. When pcm_chroma_flag[ x0 ][ y0 ] is not present and treeType is equal to DUAL_TREE_CHROMA, it is inferred to be equal to pcm_luma_flag[ x0 ][ y0 ].

The value of pcmFlag[ x0 + i ][ y0 + j ] with i = 1..cbWidth − 1, j = 1..cbHeight − 1 is inferred to be equal to pcm_chroma_flag[ x0 ][ y0 ].

pcm_ chroma_alignment_zero_bit is a bit equal to 0.

8.5.3.2	CTB modification process

Inputs to this process are:

–	the picture sample array recPicture for the colour component cIdx,

–	a variable cIdx specifying the colour component index,

–	a pair of variables ( rx, ry ) specifying the CTB location,

–	the CTB width nCtbSw and height nCtbSh.

Output of this process is a modified picture sample array saoPicture for the colour component cIdx.

The variable bitDepth is derived as follows:

–	If cIdx is equal to 0, bitDepth is set equal to BitDepthY.

–	Otherwise, bitDepth is set equal to BitDepthC.

The location ( xCtb, yCtb ), specifying the top-left sample of the current CTB for the colour component cIdx relative to the top-left sample of the current picture component cIdx, is derived as follows:

( xCtb, yCtb ) = ( rx * nCtbSw, ry * nCtbSh )		(8-905)

The sample locations inside the current CTB are derived as follows:

( xSi, ySj ) = ( xCtb + i, yCtb + j )		(8-906)

( xYi, yYj ) = ( cIdx  = =  0 ) ? ( xSi, ySj ) : ( xSi * SubWidthC, ySj * SubHeightC )	(8-907)

For all sample locations ( xSi, ySj ) and ( xYi, yYj ) with i = 0..nCtbSw − 1 and j = 0..nCtbSh − 1, depending on the values of pcm_loop_filter_disabled_flag, pcm_flag[ xYi ][ yYj ] and cu_transquant_bypass_flag of the coding unit which includes the coding block covering recPicture[ xSi ][ ySj ], the following applies:

–	If one or more of the following conditions are true, saoPicture[ xSi ][ ySj ] is not modified:

- pcm_loop_filter_disabled_flag and (cIdx = =0 ? pcm_luma_flag[ xYi ][ yYj ] : pcm_chroma_flag[ xYi ][ yYj ]) pcm_flag[ xYi ][ yYj ] are both equal to 1.

- cu_transquant_bypass_flag is equal to 1.

- SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 0.

…

8.5.4.2	Coding tree block filtering process for luma samples

Inputs of this process are:

- a reconstructed luma picture sample array recPictureL prior to the adaptive loop filtering process,

- a filtered reconstructed luma picture sample array alfPictureL,

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture.

Output of this process is the modified filtered reconstructed luma picture sample array alfPictureL.

The derivation process for filter index clause 8.5.4.3 is invoked with the location ( xCtb, yCtb ) and the reconstructed luma picture sample array recPictureL as inputs, and filtIdx[ x ][ y ] and transposeIdx[ x ][ y ] with x, y = 0..CtbSizeY − 1 as outputs.

For the derivation of the filtered reconstructed luma samples alfPictureL[ x ][ y ], each reconstructed luma sample inside the current luma coding tree block recPictureL[ x ][ y ] is filtered as follows with x, y = 0..CtbSizeY − 1:

- If pcm_loop_filter_disabled_flag and pcm_luma_flag[ xYi ][ yYj ] are both equal to 1, the following applies:

  - The locations ( hx, vy ) for each of the corresponding luma samples ( x, y ) inside the given array recPicture of luma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples − 1, xCtb + x )	(8-xxx)

vy = Clip3( 0, pic_height_in_luma_samples − 1, yCtb + y )	(8-xxx)

  - alfPictureL[ xCtb + x ][ yCtb + y ] is set to recPictureL[ hx, vy ].

- Otherwise, the folowing applies:

  - The array of luma filter coefficients f[ j ] corresponding to the filter specified by filtIdx[ x ][ y ] is derived as follows with j = 0..12:

f[ j ] = AlfCoeffL[ filtIdx[ x ][ y ] ][ j ]		(8-915)

  - The luma filter coefficients filterCoeff are derived depending on transposeIdx[ x ][ y ] as follows:

…

8.5.4.4	Coding tree block filtering process for chroma samples

Inputs of this process are:

- a reconstructed chroma picture sample array recPicture prior to the adaptive loop filtering process,

- a filtered reconstructed chroma picture sample array alfPicture,

- a chroma location ( xCtbC, yCtbC ) specifying the top-left sample of the current chroma coding tree block relative to the top left sample of the current picture.

Output of this process is the modified filtered reconstructed chroma picture sample array alfPicture.

The size of the current chroma coding tree block ctbSizeC is derived as follows:

ctbSizeC = CtbSizeY / SubWidthC		(8-956)

For the derivation of the filtered reconstructed chroma samples alfPicture[ x ][ y ], each reconstructed chroma sample inside the current chroma coding tree block recPicture[ x ][ y ] is filtered as follows with x, y = 0..ctbSizeC − 1:

  - The locations ( hx, vy ) for each of the corresponding chroma samples ( x, y ) inside the given array recPicture of chroma samples are derived as follows:

hx = Clip3( 0, pic_width_in_luma_samples / SubWidthC − 1, xCtbC + x )	(8-957)

vy = Clip3( 0, pic_height_in_luma_samples / SubHeightC − 1, yCtbC + y )	(8-958)

- If pcm_loop_filter_disabled_flag and pcm_chroma_flag[ xYi ][ yYj ] are both equal to 1, alfPicture[ xCtbC + x ][ yCtbC + y ] is set to recPicture[ hx, vy ].

- Otherwise, the folowing applies:

  - The variable sum is derived as follows:

sum = AlfCoeffC[ 0 ] * ( recPicture[ hx, vy + 2 ] + recPicture[ hx, vy − 2 ] ) + 
	AlfCoeffC[ 1 ] * ( recPicture[ hx + 1, vy + 1 ] + recPicture[ hx − 1, vy − 1 ] ) + 
	AlfCoeffC[ 2 ] * ( recPicture[ hx, vy + 1 ] + recPicture[ hx, vy − 1 ] ) + 	(8-959)
	AlfCoeffC[ 3 ] * ( recPicture[ hx − 1, vy + 1 ] + recPicture[ hx + 1, vy − 1 ] ) + 
	AlfCoeffC[ 4 ] * ( recPicture[ hx + 2, vy ] + recPicture[ hx − 2, vy ] ) + 
	AlfCoeffC[ 5 ] * ( recPicture[ hx + 1, vy ] + recPicture[ hx − 1, vy ] ) + 
	AlfCoeffC[ 6 ] * recPicture[ hx, vy ]

sum = ( sum + 64 ) >> 7		(8-960)

  - The modified filtered reconstructed chroma picture sample alfPicture[ xCtbC + x ][ yCtbC + y ] is derived as follows:

alfPicture[ xCtbC + x ][ yCtbC + y ] = Clip3( 0, ( 1 << BitDepthC ) − 1, sum )	(8-961)

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).