| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan 2019 | Document: JVET-M0313-r1 |
| --- | --- |

| Title: | CE4: Motion compensation constraints for complexity reduction (test 4.5.1 and test 4.5.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao Jingya Li Chong Soon Lim | Tel: Email: | +65-65505487 ruling.liao@sg.panasonic.com jingya.li@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| Source: | Panasonic corporation |  |  |

_____________________________

# Abstract

This contribution provides test results of the motion compensation constraints for complexity reduction in CE4. The bi-prediction is prohibited from small CU to guarantee that the worst-case memory bandwidth is not exceeded that of 8×8 bi-prediction. In test 4.5.1, bi-prediction is disabled for 4×8 and 8×4 CUs. As compared to VTM-3.0, 0.09% and 0.13% B-D rate loss are observed in random access and low delay B configurations, respectively. In test 4.5.2, bi-prediction is disabled for 4×8, 8×4, 4×16 and 16×4 CUs, and inter prediction is disabled for 4×4 CUs. As compared to VTM-3.0, 0.32% and 0.49% B-D rate loss are observed in random access and low delay B configurations, respectively.

# Introduction

To guarantee that the worst-case memory bandwidth is not exceeded that of 8×8 bi-prediction, bi-prediction is disabled for small CU. For the CU coded in merge mode, the motion vector from L0 direction of a bi-prediction motion vector candidate is used to predict the CU, which is the same situation in HEVC. For the CU coded in inter mode, only uni-prediction is allowed and the first bin of the inter direction which indicates uni-prediction or bi-prediction is not signaled. Table 1 shows the number of samples to be fetched for different CU size. The memory bandwidth of CUs that exceeded that of 8×8 bi-prediction are highlighted in gray.

Table 1: Number of luma samples needed to be fetched for different CU size

| CU size (W×H) | Number of luma samples to be fetched per luma sample (W + 7) × (H + 7) / (W × H) |
| --- | --- |
| 8×8 Bi-prediction | 7.03 |
| 4×8 Bi-prediction | 10.31 |
| 4×16 Bi-prediction | 7.91 |
| 4×32 Bi-prediction | 6.70 |
| 4×64 Bi-prediction | 6.10 |
| 8×4 Bi-prediction | 10.31 |
| 16×4 Bi-prediction | 7.91 |
| 32×4 Bi-prediction | 6.70 |
| 64×4 Bi-prediction | 6.10 |
| 4×4 Uni-prediction | 7.56 |

# Simulation results

The following experiments were conducted using the VTM-3.0 software with the JVET common test conditions [2] to collect the B-D rate, encoding and decoding time of the proposed techniques in the Random Access and Low Delay B configurations. Table 2 and Table 3 show the results of test 4.5.1 and test 4.5.2, respectively.

Table 2: Simulation results of test 4.5.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.09% | 100% | 97% |
| Class A2 | 0.04% | 0.20% | 0.33% | 100% | 96% |
| Class B | 0.09% | 0.25% | 0.26% | 99% | 95% |
| Class C | 0.18% | 0.30% | 0.29% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.19% | 0.25% | 99% | 96% |
| Class D | 0.30% | 0.31% | 0.38% | 99% | 98% |
| Class F | 0.10% | 0.13% | 0.16% | 99% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | -0.07% | 0.23% | 99% | 103% |
| Class C | 0.17% | 0.48% | 0.63% | 99% | 107% |
| Class E | 0.08% | -0.22% | 0.03% | 99% | 108% |
| Overall | 0.13% | 0.08% | 0.31% | 99% | 105% |
| Class D | 0.28% | 0.82% | 0.46% | 97% | 103% |
| Class F | 0.16% | 0.12% | 0.43% | 99% | 106% |

Table 3: Simulation results of test 4.5.2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | 0.17% | 99% | 99% |
| Class A2 | 0.23% | 0.56% | 0.82% | 100% | 97% |
| Class B | 0.30% | 0.74% | 0.70% | 99% | 97% |
| Class C | 0.64% | 1.24% | 1.43% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.32% | 0.71% | 0.81% | 99% | 97% |
| Class D | 1.18% | 1.80% | 1.94% | 100% | 98% |
| Class F | 0.47% | 0.68% | 0.77% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.35% | 0.52% | 0.73% | 98% | 99% |
| Class C | 0.83% | 1.55% | 1.34% | 99% | 99% |
| Class E | 0.27% | 0.59% | 0.21% | 98% | 100% |
| Overall | 0.49% | 0.88% | 0.80% | 98% | 99% |
| Class D | 1.42% | 1.74% | 1.79% | 99% | 97% |
| Class F | 1.30% | 0.93% | 1.13% | 100% | 100% |

# Conclusion

In this contribution, a solution is proposed to address the worst case memory bandwidth issue of VTM. Considering the trade-off of coding performance and hardware implementation complexity, we recommend to adopt test 4.5.1 into the next version of VVC.

# References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-L1024, Macao, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Macao, October 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed text

All the required changes are marked in yellow.

## Test 4.5.1

7.4.5.5	Coding unit semantics

inter_pred_idc[ x0 ][ y0 ] specifies whether list0, list1, or bi-prediction is used for the current coding unit according to Table 7-4. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-4 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |
| --- | --- | --- |
|  | ( cbWidth + cbHeight ) != 12 && ( cbWidth + cbHeight ) != 8 | ( cbWidth + cbHeight ) = = 12 && ( cbWidth + cbHeight ) = = 8 |
| 0 | PRED_L0 | PRED_L0 |
| 1 | PRED_L1 | PRED_L1 |
| 2 | PRED_BI | n.a. |

When inter_pred_idc[ x0 ][ y0 ] is not present, it is inferred to be equal to PRED_L0.

8.3.2.2	Derivation process for luma motion vectors for merge mode

- The following applies:

- If mergeCandList[ merge_idx[ xCb ][ yCb ] ] is equal to SbCol, the following assignments are made for xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1

refIdxLX = refIdxLXSbCol		(8-119)

predFlagLX[ xSbIdx ][ ySbIdx ] = predFlagLXSbCol[ xSbIdx ][ ySbIdx ]	(8-120)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = mvLXSbCol[ xSbIdx ][ ySbIdx ][ 0 ]	(8-121)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = mvLXSbCol[ xSbIdx ][ ySbIdx ][ 1 ]	(8-122)

- Otherwise, the following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ) and X being replaced by 0 or 1:

numSbX = 1		(8-123)

numSbY = 1		(8-124)

refIdxLX = refIdxLXN		(8-125)

predFlagLX[ 0 ][ 0 ] = predFlagLXN		(8-126)

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ]		(8-127)

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ]		(8-128)

- The following applies when (cbWidth + cbHeight) is equal to 8, 12 or 20:

- If mergeCandList[ merge_idx[ xCb ][ yCb ] ] is not equal to SbCol, and predFlagL0[ xSbIdx ][ ySbIdx ] is equal to 1 and predFlagL1[ xSbIdx ][ ySbIdx ]  is equal to 1, the following assignments are made:

refIdxL1 = -1

predFlagL1[ 0 ][ 0 ] = 0

## Test 4.5.2

7.3.4.5	Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I && ( cbWidth != 4 ¦ ¦ cbHeight != 4 ) ) { |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth != 4 ¦ ¦ cbHeight != 4 ) ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } else { /* MODE_INTER */ |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( sps_affine_enabled_flag && cbWidth >= 8 && cbHeight >= 8 && ( MotionModelIdc[ x0 − 1 ][ y0 + cbHeight − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 + cbHeight ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth ][ y0 − 1 ]] != 0 ) ) |  |
| merge_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type = = B && sps_triangle_enabled_flag && merge_affine_flag[ x0 ][ y0 ] = = 0 && cbWidth + cbHeight > 12 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| if( merge_affine_flag[ x0 ][ y0 ] = = 0 && merge_triangle_flag[ x0 ][ y0 ] = = 0 && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( sps_affine_enabled_flag && cbWidth >= 8 && cbHeight >= 8 && ( MotionModelIdc[ x0 − 1 ][ y0 + cbHeight − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 + cbHeight ] != 0 ¦ ¦ MotionModelIdc[ x0 − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth − 1 ][ y0 − 1 ] != 0 ¦ ¦ MotionModelIdc[ x0 + cbWidth ][ y0 − 1 ]] != 0 ) ) |  |
| merge_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type = = B && sps_triangle_enabled_flag && merge_affine_flag[ x0 ][ y0 ] = = 0 && cbWidth + cbHeight > 12 ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| if( merge_affine_flag[ x0 ][ y0 ] = = 0 && merge_triangle_flag[ x0 ][ y0 ] = = 0 && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL0[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL0[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 0 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 1 ][ 1 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 0 ] = 0 |  |
| MvdCpL1[ x0 ][ y0 ][ 2 ][ 1 ] = 0 |  |
| } else { |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } |  |
| if( sps_amvr_enabled_flag && inter_affine_flag = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) |  |
| amvr_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| } |  |

7.4.5.5	Coding unit semantics

inter_pred_idc[ x0 ][ y0 ] specifies whether list0, list1, or bi-prediction is used for the current coding unit according to Table 7-4. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-4 – Name association to inter prediction mode

| inter_pred_idc | Name of inter_pred_idc |  |
| --- | --- | --- |
|  | ( cbWidth + cbHeight ) != 12 && ( cbWidth + cbHeight ) != 8 && ( cbWidth + cbHeight ) != 20 | ( cbWidth + cbHeight ) = = 12 && ( cbWidth + cbHeight ) = = 8 && ( cbWidth + cbHeight ) = = 20 |
| 0 | PRED_L0 | PRED_L0 |
| 1 | PRED_L1 | PRED_L1 |
| 2 | PRED_BI | n.a. |

When inter_pred_idc[ x0 ][ y0 ] is not present, it is inferred to be equal to PRED_L0.

8.3.2.2	Derivation process for luma motion vectors for merge mode

- The following applies:

- If mergeCandList[ merge_idx[ xCb ][ yCb ] ] is equal to SbCol, the following assignments are made for xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1

refIdxLX = refIdxLXSbCol		(8-119)

predFlagLX[ xSbIdx ][ ySbIdx ] = predFlagLXSbCol[ xSbIdx ][ ySbIdx ]	(8-120)

mvLX[ xSbIdx ][ ySbIdx ][ 0 ] = mvLXSbCol[ xSbIdx ][ ySbIdx ][ 0 ]	(8-121)

mvLX[ xSbIdx ][ ySbIdx ][ 1 ] = mvLXSbCol[ xSbIdx ][ ySbIdx ][ 1 ]	(8-122)

- Otherwise, the following assignments are made with N being the candidate at position merge_idx[ xCb ][ yCb ] in the merging candidate list mergeCandList ( N = mergeCandList[ merge_idx[ xCb ][ yCb ] ] ) and X being replaced by 0 or 1:

numSbX = 1		(8-123)

numSbY = 1		(8-124)

refIdxLX = refIdxLXN		(8-125)

predFlagLX[ 0 ][ 0 ] = predFlagLXN		(8-126)

mvLX[ 0 ][ 0 ][ 0 ] = mvLXN[ 0 ]		(8-127)

mvLX[ 0 ][ 0 ][ 1 ] = mvLXN[ 1 ]		(8-128)

- The following applies when (cbWidth + cbHeight) is equal to 8, 12 or 20:

- If mergeCandList[ merge_idx[ xCb ][ yCb ] ] is not equal to SbCol, and predFlagL0[ xSbIdx ][ ySbIdx ] is equal to 1 and predFlagL1[ xSbIdx ][ ySbIdx ]  is equal to 1, the following assignments are made:

refIdxL1 = -1

predFlagL1[ 0 ][ 0 ] = 0