| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0318 |
| --- | --- |

| Title: | CE7-related: QP prediction and neighbour availability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P. de Lagrange P. Bordes | Email: | philippe.delagrange@technicolor.com philippe.bordes@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes modifications to QP prediction to consider specificities of VVC compared to HEVC. More specifically, it is proposed to modify the derivation of neighbour availability for luma QP prediction: avoid using qPY_PREV when only one neighbor is available and allow using left neighbor on the left edge of a CTU. Corresponding draft text modifications are proposed.

The contribution also discusses further refinement of A (left) and B (top) neighbor selection used for QP prediction.

# Introduction

In current VVC software model and draft text (JVET_L1001), local adjustment of luma quantization parameter (QpY) is possible by specifying a QP offset for a quantization group (QG). A quantization group is defined by a maximum split depth: all blocks resulting from further splits belong to the same quantization group, i.e. share the same QP prediction, and define at most one QP offset.

## Current luma QP prediction

Figure 1 shows an exemplary splitting of a luma CTB, with luma blocks labeled with their coding order. QGs are shown with solid lines and further splits with dashed lines (this example considers diff_cu_qp_delta_depth equals 3).

Considering the QG-10, luma QP prediction makes use of A and B neighbors, as shown in red color in the same figure: A is the block found at the left of the top-left sample of the current QG, and B is the block found just above the top-left sample of the current QG.

Figure 1. QG neighbors

QP prediction, qPY_PRED, is the average of the QPs of block A (qPY_A) and B (qPY_B):

qPY_PRED = ( qPY_A + qPY_B + 1 )  >>  1		(1)

## Unavailability of A or B neighbors

Currently, when A or B falls out of the current CTB, corresponding QP (either qPY_A and/or qPY_B) is replaced by qPY_PREV, which is the QP of the previous coded block, in coding order. For the first luma CTB of slice or tile, this is slice/tile QP.

This can lead to unexpected results on the top or left edge of the CTB, because previous coded block can be far from the current block, as illustrated in Figure 2 with two examples: on the left, B is replaced by block 7 for QG-8 and on the right, A is replaced by block 13 for QG-14. The resulting qPY_PRED is the average of the QP of a close neighbor, which is probably a good predictor (A for QG-8 and B for QG-14), and the QP of a remote block, which is probably not a good predictor (B for QG-8 and A for QG-14, leading to increased bit cost to code the QP offset.

Figure 2. Far-away blocks may be used for QP prediction

# Proposed changes

## Avoid replacing unavailable neighbor with qPY_PREV

Figure 3. Proposed QP prediction on CTB left and top borders.

It is suggested to ignore unavailable neighbor when at least one is available, as previously proposed in JCTVC-H0189. Or replace it by available neighbor, which is equivalent.

Proposed draft text modification follows (section “derivation process for quantization parameters”):

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg − 1, yQg ) as inputs, and the output is assigned to availableA. The variables qPY_A and validA areis derived as follows:

–	If one or more of the following conditions are true, qPY_A is set equal to qPY_PREVmarked as invalid by setting validA to FALSE:

–	availableA is equal to FALSE.

–	the CTB address ctbAddrA of the CTB containing the luma coding block covering the luma location ( xQg − 1, yQg ) is not equal to CtbAddrInTs, where ctbAddrA is derived as follows:

xTmp = ( xQg − 1 )  >>  MinTbLog2SizeY
yTmp = yQg  >>  MinTbLog2SizeY
minTbAddrA = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrA = minTbAddrA  >>  ( 2  * ( CtbLog2SizeY − MinTbLog2SizeY ) )	(8-467)

–	Otherwise, qPY_A is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg − 1, yQg ), and validA is set to TRUE.

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg, yQg − 1 ) as inputs, and the output is assigned to availableB. The variables qPY_B and validB areis derived as follows:

–	If one or more of the following conditions are true, qPY_B is marked as invalid by setting validB to FALSEset equal to qPY_PREV:

–	availableB is equal to FALSE.

–	the CTB address ctbAddrB of the CTB containing the luma coding block covering the luma location ( xQg,  yQg − 1 ) is not equal to CtbAddrInTs, where ctbAddrB is derived as follows:

xTmp = xQg  >>  MinTbLog2SizeY
yTmp = ( yQg − 1 )  >>  MinTbLog2SizeY
minTbAddrB = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrB = minTbAddrB  >>  ( 2 * ( CtbLog2SizeY − MinTbLog2SizeY ) )	(8-468)

–	Otherwise, qPY_B is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg, yQg − 1 ), and validB is set to TRUE.

- The predicted luma quantization parameter qPY_PRED is derived as follows:

–	if both validA and validB are equal to TRUE:

qPY_PRED = ( qPY_A + qPY_B + 1 )  >>  1		(8-469)

–	otherwise, if validA is equal to TRUE:

qPY_PRED = qPY_A		(8-469)

–	otherwise, if validB is equal to TRUE:

qPY_PRED = qPY_B		(8-469)

–	otherwise (both validA and validB are equal to FALSE):

qPY_PRED = qPY_PREV		(8-469)

## Allow use of left neighbor on the left edge of a CTB

In HEVC and current VVC software model and draft text (JVET_L1001), if the left neighbor is outside the CTB, it is marked as unavailable for QP prediction, to limit QP storage to a CTB area.

However, when scanning coding blocks in coding order inside a CTB, it can be observed that blocks used for QP prediction form a vertical and horizontal wavefront, as illustrated in Figure 4. Storage can be limited to ((1 << (CtbLog2SizeY – MinTbLog2SizeY )) + (1 << (CtbLog2SizeX – MinTbLog2SizeX ))) QP values.

Since CTBs are normally scanned in horizontal raster scan order, data from left CTB is readily available, as it is the previous coded one. This would allow re-use of vertical wavefront QP storage (orange in Figure 4) for QP prediction of the next CTB, allowing use of left neighbors on the left edge of a CTB, which is expected to improve QP prediction at nearly no cost.

This is probably not applicable to top neighbors on the top edge of CTBs, because all bottom QPs of the previous line of CTBs would then have to be stored.

Figure 4. Wavefront-style QP storage allowing use of left neighbor on the left edge of a luma CTB

Proposed draft text modification follows (section “derivation process for quantization parameters”; can be combined with the other changes suggested by this proposal):

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg − 1, yQg ) as inputs, and the output is assigned to availableA. The variable qPY_A is derived as follows:

–	If one or more of the following conditions are true, qPY_A is set equal to qPY_PREV:

–	availableA is equal to FALSE.

–	the CTB address ctbAddrA of the CTB containing the luma coding block covering the luma location ( xQg − 1, yQg ) is not equal to CtbAddrInTs, where ctbAddrA is derived as follows:

xTmp = ( xQg − 1 )  >>  MinTbLog2SizeY
yTmp = yQg  >>  MinTbLog2SizeY
minTbAddrA = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrA = minTbAddrA  >>  ( 2  * ( CtbLog2SizeY − MinTbLog2SizeY ) )	(8-467)

–	Otherwise, qPY_A is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg − 1, yQg ).

# Discussion on the selection of A and B neighbors

Quantization Group alignment has changed significantly from HEVC, and for the purpose of QP prediction, neighbors of the top-left sample of the current QG may not be the best option; for example one could consider neighbors of middle-top & middle-left samples, as illustrated in Figure 5, or other more complex rules (block with closest surface, or longest common boundary length, for example).

Figure 5. A and B neighbors of middle-top and middle-left samples

Proposed draft text modification follows, that uses middle-top and middle-left samples (can be combined with the other changes suggested by this proposal):

First, quantization group size should be saved along with its position. Changes are needed in coding_quadtree and multi_type_tree.

| coding_quadtree( x0, y0, log2CbSize, cqtDepth, treeType ) { | Descriptor |
| --- | --- |
| minQtSize = ( treeType = = DUAL_TREE_CHROMA ) ? MinQtSizeC : MinQtSizeY |  |
| maxBtSize = ( treeType = = DUAL_TREE_CHROMA ) ? MaxBtSizeC : MaxBtSizeY |  |
| if( ( ( ( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples ) ? 1 : 0 ) + ( ( y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples ) ? 1 : 0 ) + ( ( ( 1 << log2CbSize ) <= maxBtSize ) ? 1 : 0 ) ) >= 2 && ( 1 << log2CbSize ) > minQtSize ) |  |
| qt_split_cu_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_qp_delta_enabled_flag && cqtDepth <= diff_cu_qp_delta_depth ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| CuQgWidth = 1 << log2CbSize |  |
| CuQgHeight = 1 << log2CbSize |  |
| } |  |
| […] |  |
| } |  |

| multi_type_tree( x0, y0, cbWidth, cbHeight, cqtDepth, mttDepth, depthOffset, partIdx, treeType ) { | Descriptor |
| --- | --- |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ) && ( x0 + cbWidth <= pic_width_in_luma_samples ) && (y0 + cbHeight <= pic_height_in_luma_samples ) ) |  |
| mtt_split_cu_flag | ae(v) |
| if( cu_qp_delta_enabled_flag && ( cqtDepth + mttDepth ) <= diff_cu_qp_delta_depth ) { |  |
| IsCuQpDeltaCoded = 0 |  |
| CuQpDeltaVal = 0 |  |
| CuQgTopLeftX = x0 |  |
| CuQgTopLeftY = y0 |  |
| CuQgWidth = cbWidth |  |
| CuQgHeight = cbHeight |  |
| } |  |
| if( mtt_split_cu_flag ) { |  |

Then, modified neighboring A and B blocks are selected as follows (in “derivation process for quantization parameters”):

[…]

In this process, the luma quantization parameter Qp′Y and the chroma quantization parameters Qp′Cb and Qp′Cr are derived.

The luma location ( xQg, yQg ), specifies the top-left luma sample of the current quantization group relative to the top left luma sample of the current picture. The horizontal and vertical positions xQg and yQg are set equal to CuQgTopLeftX and CuQgTopLeftY, respectively. The luma rectangular size ( wQg, hQg ), specifies the size of the current quantization group. The horizontal and vertical sizes wQg and yQg are set equal to CuQgWidth and CuQgHeight, respectively.

[…]

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg − 1, yQg + hQg / 2 ) as inputs, and the output is assigned to availableA. The variable qPY_A is derived as follows:

–	If one or more of the following conditions are true, qPY_A is set equal to qPY_PREV:

–	availableA is equal to FALSE.

–	the CTB address ctbAddrA of the CTB containing the luma coding block covering the luma location ( xQg − 1, yQg + hQg / 2 ) is not equal to CtbAddrInTs, where ctbAddrA is derived as follows:

xTmp = ( xQg − 1 )  >>  MinTbLog2SizeY
yTmp = ( yQg + hQg / 2 )  >>  MinTbLog2SizeY
minTbAddrA = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrA = minTbAddrA  >>  ( 2  * ( CtbLog2SizeY − MinTbLog2SizeY ) )	(8-467)

–	Otherwise, qPY_A is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg − 1, yQg + hQg / 2 ).

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xQg + wQg / 2, yQg − 1 ) as inputs, and the output is assigned to availableB. The variable qPY_B is derived as follows:

–	If one or more of the following conditions are true, qPY_B is set equal to qPY_PREV:

–	availableB is equal to FALSE.

–	the CTB address ctbAddrB of the CTB containing the luma coding block covering the luma location ( xQg + wQg / 2,  yQg − 1 ) is not equal to CtbAddrInTs, where ctbAddrB is derived as follows:

xTmp = ( xQg + wQg / 2 )  >>  MinTbLog2SizeY
yTmp = ( yQg − 1 )  >>  MinTbLog2SizeY
minTbAddrB = MinTbAddrZs[ xTmp ][ yTmp ]
ctbAddrB = minTbAddrB  >>  ( 2 * ( CtbLog2SizeY − MinTbLog2SizeY ) )	(8-468)

–	Otherwise, qPY_B is set equal to the luma quantization parameter QpY of the coding unit containing the luma coding block covering ( xQg + wQg / 2, yQg − 1 ).

# References

[1]	HEVC standard, ITU-T REC H265. Version 3 (04/2015) available at http://handle.itu.int/11.1002/1000/12455. Pages 37, 51, 57, 79, 104, and 159-162 are relevant here

[2]	B. Bross, J. Chen, S. Liu, "Versatile Video Coding (Draft 3)", JVET-L1001, Macao, CN, October 2018 (version 3 is used here).

[3]	K. Sato, “Non-CE4: Low-complexity dQP Coding”, JCTVC-H0189, San José, CA, USA, February 2012

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).