| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0323 |
| --- | --- |

| Title: | CE13-related: Adaptive QP to improve subjective quality for PHEC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yule Sun Xuchang Huangfu Lu Yu | Tel: Email: | +86 57187953107 sunyule@zju.edu.cn huangfuxc@zju.edu.cn yul@zju.edu.cn |
| Source: | Zhejiang University |  |  |

_____________________________

- Abstract

This contribution proposes a geometry based adaptive QP method together with in-loop filters disabled across face discontinuities and padding (padding of 2 samples around face row) on PHEC. Boundary areas are coded with better quality by using lower QP to further reduce visible artifacts. The experimental results show that the proposed method can significantly improve subjective quality with some coding loss in objective performance.

Introduction

In-loop filters disabled across face discontinuities and padding is same as that in JVET-M0322 [1]. However, the seam artifacts are still visible when coded QP is large near face row boundaries. To further reduce the coding distortion near face boundaries, which can improves consistency of pixel values for spherical neighboring blocks, we propose a geometry based adaptive QP method to use lower QP on area near the face row boundaries. Figure 1 is the illustration of the proposed method for HEC. The adaptive QP is implemented on coding tree unit (CTU) level and enabled on the slice with QP larger than 32. QP offsets of boundary CTUs are derived according to slice QP:

QP_offi=β*(slice QPi-32)4+slice QPi % 4

where x is the floor function which maps x to the greatest integer less than or equal to x and % means modulo operation. β is set to 2 after the tradeoff between objective and subjective performance. Table 1 presents the coded QP of each boundary CTUs according to slice QP in VTM. The CTUs in face row center still use slice QP.

Fig. 1. Illustration of the proposed method for HEC.

Table 1. Coded QP of boundary CTUs according to slice QP in current VTM.

| Slice QP | Coded QP |
| --- | --- |
| 0 - 31 | Slice QP |
| 32 - 35 | 32 |
| 36 - 39 | 34 |
| 40 - 43 | 36 |
| 44 - 47 | 38 |
| 48 - 51 | 40 |
| 52 - 55 | 42 |
| 56 - 59 | 44 |
| 60 - 63 | 46 |

Experimental results

The experiments are conducted according to JVET common test conditions (CTC) [2]. The results of the proposed method are shown below against PHEC and PHEC with in-loop filters disabled (CE test 1.1(b)).

Table II BD-rate using end-to-end WS-PSNR for the proposed method

|  | Anchor: PHEC |  |  | Anchor：CE test 1.1 (b) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| SkateboardInLot | 2.77% | -0.75% | 0.25% | 2.63% | -0.26% | -0.13% |
| ChairLift | 2.32% | -2.80% | -2.15% | 2.39% | -2.69% | -2.06% |
| KiteFlite | 0.65% | -4.09% | -4.04% | 0.68% | -4.11% | -3.92% |
| Harbor | 1.32% | -5.09% | -4.71% | 1.42% | -4.79% | -4.64% |
| Trolley | 1.42% | -3.03% | -3.19% | 1.46% | -2.92% | -2.96% |
| GasLamp | 0.37% | -6.11% | -5.60% | 0.44% | -5.96% | -5.43% |
| Balboa | 3.80% | 1.81% | 2.65% | 3.75% | 2.26% | 2.56% |
| Broadway | 3.84% | 1.29% | 1.15% | 3.77% | 1.40% | 1.25% |
| Landing2 | 3.37% | -1.91% | 0.37% | 3.41% | -0.56% | 1.39% |
| BranCastle2 | 4.06% | 1.52% | 2.23% | 4.05% | 1.33% | 2.12% |
| Class S1 | 1.48% | -3.65% | -3.24% | 1.50% | -3.46% | -3.19% |
| Class S2 | 3.77% | 0.68% | 1.60% | 3.75% | 1.11% | 1.83% |
| Overall | 2.39% | -1.92% | -1.31% | 2.40% | -1.63% | -1.18% |

Conclusion

In this contribution, we propose a geometry based adaptive QP method with in-loop filters disabled across face discontinuities and padding to improve subjective quality of HEC. The proposed method significantly improves subjective quality while it has 2.40% coding loss compared to PHEC with in-loop filters disabled across face discontinuities.

Reference

- Y. Sun, X. Huangfu and L. Yu, “CE13-related: In-loop filters disabled across face discontinuities on PHEC with 2-pixel padding,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0322, Jan. 2019.

- P. Hanhart, J. Boyce, K. Choi and J. Lin, “JVET common test conditions and evaluation procedures for 360° video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1012, Oct. 2018.

Patent rights declaration(s)

Zhejiang University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).