| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0330_v3 |
| --- | --- |

| Title: | CE4-related: Simplification of candidate list derivation for MMVD mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liying Xu Fangdong Chen Li Wang | Tel: Email: | 18100191186 xuliying5@hikvision.com chenfangdong@hikvision.com wangli7@hikvision.com |
| Source: | HIKVISION |  |  |

_____________________________

- Abstract

This contribution presents a simplified MMVD (Merge with MVD) scheme with one base candidate. MMVD is a motion vector expression method for skip and merge modes which has been adopted. Based on the MMVD scheme, in this contribution, three simplifications are conducted. (1) Reduce the base candidate from two to one, thus the base candidate index is not signaled. (2) Use a reordered merge candidate list to provide a better base candidate. (3) Combination of the CE4.4.4 which tries to optimize the MVD coding. Here, 5 tests are conducted:

Test a: 1 base candidate

RA: 0.25%	0.20%	0.22%	93%	98%

LB: 0.07%	0.01%	0.04%	98%	100%

Test b: 1 base candidate and reordered merge candidate list

RA: 0.07%	0.05%	0.05%	94%	98%

LB: -0.11%	-0.18%	-0.21%	98%	100%

Test c: 2 base candidate and reordered merge candidate list

RA: -0.06%	-0.05%	-0.04%	100%	101%

LB: -0.12%	-0.32%	-0.14%	101%	100%

Test d: 1 base candidate and CE4.4.4 Test c

RA: 0.04%	0.05%	0.03%	99%	100%

LB: 0.11%	0.18%	0.09%	101%	100%

Test e: 1 base candidate and CE4.4.4 Test c and reordered merge candidate

RA: -0.19%	-0.12%	-0.14%	99%	101%

LB: -0.12%	-0.09%	0.07%	102%	100%

Introduction of MMVD scheme

In this section, MMVD scheme [1] is introduced. MMVD is used for either skip or merge modes with a proposed motion vector expression method, and reuses the same merge candidate as that in VVC. Among the merge candidates, a candidate can be selected and further expanded by the proposed motion vector expression method. MMVD provides a new motion vector expression with simplified signaling. The expression method includes starting point, motion magnitude, and motion direction.

-

Figure 1-1 MMVD Search Point

Base candidate index indicates the best candidate among candidates in the list as follows. If the number of base candidate is equal to 1, the index is not signaled. Currently, two base candidates are used in VTM 3.0.

Table 1. Base candidate IDX

| Base candidate IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Nth MVP | 1st MVP | 2nd MVP | 3rd MVP | 4th MVP |

Distance index is motion magnitude information. Distance index indicates the pre-defined distance from the starting point information, which is as follows:

Table 2. Distance IDX

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Direction index represents the direction of the MVD relative to the starting point. The direction index can represent the four directions as shown below.

Table 3. Direction IDX

| Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-axis | + | – | N/A | N/A |
| y-axis | N/A | N/A | + | – |

Based on the different base candidate number and distance and direction, different candidate number will be added. And these different numbers are given in Table 4.

Table 4. Candidate number for MMVD

| Base candidate number | Added Candidate number for MMVD |
| --- | --- |
| 1 | 1x8x4=32 |
| 2 | 2x8x4=64 |
| 3 | 3x8x4=96 |
| 4 | 4x8x4=128 |

Introduction of CE4.4.4 scheme

In CE4.4.4 test c, two modifications are conducted.

- Directional candidates

Directional candidates are used in addition to horizontal/vertical candidates in VTM-3.0. The directional candidates are shown as below.

Table 5. Direction IDX for CE4.4.4 C

| Direction IDX | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-axis | +1 | –1 | 0 | 0 | +1/2 | -1/2 | -1/2 | +1/2 |
| y-axis | 0 | 0 | +1 | –1 | +1/2 | -1/2 | +1/2 | -1/2 |

Note: the value of x-axis and y-axis of diagonal direction are half of that of horizontal and vertical direction respectively.

(2) Multiple sets of the distance lists

Considering the amount of distance (motion difference) would be different from the area and sequence characteristics, two sets of distance list are designed in this method, which are given in Table 6 and Table 7, respectively.

Table 6. First distance list

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 3/4-pel | 5/4-pel |

Table 7. Second distance list

| Distance IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Pixel distance | 1-pel | 2-pel | 4-pel | 8-pel |

The proposed simplification methods

Part A

To decrease the complexity, it is proposed to reduce the base candidate number from two to one, thus the base candidate index is not signaled. In this case, base candidate index require no parse process in the decoder.

Part B

To provide a better base candidate with only one base candidate, it is proposed to reorder the spatial merge candidates as shown in Table 8.

Table 8. The proposed candidate order for Merge mode

| Width>= height | B1A1A0B0 |
| --- | --- |
| Width<height | A1B1B0A0 |

The final scheme

As a result, the final scheme is concluded as follows.

1) The number of base candidate is equal to 1.

2) Merge candidates are reordered as Table 8.

3) The direction and distance keep unchanged as the original MMVD scheme.

In this case, the total candidate number is reduced to 1x8x4=32.

Combination with CE4.4.4

Moreover, the combination the proposed simplification scheme with CE4.4.4 [2] (Test C) is also tested. The combined scheme is concluded as follows.

Combination with CE 4.4.4 Test C

1) The number of base candidate is equal to 1, base candidate index is not signaled.

2) Merge candidates are reordered.

3) Multiple set of the distance lists and Directional candidates is used.

Therefore, the total candidate number still 64 (1x8x8), the same as which is used in the current adopted scheme.

Complexity Analysis

In summary, the candidate number of different tests are given in Table 9.

Table 9. Summary of candidate number for MMVD

|  | Candidate number for MMVD |
| --- | --- |
| 2 base (VTM3.0) | 2x8x4=64 |
| 1 base (VTM3.0) | 1x8x4=32 |
| 1 base + CLR | 1x8x4=32 |
| 2 base + CE444(Test C) | 2x8x8=128 |
| 1 base + CE444(Test C) | 1x8x8=64 |
| 1 base + CE444(Test C) + CLR | 1x8x8=64 |

Experimental results

The simulation is carried out under the JVET common test condition [2]. Five tests are conducted:

Test a: 1 base candidate

Test b: 1 base candidate and reordered merge candidate list

Test c: 2 base candidate and reordered merge candidate list

Test d: 1 base candidate and CE4.4.4 Test c

Test e: 1 base candidate and CE4.4.4 Test c and reordered merge candidate

All the test results of test a-e are given in the following tables.

Table 10. Results for test a

Table 11. Results for test b

Table 12. Results for test c

Table 13. Results for test d

Table 14. Results for test e

Conclusion

In this contribution, a simplification scheme of MMVD is proposed. The proposed scheme will reduce the encoder complexity while keeping the most coding gains. Moreover, this method can bring additional coding gains for LB configuration.

In addition, in the combination scheme with CE4.4.4, the combined scheme can increase the coding gain without increasing any coding time. It is recommended to be adopted in VTM or further studied in the CE.

References

[1] “CE4 Ultimate motion vector expression (Test 4.5.4)”, JVET_L0054, Joint Video Experts Team (JVET)

of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, 3–12 Oct. 2018

[2] “Non-CE4: Enhanced ultimate motion vector expression”, JVET_L0355, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, , Macao, CN, 3–12 Oct. 2018

;

[3] “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).