| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0333r1 |
| --- | --- |

| Title: | Non-CE8: Coding on block vector difference |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Jaehyun Lim Seung Hwan Kim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Tel: Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a block vector difference coding for CPR mode. Considering that the absolute value of block vector difference (i.e., abs_mvd_minus2) for CPR mode is larger than that of conventional motion vector difference, higher order (K is equal to 3) of Exp-Golomb code is applied. From experiment results, following luma BD-rate changes are observed over VTM-3.0 with CPR:

- AI: -0.02% (CTC), -0.19% (Class F), -0.41% (SCC)

- RA: 0.00% (CTC), -0.27% (Class F), -0.10% (SCC)

- LD: 0.00% (CTC), -0.15% (Class F), -0.76% (SCC)

Introduction

In the last meeting, CPR mode with search range that is current CTU was adopted to VTM. Currently block vector difference (BVD) for CPR mode is signaled by using motion vector difference (MVD) coding syntax which consists of four parts; abs_mvd_greater0_flag[], abs_mvd_greater1_flag[], abs_mvd_minus2[], mvd_sign_flag[]. However, as the search range constraint, not only block vector (BV) but also BVD could be bounded by CTU size. In this contribution, a method about BVD coding is introduced.

Proposed method

This contribution proposes a coding method of BVD when current coding block is coded by CPR mode. Binarization process of abs_mvd_minus2 syntax, which is signalled when the absolute value of a motion vector component difference is greater than 1, is changed. For abs_mvd_minus2 syntax, which is not in CPR mode, Exp-Golomb order K equal to 1 is used to code the syntax. However, for CPR mode, abs_mvd_minus2 is coded with Exp-Golomb order K equal to 3 to reflect relatively large value of abs_mvd_minus2 in CPR mode.

Experimental results

The proposed methods have been implemented on top of the CE8 software based on VTM-3.0 and tested under common test conditions in [1].

Table 1. Performance of proposed method with CPR off in AI

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.13% | 0.04% | 129% | 102% |
| Class A2 | -0.49% | -0.54% | -0.57% | 132% | 103% |
| Class B | -0.30% | -0.43% | -0.41% | 139% | 104% |
| Class C | -0.10% | -0.25% | -0.29% | 143% | 108% |
| Class E | -0.69% | -0.79% | -0.70% | 143% | 105% |
| Overall | -0.29% | -0.42% | -0.38% | 137% | 105% |
| Class D | -0.14% | -0.16% | 0.20% | 139% | 106% |
| Class F | -12.23% | -12.17% | -12.25% | 158% | 105% |
| Class SCC | -37.20% | -36.34% | -36.60% | 111% | 89% |

Table 2. Performance of proposed method with CPR off in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.02% | 0.32% | 100% | 101% |
| Class A2 | 0.00% | 0.25% | 0.15% | 98% | 102% |
| Class B | 0.02% | -0.12% | -0.30% | 100% | 101% |
| Class C | 0.20% | -0.20% | 0.28% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -0.04% | 0.07% | 100% | 102% |
| Class D | 0.14% | 0.08% | 0.17% | 103% | 103% |
| Class F | -9.94% | -9.92% | -10.08% | 108% | 102% |
| Class SCC | -22.13% | -21.64% | -22.19% | 104% | 100% |

Table 3. Performance of proposed method with CPR off in LDB

|  |  |  | LDB Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.12% | 0.30% | 107% | 102% |
| Class C | 0.13% | 0.35% | 0.40% | 111% | 103% |
| Class E | 0.10% | 1.13% | -0.20% | 106% | 102% |
| Overall | 0.10% | 0.35% | 0.21% | 108% | 102% |
| Class D | 0.13% | 0.72% | 0.17% | 113% | 105% |
| Class F | -5.21% | -5.11% | -4.87% | 109% | 103% |
| Class SCC | -13.10% | -13.34% | -13.60% | 110% | 101% |

Table 4. Performance of proposed method with CPR on in AI

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.01% | 102% | 102% |
| Class A2 | -0.02% | -0.03% | -0.02% | 102% | 103% |
| Class B | -0.01% | -0.04% | -0.05% | 102% | 103% |
| Class C | -0.01% | -0.01% | -0.04% | 102% | 108% |
| Class E | -0.05% | 0.02% | -0.11% | 102% | 105% |
| Overall | -0.02% | -0.02% | -0.05% | 102% | 104% |
| Class D | -0.04% | 0.07% | -0.02% | 102% | 105% |
| Class F | -0.19% | -0.18% | -0.20% | 102% | 107% |
| Class SCC | -0.41% | -0.33% | -0.35% | 101% | 107% |

Table 5. Performance of proposed method with CPR on in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.07% | 0.10% | 101% | 101% |
| Class A2 | -0.01% | 0.18% | -0.09% | 101% | 102% |
| Class B | 0.00% | -0.12% | -0.09% | 101% | 101% |
| Class C | 0.01% | -0.14% | 0.20% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | 0.02% | 101% | 102% |
| Class D | -0.02% | 0.36% | 0.01% | 101% | 104% |
| Class F | -0.08% | -0.04% | -0.17% | 101% | 103% |
| Class SCC | -0.10% | -0.06% | -0.09% | 101% | 104% |

Table 6. Performance of proposed method with CPR on in LDB

|  |  |  | LDB Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-CPR on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.20% | 0.43% | 101% | 101% |
| Class C | 0.00% | -0.04% | 0.07% | 101% | 103% |
| Class E | -0.18% | 0.09% | -0.03% | 101% | 101% |
| Overall | 0.00% | 0.09% | 0.20% | 101% | 102% |
| Class D | 0.00% | 0.89% | -0.51% | 101% | 104% |
| Class F | -0.09% | 0.26% | 0.62% | 101% | 103% |
| Class SCC | -0.01% | 0.08% | 0.03% | 101% | 104% |

Conclusion

In this contribution, the coding method on block vector difference are proposed. It is recommended to study the block vector coding for CPR mode in CE.

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET- L1010, 12th meeting, Macau, CN, 08–12 Oct. 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).