| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0339 |
| --- | --- |

| Title: | CE11-related: subblock boundary filter at 8x8 Grid |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Seung-Hwan Kim Jayhyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a modification on subblock boundary filtering on 8x8 grid. In order to filter all subblock boundaries on 8x8 grid, subblock boundaries where the current PU is not aligned with 8x8 grid are also filtered in the proposed method, 4x4 block boundaries for affine and 8x8 block boundaries for SbTMVP. The proposed method shows -0.01% BD-rate change for RA configuration and also gives subjective quality improvement on subblock boundaries.

- Problem Statement

At 12th JVET meeting, Deblock filtering on subblock boundaries was proposed by JVET-L0074[1]. From this proposal subblock boundaries are filtered when current PU is aligned at 8x8 grid. However still it is not filtered that subblock boundaries are aligned 8x8 grid but current PU is not at 8x8 grid. For example affine PU is not aligned at 8x8 grid as shown in Figure 1.(a). all of the block boundaries are not filtered even though some subblock boundaries are aligned at 8x8 grid. It decreases subjective quality on subblock boundaries.

Figure 1. Example of the problem statement

Proposed method

The proposed method suggests that all of subblock boundaries on 8x8 grid is filtered. To support this method, affine subblock boundaries are derived based on 4x4 affine subblock size when the current PU is coded as Affine and 8x8 SbTMVP subblock size when the current block is coded as SbTMVP. In order to filter subblock boundaries only at 8x8 grid, only subblock boundary which is aligned at 8x8 grid is filtered.

Experimental result

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.03% | 100% | 100% |
| Class A2 | -0.03% | -0.05% | 0.02% | 100% | 100% |
| Class B | -0.01% | 0.03% | -0.04% | 100% | 100% |
| Class C | 0.00% | -0.01% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | -0.02% | 100% | 101% |
| Class D | 0.00% | 0.04% | 0.03% | 100% | 99% |
| Class F | -0.01% | -0.02% | -0.01% | 100% | 100% |

Reference

- K. Andersson, Z. Zhang, R. Sjöberg, “CE11: Deblocking of sub-block boundaries for luma” JVET-L0074, Macao, CN, October 2018.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).