| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0354 |
| --- | --- |

| Title: | CE6-related: MTS with Haar transform for Screen Contents Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Franck GALPIN Tangi POIRIER | Tel: Email: | karam.naser@technicolor.com franck.galpin@technicolor.com tangi.poirier@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution proposes to introduce Haar transform inside the multiple transform selection for the coding of the screen contents. It is motivated by the fact that the basis functions of Haar transform well matches the properties of the screen contents, where blocks with constant values and sharp edges frequently occurs. Specifically, the DCT8 transform is replaced by Haar transform. The proposal is tested with CTC for class F and TGM. For further analysis, CPR mode was also enabled to study the cross-effect between them. The experimental results show the average BD-rate saving as follows (AI%, RA%, and LDB%):

- Class F (0.46, 0.35, 0.32)

- Class TGM (1.34, 0.83, xx)

When CPR is enabled: (AI%, RA% and LDB%):

- Class F (0.25, 0.10, xx)

- Class TGM (0.69, 0.45, xx)

# Introduction

Haar transform is a simple transform that requires only additions and subtractions for its computation. Its basis functions are characterized by being constant with sharp edges (see Figure 1). Due to these properties, Haar transform is often proposed as a core transform for residual signals that have the same properties. For example, it was used for the coding of 3D depth maps residuals in [2] and also for the screen contents in [1].

This contribution proposes to include the Haar transform in the MTS set of VTM software. Extensive experiments show the effectiveness of the proposed method.

Figure 1 2D basis functions of Haar transform

# Proposed methods

The current design of MTS in the VTM software consists of the DST7 and DCT8 transforms. These transforms, in addition to the core transform of DCT2, well matches the properties of a typical residual signal. However, the Haar transform well fits the properties of screen contents residual. Experimentally, it is found that the best embedding of the Haar transform is achieved by placing it in lieu of DCT8. By doing so, the current transform pairs (Horizontal, Vertical) in MTS:

(DST7, DST7), (DST7, DCT8), (DCT8, DST7), (DCT8, DCT8)

Are replaced by:

(DST7, DST7), (DST, Haar), (Haar, DST7), (Haar, Haar)

# Simulation results

The proposed method is implemented in VTM3.0 software. Two test sequences are considered: Class F and TGM. Since Current Picture Referencing (CPR) may influence the results, both conditions CPR=0 and CPR=1 are studied.

## CPR disabled

The results of Class F are provided in Table 1, Table 2 and Table 3 for random access, all intra and low delay B testing conditions. Class TGM results are provided in Table 4 and Table 5.

Table 1 Results of class F on RA CTC

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| BasketballDrillText | 0.08% | -0.26% | -0.13% |  |  |
| ArenaOfValor | 0.11% | 0.17% | -0.05% |  |  |
| SlideEditing | -1.23% | -0.66% | -0.57% |  |  |
| SlideShow | -0.34% | 0.08% | 0.42% |  |  |
| Average | -0.35% | -0.17% | -0.08% | 101% | 100% |

Table 2 Results of class F on AI CTC

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| BasketballDrillText | -0.03% | -0.02% | -0.15% |  |  |
| ArenaOfValor | -0.02% | -0.09% | -0.03% |  |  |
| SlideEditing | -1.30% | -0.62% | -0.64% |  |  |
| SlideShow | -0.49% | -0.36% | 0.19% |  |  |
| Average | -0.46% | -0.27% | -0.16% | 102% | 99% |

Table 3 Results of class F on LDB CTC

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| BasketballDrillText | 0.12% | -0.26% | 0.02% |  |  |
| ArenaOfValor | -0.03% | 0.03% | 0.01% |  |  |
| SlideEditing | -0.94% | -0.23% | -0.51% |  |  |
| SlideShow | -0.44% | 1.36% | 0.23% |  |  |
| Average | -0.32% | 0.22% | -0.06% | 101% | 107% |

Table 4 Results of class TGM on RA CTC

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| sc_flyingGraphics | -0.32% | -0.23% | -0.34% |  |  |
| sc_console | -0.86% | -0.38% | -0.35% |  |  |
| sc_desktop | -0.75% | -0.31% | -0.30% |  |  |
| ChineseEditing | -1.40% | -0.69% | -0.69% |  |  |
| Average | -0.83% | -0.40% | -0.42% | 103% | 101% |

Table 5 Results of class TGM on AI CTC

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| sc_flyingGraphics | -1.00% | -0.40% | -0.35% |  |  |
| sc_console | -1.61% | -0.63% | -0.63% |  |  |
| sc_desktop | -1.06% | -0.48% | -0.37% |  |  |
| ChineseEditing | -1.67% | -0.73% | -0.76% |  |  |
| Average | -1.34% | -0.56% | -0.53% | 102% | 103% |

## CPR Enabled

The following results are generated by setting the encoder option: --CPR=1. Class F results are provided in Table 6 and Table 7 whereas class TGM results are in Table 8 and Table 9.

Table 6 Results of class F on RA CTC, with CPR=1

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| BasketballDrillText | 0.01% | -0.52% | -0.25% |  |  |
| ArenaOfValor | 0.12% | 0.00% | 0.00% |  |  |
| SlideEditing | -0.55% | -0.18% | -0.23% |  |  |
| SlideShow | 0.02% | -0.09% | 0.03% |  |  |
| Average | -0.10% | -0.20% | -0.11% | 102% | 106% |

Table 7 Results of class F on AI CTC, with CPR=1

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| BasketballDrillText | -0.05% | -0.13% | -0.17% |  |  |
| ArenaOfValor | -0.03% | -0.09% | -0.05% |  |  |
| SlideEditing | -0.59% | -0.19% | -0.23% |  |  |
| SlideShow | -0.35% | -0.46% | 0.13% |  |  |
| Average | -0.25% | -0.22% | -0.08% | 103% | 109% |

Table 8 Results of class TGM on RA CTC, with CPR=1

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| sc_flyingGraphics | -0.22% | -0.05% | -0.16% |  |  |
| sc_console | -0.54% | -0.30% | -0.16% |  |  |
| sc_desktop | -0.46% | -0.19% | -0.19% |  |  |
| ChineseEditing | -0.46% | -0.19% | -0.19% |  |  |
| Average | -0.45% | -0.19% | -0.16% | 104% | 105% |

Table 9 Results of class TGM on AI CTC, with CPR=1

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| sc_flyingGraphics | -0.48% | -0.12% | -0.07% |  |  |
| sc_console | -1.08% | -0.38% | -0.45% |  |  |
| sc_desktop | -0.54% | -0.13% | -0.22% |  |  |
| ChineseEditing | -0.66% | -0.14% | -0.20% |  |  |
| Average | -0.69% | -0.19% | -0.24% | 108% | 101% |

# Conclusion

This contribution proposed to embed Haar transform for the screen contents coding by replacing the existing DCT8 transform in the MTS set. Significant coding gains have been shown for both class F and TGM sequences. In addition, the Haar transform is much simpler to implement as it requires only additions and subtractions.

# References

- Chen, Cheng, et al. "A staircase transform coding scheme for screen content video coding." Image Processing (ICIP), 2016 IEEE International Conference on. IEEE, 2016.

- Jager, Fabian, and Karam Naser. "Low complexity transform coding for depth maps in 3D video." Visual Communications and Image Processing (VCIP), 2013. IEEE, 2013.

# Patent rights declaration(s)

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).