| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Marrakesh, MA, 9–18 January 2019 | Document: JVET-M0377-v2 |
| --- | --- |

| Title: | AHG17: Picture header NAL unit type |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Mitra Damghanian Martin Pettersson | Email: | rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes a picture header network abstraction layer (NAL) unit with a dedicated NAL unit type PIC_NUT and a payload consisting of syntax elements from the current tile group header.

The main idea is to put syntax elements that are common for multiple tile groups in a single picture into a separate syntax table called common_tile_group_header(). It is proposed to add an SPS flag to control whether the syntax table elements will be carried in a picture header NAL unit sent in every AU in the CVS or carried in the tile group header of each tile group NAL unit in the CVS.

The proposed design contains the following elements:

- A new NAL unit type PIC_NUT with picture_header_rbsp() payload.

- A new syntax table, common_tile_group_header() consisting of syntax elements from the current tile group header.

- A new SPS flag, sps_picture_header_enabled_flag, that specifies whether PIC_NUT NAL units are present in the CVS or not. The flag may be exposed as a tool on/off indicator.

- If the flag is equal to 1, each access unit (AU) in the CVS shall contain a PIC_NUT NAL unit carrying common_tile_group_header() syntax and there shall be no common_tile_group_header() syntax in the tile group header.

- If the flag is equal to 0, there shall be no PIC_NUT NAL unit in the CVS. Each tile group header in the CVS shall then include the common_tile_group_header() syntax.

The proponents claim that the proposed design reduces the header overhead for use cases where each picture consists of a substantial number of tile groups.

The proponents also claim that the picture header design enables detection of lost picture header NAL units (since each AU shall contain one if they are enabled).

Version 2 of this contribution contains proposed syntax and semantics on top of the VVC draft 3 (JVET-L1001-v9).

# Introduction

At the 12th JVET meeting in Macau the idea of using picture headers was discussed. The conclusion as recorded in the meeting notes was to use a PPS for now and let the picture header topic as a substitute or additional header be for further study. It was also noted that whether to have some picture header is primarily a coding efficiency issue.

One motivation for using a picture header is to reduce the header overhead for the 360-video MCTS use case where each picture consists of a substantial number of tile groups. In an error-free environment, the repetition of identical header information in each tile group is not required but can add up to a noticeable part of the total bit-rate.

We believe the following requirements related to picture headers are important:

- Substantial bitrate savings should be achieved

- Error robustness and NAL unit loss detection should be provided in any reasonable encoder configuration unless that comes at a substantial bit cost

This contribution proposes a solution that meets these requirements.

# Proposal

We propose a picture header NAL unit with its own NAL unit type PIC_NUT and a payload consisting of syntax elements from the current tile group header.

The main idea is to put syntax elements that are common for multiple tile groups in a picture into a separate syntax table called common_tile_group_header(). We further propose to add an SPS flag to control whether there will be a PIC_NUT in every AU or no PIC_NUT at all in the CVS.

The proposed design contains the following elements:

- A new NAL unit type PIC_NUT with picture_header_rbsp() payload.

- A new syntax table, common_tile_group_header() consisting of syntax elements from the current tile group header

- A new SPS flag, sps_picture_header_enabled_flag, that specifies whether PIC_NUT NAL units are present in the CVS or not. The flag may be exposed as a tool on/off indicator

- If the flag is equal to 1, each access unit (AU) in the CVS shall contain a PIC_NUT NAL unit carrying common_tile_group_header() syntax and there shall be no common_tile_group_header() syntax in the tile group header

- If the flag is equal to 0, there shall be no PIC_NUT NAL unit in the CVS. The tile group header shall then carry the common_tile_group_header() syntax.

Table 1 below shows syntax elements that are proposed to be included in common_tile_group_header() and thereby be carried in the picture header.

Table 1 – Proposed tile group header syntax element categorization

| tile_group_header( ) { | Descriptor | Tile group header | Picture header |
| --- | --- | --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |  | X |
| if( NumTilesInPic > 1 ) { |  |  |  |
| tile_group_address | u(v) | X |  |
| num_tiles_in_tile_group_minus1 | ue(v) | X |  |
| } |  |  |  |
| tile_group_type | ue(v) |  | X |
| tile_group_pic_order_cnt_lsb | u(v) |  | X |
| if( partition_constraints_override_enabled_flag ) { |  |  |  |
| partition_constraints_override_flag | ue(v) |  | X |
| if( partition_constraints_override_flag ) { |  |  |  |
| tile_group_log2_diff_min_qt_min_cb_luma | ue(v) |  | X |
| tile_group_max_mtt_hierarchy_depth_luma | ue(v) |  | X |
| if( tile_group_max_mtt_hierarchy_depth_luma != 0 ) |  |  |  |
| tile_group_log2_diff_max_bt_min_qt_luma | ue(v) |  | X |
| tile_group_log2_diff_max_tt_min_qt_luma | ue(v) |  | X |
| } |  |  |  |
| if( tile_group_type = = I && qtbtt_dual_tree_intra_flag ) { |  |  |  |
| tile_group_log2_diff_min_qt_min_cb_chroma | ue(v) |  | X |
| tile_group_max_mtt_hierarchy_depth_chroma | ue(v) |  | X |
| if( tile_group_max_mtt_hierarchy_depth_chroma != 0 ) |  |  |  |
| tile_group_log2_diff_max_bt_min_qt_chroma | ue(v) |  | X |
| tile_group_log2_diff_max_tt_min_qt_chroma | ue(v) |  | X |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| if ( tile_group_type != I ) { |  |  |  |
| if( sps_temporal_mvp_enabled_flag ) |  |  |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |  | X |
| if( tile_group_type = = B ) |  |  |  |
| mvd_l1_zero_flag | u(1) |  | X |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |  |  |
| if( tile_group_type = = B ) |  |  |  |
| collocated_from_l0_flag | u(1) |  | X |
| } |  |  |  |
| six_minus_max_num_merge_cand | ue(v) |  | X |
| if( sps_affine_enable_flag ) |  |  |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |  | X |
| } |  |  |  |
| tile_group_qp_delta | se(v) |  | X |
| if( pps_tile_group_chroma_qp_offsets_present_flag ) { |  |  |  |
| tile_group_cb_qp_offset | se(v) |  | X |
| tile_group_cr_qp_offset | se(v) |  | X |
| } |  |  |  |
| if( sps_sao_enabled_flag ) { |  |  |  |
| tile_group_sao_luma_flag | u(1) |  | X |
| if( ChromaArrayType != 0 ) |  |  |  |
| tile_group_sao_chroma_flag | u(1) |  | X |
| } |  |  |  |
| if( sps_alf_enabled_flag ) { |  |  |  |
| tile_group_alf_enabled_flag | u(1) |  | X |
| if( tile_group_alf_enabled_flag ) |  |  |  |
| alf_data( ) |  |  | X |
| } |  |  |  |
| if( tile_group_type = = P ¦ ¦ tile_group_type = = B ) { |  |  |  |
| num_ref_idx_l0_active_minus1 | ue(v) |  | X |
| if( tile_group_type = = B ) |  |  |  |
| num_ref_idx_l1_active_minus1 | ue(v) |  | X |
| } |  |  |  |
| dep_quant_enabled_flag | u(1) |  | X |
| if( !dep_quant_enabled_flag ) |  |  |  |
| sign_data_hiding_enabled_flag | u(1) |  | X |
| if( deblocking_filter_override_enabled_flag ) |  |  |  |
| deblocking_filter_override_flag | u(1) |  | X |
| if( deblocking_filter_override_flag ) { |  |  |  |
| tile_group_deblocking_filter_disabled_flag | u(1) |  | X |
| if( !tile_group_deblocking_filter_disabled_flag ) { |  |  |  |
| tile_group_beta_offset_div2 | se(v) |  | X |
| tile_group_tc_offset_div2 | se(v) |  | X |
| } |  |  |  |
| } |  |  |  |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |  |  |
| offset_len_minus1 | ue(v) | X |  |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |  |  |
| entry_point_offset_minus1[ i ] | u(v) | X |  |
| } |  |  |  |
| byte_alignment( ) |  | X | X |
| } |  |  |  |

# Proposed syntax and semantics

This section contains proposed syntax and semantics relative to JVET-L1001-v9.

Inserted text is shown by red text and removed text is shown by red strikethrough text.

## Proposed NAL unit types

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | NON_IRAP_NUT | Coded tile group of a non-IRAP picture tile_group_layer_rbsp( ) | VCL |
| 1 | IRAP_NUT | Coded tile group of an IRAP picture tile_group_layer_rbsp( ) | VCL |
| 2-15 | RSV_VCL_NUT | Reserved VCL NAL Units | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20, 21 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 22 | PIC_NUT | Picture header picture_header_rbsp() | non-VCL |
| 23-26 | RSV_NVCL | Reserved | non-VCL |
| 27-31 | UNSPEC | Unspecified | non-VCL |

A picture header NAL unit shall preceded all VCL NAL units in the access unit.

## Proposed picture header RBSP syntax and semantics

| picture_header_rbsp ( ) { | Descriptor |
| --- | --- |
| pic_header_pic_parameter_set_id | ue(v) |
| common_tile_group_header() |  |
| byte_alignment( ) |  |
| } |  |

The semantics of pic_header_pic_parameter_set_id is similar to the tile_group_pic_parameter_set_id text in JVET-L1001-v9:

pic_header_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the PPS in use. The value of pic_header_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

## Proposed SPS syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| sps_picture_header_enabled_flag | u(1) |
| ... |  |
| } |  |

sps_picture_header_enabled_flag equal to 1 specifies that one picture header NAL unit shall be present in all access units in the CVS. sps_picture_header_enabled_flag equal to 0 specifies that there shall be no picture header NAL units in the CVS.

## Proposed common syntax and semantics

The proposed common syntax is shown on top of the current tile_group_header() syntax. As shown, the syntax consists of all tile group header syntax except tile group address and tile pointer syntax. Note that pic_order_cnt_lsb was added since that syntax element was adopted in Macau.

| tile_group_header( ) { | Descriptor |
| --- | --- |
| common_tile_group_header() { | Descriptor |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| tile_group_type | ue(v) |
| tile_group_pic_order_cnt_lsb | u(v) |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | ue(v) |
| if( partition_constraints_override_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_luma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_luma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_luma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_luma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_luma | ue(v) |
| } |  |
| if( tile_group_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| tile_group_log2_diff_min_qt_min_cb_chroma | ue(v) |
| tile_group_max_mtt_hierarchy_depth_chroma | ue(v) |
| if( tile_group_max_mtt_hierarchy_depth_chroma != 0 ) |  |
| tile_group_log2_diff_max_bt_min_qt_chroma | ue(v) |
| tile_group_log2_diff_max_tt_min_qt_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( tile_group_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| tile_group_temporal_mvp_enabled_flag | u(1) |
| if( tile_group_type = = B ) |  |
| mvd_l1_zero_flag | u(1) |
| if( tile_group_temporal_mvp_enabled_flag ) { |  |
| if( tile_group_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| } |  |
| six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enable_flag ) |  |
| five_minus_max_num_subblock_merge_cand | ue(v) |
| } |  |
| tile_group_qp_delta | se(v) |
| if( pps_tile_group_chroma_qp_offsets_present_flag ) { |  |
| tile_group_cb_qp_offset | se(v) |
| tile_group_cr_qp_offset | se(v) |
| } |  |
| if( sps_sao_enabled_flag ) { |  |
| tile_group_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| tile_group_sao_chroma_flag | u(1) |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| tile_group_alf_enabled_flag | u(1) |
| if( tile_group_alf_enabled_flag ) |  |
| alf_data( ) |  |
| } |  |
| if( tile_group_type = = P ¦ ¦ tile_group_type = = B ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( tile_group_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| dep_quant_enabled_flag | u(1) |
| if( !dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| tile_group_deblocking_filter_disabled_flag | u(1) |
| if( !tile_group_deblocking_filter_disabled_flag ) { |  |
| tile_group_beta_offset_div2 | se(v) |
| tile_group_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

The semantics of all syntax elements in common_tile_group_header() is identical to the syntax elements in tile_group_header() in JVET-L1001-v9.

## Proposed tile group header syntax and semantics

Syntax in the tile group header of JVET-L1001-v9 is shown in black and syntax not present in the tile group header of JVET-L1001-v9 is shown in red.

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| if( !sps_picture_header_enabled_flag ) |  |
| common_tile_group_header() |  |
| if( num_tiles_in_tile_group_minus1 > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < num_tiles_in_tile_group_minus1; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

The semantics of all syntax elements in tile_group_header() is identical to the corresponding syntax elements in tile_group_header() in JVET-L1001-v9.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).