| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Marrakesh, MA, 9–18 January 2019 | Document: JVET-M0378-v1 |
| --- | --- |

| Title: | AHG17: RPS for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg Mitra Damghanian Martin Pettersson | Email: | rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In this contribution it is proposed that a reference picture set (RPS) mechanism is adopted into the VVC draft. The proposal is to adopt the HEVC v5 RPS text, adapted for VVC with removal of inter-RPS.

The proponents believe that inter-RPS is beneficial for reducing the bit cost, but do not propose to include it into the VVC draft at this point. The authors propose to keep inter-RPS in the software for further study.

The proponents believe that the RPS text in HEVC v5 could have been written more compactly by editorial changes only. However, the proposed VVC text is not compacted since the authors believes there are no issues with a somewhat lengthy text.

The proposal is to keep the long-term picture handling as-is from HEVC, including e.g. delta_poc_msb_present_flag[ i ] and long-term pictures in the SPS. As a possible alternative, long-term pictures could be left out for now to be further studied.

Proposed text on top of JVET-L1001-v7 is attached with the contribution.

Text without long-term pictures showing how the VVC RPS text could be written in a manner that the authors find compact is provided as a separate modified JVET-L1001-v7 document.

For VTM, it is proposed to disable inter-RPS in the code as the only modification.

# Introduction

Reference pictures in HEVC are handled by the so-called reference picture set (RPS). The RPS is a set of reference picture indicators and is present in each slice header in HEVC (except for IDR picture slices). This means that the decoder before decoding each slice can check whether the status of the reference pictures in the decoder are in sync with the encoder or not. The RPS in HEVC is also used for marking pictures as one of "unused for reference", "used for short-term reference" or "used for long-term reference".

# Proposal

As a starting point in VVC, we propose RPS text adapted from HEVC v5. One major change is that Inter-RPS is proposed to not be included. We propose that Inter-RPS is kept in the source code for further study.

The proposed VVC draft text is attached as a separate document relative to JVET-L1001-v7. JVET-L1001-v7 was the latest VVC draft at the time of writing. It should be noted that JVET-L1001-v7 does not contain all high-level syntax adoptions from Macau.

The proposal is to keep the long-term picture handling as-is from HEVC, including e.g. delta_poc_msb_present_flag[ i ] and long-term pictures in the SPS. As a possible alternative, long-term pictures could be left out for now to be further studied.

Proposed text without long-term pictures showing how the HEVC RPS text could be compacted is provided as a separate attachment with the contribution.

For VTM, we propose to disable inter-RPS in the code as the only modification.

# Compact RPS text without long-term pictures

## Syntax and semantics

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_max_dec_pic_buffering_minus1 | ue(v) |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| ref_pic_set( i ) |  |
| ... |  |
| } |  |

log2_max_pic_order_cnt_lsb_minus4 specifies the value of the variable MaxPicOrderCntLsb that is used in the decoding process for picture order count as follows:

MaxPicOrderCntLsb = 2( log2_max_pic_order_cnt_lsb_minus4 + 4 )	(2-1)

The value of log2_max_pic_order_cnt_lsb_minus4 shall be in the range of 0 to 12, inclusive.

sps_max_dec_pic_buffering_minus1 plus 1 specifies the maximum required size of the decoded picture buffer for the CVS in units of picture storage buffers. The value of sps_max_dec_pic_buffering_minus1 shall be in the range of 0 to MaxDpbSize − 1, inclusive.

num_short_term_ref_pic_sets specifies the number of ref_pic_set( ) syntax structures included in the SPS. The value of num_short_term_ref_pic_sets shall be in the range of 0 to 64, inclusive.

NOTE 5 – A decoder should allocate memory for a total number of num_short_term_ref_pic_sets + 1 ref_pic_set( ) syntax structures since there may be a ref_pic_set( ) syntax structure directly signalled in the slice headers of a current picture. A ref_pic_set( ) syntax structure directly signalled in the slice headers of a current picture has an index equal to num_short_term_ref_pic_sets.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| for( i = 0; i < 2; i++ ) |  |
| num_ref_idx_default_active_minus1[ i ] | ue(v) |
| lists_modification_present_flag | u(1) |
| ... |  |
| } |  |

num_ref_idx_default_active_minus1[ i ] plus 1, when i is equal to 0, specifies the inferred value of the variable NumRefIdxActive[ 0 ] for P or B slices with num_ref_idx_active_override_flag equal to 0, and, when i is equal to 1, specifies the inferred value of NumRefIdxActive[ 1 ] for B slices with num_ref_idx_active_override_flag equal to 0. The value of num_ref_idx_default_active_minus1[ i ] shall be in the range of 0 to 14, inclusive.

lists_modification_present_flag equal to 1 specifies that the syntax structure ref_pic_lists_modification( ) is present in the slice segment header. lists_modification_present_flag equal to 0 specifies that the syntax structure ref_pic_lists_modification( ) is not present in the slice segment header.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| slice_address | u(v) |
| slice_type | ue(v) |
| if( nal_unit_type != IRAP_NUT) { |  |
| slice_pic_order_cnt_lsb | u(v) |
| short_term_ref_pic_set_sps_flag | u(1) |
| if( !short_term_ref_pic_set_sps_flag ) |  |
| ref_pic_set( num_short_term_ref_pic_sets ) |  |
| else if( num_short_term_ref_pic_sets > 1 ) |  |
| short_term_ref_pic_set_idx | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| for( i = 0; i < ( slice_type = = P ? 1: 2 ); i++ ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| if( lists_modification_present_flag && NumPicTotalCurr > 1 ) |  |
| for( i = 0; i < (slice_type == P ? 1 : 2 ); i++ ) { |  |
| ref_pic_list_modification_flag[ i ] | u(1) |
| if( ref_pic_list_modification_flag[ i ] ) |  |
| for( j = 0; j <= num_ref_idx_active_minus1[ i ]; j++ ) |  |
| list_entry[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| ... |  |
| } |  |

slice_pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the slice_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the slice_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive. When slice_pic_order_cnt_lsb is not present, slice_pic_order_cnt_lsb is inferred to be equal to 0.

short_term_ref_pic_set_sps_flag equal to 1 specifies that the short-term RPS of the current picture is derived based on one of the ref_pic_set( ) syntax structures in the active SPS that is identified by the syntax element short_term_ref_pic_set_idx in the slice header. short_term_ref_pic_set_sps_flag equal to 0 specifies that the short-term RPS of the current picture is derived based on the ref_pic_set( ) syntax structure that is directly included in the slice headers of the current picture. When num_short_term_ref_pic_sets is equal to 0, the value of short_term_ref_pic_set_sps_flag shall be equal to 0.

short_term_ref_pic_set_idx specifies the index, into the list of the ref_pic_set( ) syntax structures included in the active SPS, of the ref_pic_set( ) syntax structure that is used for derivation of the short-term RPS of the current picture. The syntax element short_term_ref_pic_set_idx is represented by Ceil( Log2( num_short_term_ref_pic_sets ) ) bits. When not present, the value of short_term_ref_pic_set_idx is inferred to be equal to 0. The value of short_term_ref_pic_set_idx shall be in the range of 0 to num_short_term_ref_pic_sets − 1, inclusive.

The variable CurrRpsIdx is derived as follows:

–	If short_term_ref_pic_set_sps_flag is equal to 1, CurrRpsIdx is set equal to short_term_ref_pic_set_idx.

–	Otherwise, CurrRpsIdx is set equal to num_short_term_ref_pic_sets.

num_ref_idx_active_override_flag equal to 1 specifies that the syntax element num_ref_idx_active_minus1[0] is present for P and B slices and that the syntax element num_ref_idx_active_minus1[1] is present for B slices. num_ref_idx_active_override_flag equal to 0 specifies that the syntax elements num_ref_idx_active_minus1[0] and num_ref_idx_active_minus1[1] are not present.

num_ref_idx_active_minus1[ i ], when present, specifies the value of the variable NumRefIdxActive[ i ] as follows:

NumRefIdxActive[ i ] = num_ref_idx_active_minus1[ i ] + 1

The value of num_ref_idx_active_minus1[ i ] shall be in the range of 0 to 14, inclusive.

The value of NumRefIdxActive[ i ] − 1 specifies the maximum reference index for reference picture list i that may be used to decode the slice. When the value of NumRefIdxActive[ i ] is equal to 0, no reference index for reference picture list i may be used to decode the slice.

For i equal to 0 or 1, when the current slice is a B slice and num_ref_idx_active_override_flag is equal to 0, NumRefIdxActive[ i ] is inferred to be equal to num_ref_idx_default_active_minus1[ i ] + 1.

When the current slice is a P slice and num_ref_idx_active_override_flag is equal to 0, NumRefIdxActive[ 0 ] is inferred to be equal to num_ref_idx_default_active_minus1[ 0 ] + 1.

When the current slice is a P slice, NumRefIdxActive[ 1 ] is inferred to be equal to 0.

When the current slice is an I slice, both NumRefIdxActive[ 0 ] and NumRefIdxActive[ 1 ] are inferred to be equal to 0.

ref_pic_list_modification_flag[ i ] equal to 1, with i equal to 0 or 1, indicates that reference picture list i is specified explicitly by a list of list_entry[ i ][ j ] values. ref_pic_list_modification_flag[ i ] equal to 0 indicates that reference picture list i is determined implicitly. When ref_pic_list_modification_flag[ i ] is not present in the slice header, it is inferred to be equal to 0.

list_entry[ i ][ j ] specifies the index of the reference picture in RefPicList[ i ]  to be placed at the current position of reference picture list i. The length of the list_entry[ i ][ j ] syntax element is Ceil( Log2( NumPicTotalCurr ) ) bits. The value of list_entry[ i ][ j ] shall be in the range of 0 to NumPicTotalCurr − 1, inclusive. When the syntax element list_entry[ i ][ j ] is not present in the slice header, it is inferred to be equal to 0.

The variable NumPicTotalCurr is derived as follows:

NumPicTotalCurr = 0
for(i = 0 ; i < NumPics ; i++ ) 
	NumPicTotalCurr += UsedByCurrPic[ i ]

It is a requirement of bitstream conformance that when the current picture contains a P or B slice, the value of NumPicTotalCurr shall not be equal to 0.

| ref_pic_set( stRpsIdx ) { | Descriptor |
| --- | --- |
| num_negative_pics[ stRpsIdx ] | ue(v) |
| num_positive_pics[ stRpsIdx ] | ue(v) |
| for( i = 0; i < num_negative_pics[ stRpsIdx ] + num_positive_pics[ stRpsIdx ]; i++ ) { |  |
| delta_poc_minus1[ stRpsIdx ][ i ] | ue(v) |
| used_by_curr_pic_flag[ stRpsIdx ][ i ] | u(1) |
| } |  |
| } |  |

num_negative_pics[ stRpsIdx ] specifies the number of entries in the stRpsIdx-th candidate short-term RPS that have picture order count values less than the picture order count value of the current picture.

num_positive_pics[ stRpsIdx ] specifies the number of entries in the stRpsIdx-th candidate short-term RPS that have picture order count values greater than the picture order count value of the current picture.

delta_poc_minus1[ stRpsIdx ][ i ] plus 1, when i is equal to 0, specifies the difference between the picture order count values of the current picture and i-th entry in the stRpsIdx-th candidate short-term RPS that has picture order count value less than that of the current picture, or, when i is greater than 0, specifies the difference between the picture order count values of the ( i − 1 )-th entry and the i-th entry in the stRpsIdx-th candidate short-term RPS that have picture order count values less than the picture order count value of the current picture. The value of delta_poc_minus1[ i ] shall be in the range of 0 to 215 − 1, inclusive.

used_by_curr_pic_flag[ stRpsIdx ][ i ] equal to 0 specifies that the i-th entry in the stRpsIdx-th candidate short-term RPS that has picture order count value less than that of the current picture is not used for reference by the current picture.

The variables NumNegativePics, NumPositivePics, NumPics, UsedByCurrPic[ i ] and DeltaPoc[ i ] are derived as follows:

NumNegativePics = num_negative_pics[ CurrRpsIdx ]
NumPositivePics = num_positive_pics[ CurrRpsIdx ]
NumPics = NumNegativePics + NumPositivePics

for(i=0 ; i< NumPics; i++) {
	UsedByCurrPic[ i ] = used_by_curr_pic_flag[ CurrRpsIdx ][ i ]
	DeltaPoc[ i ] = (i < NumNegativePics) ? 				delta_poc_minus1[ CurrRpsIdx ][ i ] + 1 :  - delta_poc_minus1[ CurrRpsIdx ][ i ] - 1
}

It is a requirement of bitstream conformance that DeltaPoc[ i ] is not equal to DeltaPoc[ j ], for any i and j in the range of 0 to NumPics-1, inclusive, unless i is equal to j.

## Decoding process for reference picture set

This process is invoked once per picture, after decoding of a slice header but prior to the decoding of any coding unit. This process may result in one or more reference pictures in the DPB being marked as "unused for reference".

NOTE 1 – The RPS is an absolute description of the reference pictures used in the decoding process of the current and future coded pictures. The RPS signalling is explicit in the sense that all reference pictures included in the RPS are listed explicitly.

A decoded picture in the DPB can be marked as "unused for reference" or "used for short-term reference", but only one among these two at any given moment during the operation of the decoding process. Assigning one of these markings to a picture implicitly removes another of these markings when applicable.

When the current picture is an IRAP picture, all reference pictures currently in the DPB (if any) are marked as "unused for reference".

Short-term reference pictures are identified by their PicOrderCntVal values.

NOTE 2 – A value of CurrRpsIdx in the range of 0 to num_short_term_ref_pic_sets − 1, inclusive, indicates that a candidate RPS from the active SPS for the current layer is being used, where CurrRpsIdx is the index of the candidate RPS into the list of candidate RPSs signalled in the active SPS for the current layer. CurrRpsIdx equal to num_short_term_ref_pic_sets indicates that the RPS of the current picture is directly signalled in the slice header.

NOTE 3 – The entries in RefPicSet[i] for which UsedByCurrPic[ i ] is equal to 1 are all reference pictures that may be used for inter prediction of the current picture and one or more pictures that follow the current picture in decoding order. The entries in RefPicSet[i] for which UsedByCurrPic[ i ] is equal to 0 are all reference pictures that are not used for inter prediction of the current picture but may be used in inter prediction for one or more pictures that follow the current picture in decoding order.

The derivation process for the RPS and picture marking are performed according to the following ordered steps:

pocBase = PicOrderCntVal
for( i = 0; i < NumPics; i++ ) {
		RefPicPocList[ i ] = pocBase – DeltaPoc[ i ]
		if( there is a reference picture picA in the DPB with PicOrderCntVal equal to RefPicPocList[ i ] )
			RefPicSet[ i ] = picA
		else
			RefPicSet[ i ] = "no reference picture"
		if( i == NumNegativePics )
			pocBase = PicOrderCntVal
		else
			pocBase = RefPicPocList[ i ]
}

The following applies:

- Each reference picture in the DPB that is not referred to by any entry in RefPicSet[ i ] is marked as "unused for reference".

NOTE 4 – There may be one or more entries in RefPicSet that are equal to "no reference picture" because the corresponding pictures are not present in the DPB. All entries RefPicSet[i] that are equal to "no reference picture" for which UsedByCurrPic[i] is equal to 0 should be ignored. An unintentional picture loss should be inferred for each entry in RefPicSet[i] equal to "no reference picture" for which UsedByCurrPic[i] is equal to 0.

It is a requirement of bitstream conformance that the RPS is restricted as follows:

- There shall be no entry in RefPicSet[i] for i in the range of 0 to NumPics − 1, inclusive, for which UsedByCurrPic[ i ] is equal to 1, for which one or more of the following are true:

  - The entry is equal to "no reference picture".

  - The entry is a picture that has TemporalId greater than that of the current picture.

- The number of pictures in RefPicSet shall be less than or equal to sps_max_dec_pic_buffering_minus1.

## Decoding process for reference picture lists construction

This process is invoked at the beginning of the decoding process for each P or B slice.

Reference pictures are addressed through reference indices. A reference index is an index into a reference picture list. When decoding a P slice, there is a single reference picture list RefPicList[0]. When decoding a B slice, there is a second independent reference picture list RefPicList[1] in addition to RefPicList[0].

At the beginning of the decoding process for each slice, the reference picture lists RefPicList[0] and, for B slices if present, RefPicList [1] are derived as follows:

The variable NumRpsCurrList[ i ] is set equal to Max( num_ref_idx_active_minus1[ i ] + 1, NumPicTotalCurr ) and the list RefPicList[ i ]  is constructed as follows:

for (i = 0; i < (slice_type == P ? 1 : 2); i++) 
		for( rIdx = 0; rIdx  <  num_ref_idx_active_minus1[ i ] + 1; rIdx++)
			if ( rIdx < NumRpsCurrList[ i ] )
				for( j = 0; j < NumPics; j++ ){
					index = j
					if( i == 1 )
						index = ( j < NumNegativePics ) ? j + NumPositivePics : j – NumNegativePics
					if( UsedByCurrPic[ j ] )
						RefPicList[ i ][ ref_pic_list_modification_flag[ i ] ? list_entry[i][ rIdx ] : rIdx ] =
							RefPicSet [ index ]
			}

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).