| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0396 |
| --- | --- |

| Title: | CE6-related: MTS kernel derivation for efficient memory usage |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sandeep Shrestha Ankit Kumar Bumshik Lee Yonggil Lee Junsuk Park | Email: | sandeepshrestha407@chosun.kr ankitku@chosun.kr bslee@chosun.ac.kr yglee@humaxdigital.com parkjs@humaxdigital.com |
| Source: | Chosun University and Humax |  |  |

_____________________________

- Abstract

In Multiple Transform Selection (MTS) of VTM3.0, DCT-2, DST-7 and DCT-8 have been used as transform kernels. In this contribution, we propose an MTS kernel derivation scheme to save memory consumption for kernel storage. In the proposed method, a complete matrix kernel of DST-7 can be derived with only a few numbers of rows of DST-7. The DCT-8 can also be derived from DST-7 with simple sign changing and flipping computations. For derivation of DCT-2, a sparse unified matrix from DST-3 is defined and it can be obtained using a sparse unified matrix and unit-element matrices consisting of -1, 0 and 1. The proposed method makes use of 1680 bytes for 8-bit precision for all kernels with different block size for MTS, which is significantly less than 4096 bytes in current VTM3.0.

Problem Statement

In MTS, three primary transform kernels with DCT-2, DCT-8 and DST-7 have been used. For storing various kernels with various block sizes, it is required to secure enough memory storage. In this contribution, we proposed a scheme to derive the DST-7 and DCT-8 kernels by storing only few numbers of row elements so that memory to save kernels can be saved. In addition, a DCT-2 kernel derivation scheme from a single sparse 64×64 matrix is also proposed.

Proposed method

The complete DST-7 kernel can be derived only with few numbers of rows of DST-7. Fig. 1 shows an example of the proposed method for the 8×8 DST-7 matrix. With the first and second row of the 8×8 DST-7, a complete 8×8 DST-7 kernel can be derived. The permutation matrices C, D and E in Fig. 1 should be multiplied to the first and second row to derive the other row elements, respectively.

- Obtaining a complete DST-7 kernel using two rows of DST-7 and permutation matrices

(b) The resulting matrix of DST-7

Fig. 1. A complete 8×8 DST-7 kernel can be derived only with the first and second row of DST-7 by multiplying permutation matrices C, D and E.

Fig. 2 also shows the example of the proposed method for 32×32 size of DST-7. As shown in Fig. 2, with only row 0-th, 1-st, 2-nd, 3-rd, 5-th and 6-th, a complete 32×32 DST-7 kernel can be derived.

Fig. 2. Rows (0,1,2,3,5 and 6) are used to derive all other rows of DST-7

In the case of 32×32 DST-7, five permutation matrices should be multiplied to each row to obtain the complete DST-7. The n number of permutation matrices are used for N×N kernel size as

2n=N                                                                           (1)

The five permutation matrices for 32×32 DST-7 are given in the Excel. Table 1 shows row numbers of DST-7 using permutation matrices and the 0-th, 1-st, 2-nd, 3-rd, 5-th and 6-th row of the DST-7.

Table 1. Permutation matrices for required to obtain the respective rows. (G, H, I, J and K are the permutation matrices)

| Rows | G | H | I | J | K |
| --- | --- | --- | --- | --- | --- |
| 0 | Generate 31-st row | Generate 30-th row | Generate 28-th row | Generate 24-th row | Generate 16-th row |
| 1 | Generate 29-th row | Generate 26-th row | Generate 20-th row | Generate 8-th row | Generate 15-th row |
| 2 | Generate 27-th row | Generate 22-nd row | Generate 12-th row | Generate 7-th row | Generate 17-th row |
| 3 | Generate 25-th row | Generate 18-th row | Generate 4-th row | Generate 23-rd row | Generate 14-th row |
| 5 | Generate 21-st row | Generate 10-th row | Generate 11-th row | Generate 9-th row | Generate 13-th row |
| 6 | Generate 19-th row |  |  |  |  |

A number of permutation matrices might fail to save memory for kernel storage as Table 2. Thus, in the proposed method, only G matrix can also be used instead of using the different permutation matrices (G, H, I , J and K for 32×32) which may cause the increase of memory storage. Fig. 3 shows the process of generating row elements of DST-7 only using G matrix with the 0-th row of DST-7.

Fig. 3. Generation of row element of DST-7 only using permutation G matrix

The step for obtaining 31-st, 30-th, 28-th, 24-th and 16-th rows using G and 0-th row of DST-7.

- Generate 31-st row using 0-th row of DST-7 and G matrix.

- Generate 30-th row using already generated 31-st row and G matrix

- Repeat until the 16-th row as Fig. 3.

- If the first element of each generated row is negative, multiply minus to the row elements to change the sign.

Table 2. The generated row number of DST-7 using matrix G and 0, 1, 2, 3, 5, 6-th row of DST-7

| Row number of DST-7 | The generated row number of DST-7 using matrix G and DST-7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | G |  |  |  |  |
| 0 | Generate 31-st row | Generate 30-th row | Generate 28-th row | Generate 24-th row | Generate 16-th row |
| 1 | Generate 29-th row | Generate 26-th row | Generate 20-th row | Generate 8-th row | Generate 15-th row |
| 2 | Generate 27-th row | Generate 22-nd row | Generate 12-th row | Generate 7-th row | Generate 17-th row |
| 3 | Generate 25-th row | Generate 18-th row | Generate 4-th row | Generate 23-rd row | Generate 14-th row |
| 5 | Generate 21-st row | Generate 10-th row | Generate 11-th row | Generate 9-th row | Generate 13-th row |
| 6 | Generate 19-th row |  |  |  |  |

Table 2 shows the generated row number of DST-7 using G and the 0-th, 1-st, 2-nd, 3-rd, 5-th, 6-th row of 32×32 DST-7

Fig. 4. Example of generating all row elements for 32×32 DST-7 only using G matrix

Similarly, other rows of DST-7 can be obtained using G matrix and Table 2. Fig. 4 shows the resulting DST-7 only using G matrix and total six rows f DST-7. Thus, only storing six rows can generate a complete of DST-7 for 32×32 block size. This can be applied to the different size of DST-7 kernels.

Once we obtained DST-7, we can get the DCT-8 of various block size as

DCT-8=S×DST-7×F                                                              (2)

where, F is the flipping matrix and S is the sign changing matrix and defined as

Fm,n=1,if n=N-1-m,0,otherwise,                                                      (3)

Sm,n=(-1)m,if n=m,0,otherwise.                                                  (4)

For DCT-2, a 64×64 DST-3 matrix can be decomposed with a unified matrix (U) and five unit-element matrices (A, B, C, D and E) consisting of 0, 1 and -1 as

DST-364=UABCDE                                                                (5)

where the U represent the unified matrix and can be expressed as Fig. 1, A, B, C, D and E matrices are the matrices consisting of -1, 0 and 1.

Fig. 5. Decomposition of a 64×64 DST-3 matrix

In order to obtain the DCT-2 matrix of various block sizes, DST-3 matrices are first obtained using the unified matrix and unit-element matrices.

DST-34=T[U64×A64]4                                                  (6)

DST-38=T[U64×M64]8                                                 (7)

DST-316=T[U64×N64]16                                              (8)

DST-332=T[U64×O64]32                                              (9)

DST-364=U64×P64                                                       (10)

where T[ ]N represent the function that takes N×N block of the right-bottom part of the matrix,  U indicates the unified matrix as (5) and M, N, O and P matrices can be obtained using unit-element matrices as

| M=A×B N=A×B×C O=A×B×C×D P=A×B×C×D×E | ( 11) |
| --- | --- |

Finally, DCT-2 can readily be derived using (12).

DCT-2=F×DST-3×S                                                     (12)

where F and S are a flipping and sign changing matrices, respectively.

Experimental results

The proposed method has been implemented on VTM-3.0 reference software and experimentally evaluated for VTM test according to common test conditions for AI, RA and LDB configurations.

Table 1. Simulation results for the AI configuration over VTM-3.0.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.67% | 0.08% | 0.13% | 98% | 96% |
| Class A2 | 1.07% | 0.61% | 0.55% | 99% | 96% |
| Class B | 1.13% | 0.75% | 0.78% | 100% | 95% |
| Class C | 1.47% | 1.13% | 1.09% | 102% | 95% |
| Class E | 2.06% | 1.71% | 1.81% | 101% | 95% |
| Overall | 1.27% | 0.86% | 0.87% | 100% | 95% |
| Class D | 1.49% | 0.99% | 1.11% | 103% | 96% |
| Class F (optional) | 1.24% | 0.99% | 1.08% | 102% | 98% |

Table 2. Simulation results for the RA configuration over VTM-3.0.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.25% | -0.05% | 0.25% | 101% | 99% |
| Class A2 | 0.39% | 0.28% | 0.19% | 100% | 100% |
| Class B | 0.51% | 0.46% | 0.27% | 100% | 99% |
| Class C | 0.76% | 0.48% | 0.58% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.50% | 0.33% | 0.33% | 101% | 99% |
| Class D | 0.71% | 0.26% | 0.52% | 101% | 99% |
| Class F (optional) | 0.76% | 0.53% | 0.76% | 101% | 99% |

Table 3. Simulation results for the LDB configuration over VTM-3.0.

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.26% | 0.59% | 0.49% | 101% |  |
| Class E | 0.37% | 0.94% | -0.25% | 101% |  |
| Overall |  |  |  |  |  |
| Class D | 0.19% | 0.80% | 0.78% | 101% |  |
| Class F (optional) | 0.24% | -0.09% | -0.39% | 100% |  |

- 4 Patent rights declaration(s)

Chosun University and Humax may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).