| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0412 |
| --- | --- |

| Title: | CE5: Per-context CABAC initialization with double windows (Test 5.1.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Jie Dong, Hilmi Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin | Tel: Email: | +1-408-533-9515 jiedong@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results for Test CE5.1.3 [1]. In this test, the CABAC engine uses double windows to estimate the binary symbol probabilities, where the window parameters are different for different contexts and are stored in a pre-defined look-up table (LUT). The range update method used for the CABAC engine is either the one proposed in JVET-L0117 [2] or the one proposed in JVET-L0335 [3]. Compared with VTM3.0, the average coding gains of the proposed CABAC engine are -0.99% AI, -0.95% RA, and -0.82% LDB, when JVET-L0117 is used for range update; and are -0.95% AI, -0.90% RA, and -0.77% LDB, when JVET-L0335 is used for range update.

# Introduction

## Probability Estimation

The CABAC probability estimation method of this test was proposed in JVET-L0115 [4]. Maintaining the original notation, U[k] and V[k] represent the probabilities estimated by the two windows, respectively. They have n-bit precision (n=10 and 14), as they are obtained by scaling the real probabilities by 2n and rounding. These probability estimates are updated using the recursive equations, as below.

If the current bin is 1, U[k] and V[k] are updated to U[k+1] and V[k+1], by (1) and (2), respectively.

U[k+1] = U[k] + ((210 – U[k]) >> a)                                                   (1)

V[k+1] = V[k] + ((214 – V[k]) >> b)                                                   (2)

Otherwise, if the current bin is 0, U[k] and V[k] are updated to U[k+1] and V[k+1], by (3) and (4), respectively.

U[k+1] = U[k] – (U[k] >> a)                                                         (3)

V[k+1] = V[k]  – (V[k]) >> b)                                                         (4)

The probability used for binary arithmetic coding, P[k], is the average of U[k] and V[k], as in (5).

P[k] = (U[k] + V[k]) >> 1                                                           (5)

The two positive integer window shift parameters a and b can be different for each context and are store in a look-up table(LUT). In each entry of the LUT, a 4-bit parameter c is stored to represent a and b, as shown in (6) and (7), respectively.

a = (c >> 2) + 2                                                                         (6)

b = a + 1 + (c & 3)                                                                      (7)

Since the value of c for each context are never modified, in custom hardware implementation they can be stored in less expensive and power-efficient read-only memory (ROM).

## Range Update

The two CABAC range update methods of this test were proposed in JVET-L0117 [2] and JVET-L0335 [3], respectively.

In JVET-L0117, before the computation of the interval sub-division, the probabilities are scaled using the integer function, as shown in (8),

z(P) = floor(log2(P))                                                                (8)

which can be computed efficiently with “bit-scan-reverse” instructions, simple hardware, or table look-up, using the same table employed for renormalization of the arithmetic coding interval.

When the probability estimate is scaled by 2R, and 3 bits are used for representing probability (after the most-significant 1), and 3 bits for the range (in the same manner), the operands (or indices for a look-up table) can be obtained as in (9) and (10), respectively.

m(P) = (P >> (z(P) – 3)) & 7                                                          (9)

n(R) = (R >> 5) & 7                                                               (10)

The range is updated using (11),

RL = ( TN [m(P), n(R)] >> (R – 2 – z(P)) ) | 2                                          (11)

where TN [m(P), n(R)] is obtained by using (12) as below.

TN [m(P), n(R)] = ( (17 + 2×m(P)) × (17+2× n(R)) ) >> 2                                (12)

In practice, TN[m, n] can be generated on-the-fly by substituting m(P) and n(R) into (6) or by looking up a pre-defined 2-D table with m(P) and n(R) as the searching indices.

In JVET-L0335, the modifications were summarized as:

- Modify the constants 214 to 214−1 (e.g., 16384 to 16383) and 210 to 210−1 (e.g., 1024 to 1023) in the probability estimate update equations, i.e., (1) and (2).

- Modify the subinterval range computation for the LPS symbol to ((r >> 5) * (qLPS >> (b − 5)) >> 1) + 4. Note that, alternatively, this equation can be implemented using a 32×8×8 = 2048 bits LUT.

# Results

The proposed CABAC engines are integrated into VTM3.0, and the coding gains are summarized as below. More details can be found in the enclosed spreadsheets.

Table 1 Proposed probability estimation with JVET-L0117 for range update

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.05% | -1.65% | -1.50% | 112% | 107% |
| Class A2 | -1.02% | -1.59% | -1.48% | 106% | 98% |
| Class B | -0.97% | -1.27% | -1.47% | 105% | 103% |
| Class C | -0.94% | -1.24% | -1.49% | 106% | 107% |
| Class E | -0.98% | -1.55% | -1.77% | 109% | 109% |
| Overall | -0.99% | -1.42% | -1.53% | 107% | 105% |
| Class D | -0.81% | -1.15% | -1.09% | 105% | 111% |
| Class F (optional) | -0.72% | -1.14% | -1.08% | 106% | 108% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.98% | -1.43% | -1.41% | 105% | 102% |
| Class A2 | -0.92% | -0.86% | -1.02% | 105% | 104% |
| Class B | -0.98% | -0.97% | -1.03% | 107% | 105% |
| Class C | -0.90% | -1.09% | -0.79% | 108% | 110% |
| Overall | -0.95% | -1.07% | -1.04% | 106% | 106% |
| Class D | -0.73% | -1.26% | -0.74% | 106% | 109% |
| Class F (optional) | -0.56% | -0.73% | -0.74% | 118% | 108% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.86% | -0.36% | 0.30% | 109% | 112% |
| Class C | -0.79% | -0.20% | -0.17% | 105% | 97% |
| Class E | -0.78% | 0.69% | -0.86% | 105% | 99% |
| Overall | -0.82% | -0.04% | -0.15% | 107% | 104% |
| Class D | -0.48% | -1.05% | 0.10% | 106% | 113% |
| Class F (optional) | -0.39% | 0.14% | -0.19% | 108% | 104% |

Table 2 Proposed probability estimation with JVET-L0335 for range update

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.00% | -1.56% | -1.38% | 113% | 106% |
| Class A2 | -0.98% | -1.57% | -1.48% | 107% | 101% |
| Class B | -0.94% | -1.15% | -1.40% | 106% | 100% |
| Class C | -0.89% | -1.12% | -1.33% | 105% | 105% |
| Class E | -0.95% | -1.54% | -1.65% | 109% | 107% |
| Overall | -0.95% | -1.35% | -1.44% | 107% | 103% |
| Class D | -0.78% | -1.23% | -1.03% | 104% | 106% |
| Class F (optional) | -0.71% | -1.02% | -1.03% | 105% | 105% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.93% | -1.34% | -1.34% | 106% | 106% |
| Class A2 | -0.87% | -0.92% | -1.00% | 106% | 103% |
| Class B | -0.93% | -1.03% | -0.95% | 105% | 104% |
| Class C | -0.87% | -1.00% | -0.67% | 105% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.90% | -1.06% | -0.97% | 105% | 106% |
| Class D | -0.67% | -1.21% | -0.89% | 102% | 105% |
| Class F (optional) | -0.54% | -0.63% | -0.76% | 116% | 106% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.82% | -0.05% | 0.37% | 107% | 111% |
| Class C | -0.74% | -0.25% | -0.16% | 105% | 98% |
| Class E | -0.74% | 0.39% | -1.33% | 105% | 98% |
| Overall | -0.77% | -0.01% | -0.23% | 106% | 103% |
| Class D | -0.48% | -0.61% | -0.30% | 106% | 111% |
| Class F (optional) | -0.18% | 0.06% | 0.04% | 105% | 103% |

# Reference

- H. Kirchhoffer, A. Said, “Description of Core Experiment 5 (CE5): Arithmetic Coding Engine,” JVET-L1025, 12th Meeting, Macao, China, October 2018.

- A. Said, et al., “CE5: Binary arithmetic coding range update with small table or short multiplications (Test 5.2.4),” JVET-L0117, 12th Meeting, Macao, China, October 2018.

- F. Bossen, “CE5: probability update (5.1.1) and range computation (5.2.1) tests,” JVET-L0335, 12th Meeting, Macao, China, October 2018.

- A. Said, et al., “CE5: Per-context CABAC initialization with double windows (Test 5.1.6),” JVET-L0115, 12th Meeting, Macao, China, October 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).