| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0421 |
| --- | --- |

| Title: | Non-CE1: Split-first signalling for partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Wieckowski, T. Nguyen, H. Schwarz, D. Marpe, and T. Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin Germany | Tel: Email: | +49 30 31002 833 adam.wieckowski@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In the current draft of VVC, the partitioning is a straightforward extension of the HEVC partitioning approach. At the CTU level, a quad-split (QT) flag is signalled indicating whether a square block should be split into four equally sized square blocks. Once no further quad-split is signalled, a binary-and-ternary tree (BTT) is signalled, for which the splits are signaled more extensively using up to three flags, indicating if a block should be split, in which direction it should be split, and if a ternary or binary split is to be performed. If a block should not be split, it is required to signal up to two flags indicating that neither a QT nor BTT split is to be performed. The proposed approach in this document adapted signalling in which a single flag indicates if a block should be split, followed by coding of the split mode. The proposed adaptation of split binarization, together with adapted context modelling introduces AI/RA/LB/LP luma BD-rate gains of -0.03/-0.07/-0.06/-0.05.

# Introduction

In the current implementation of VVC, the signaling of partitioning structure is still rooted in the HEVC model. The coding tree unit CTU is first partitioning using the HEVC quad-tree structure. At the roots of this structure (i.e., QT-nodes), further signaling is utilized to signal the second level of partitioning, the binary-and-ternary tree (BTT) structure. This separation causes some of the information to be required to be signalled multiple times, e.g., in case of a no-split, it might be required two signaled flags as ‘0’ (qt_split_flag and mtt_split_flag). It is asserted that it is not required to separate the two partitioning structures at the syntax level (i.e., the BTT structure being rooted at the nodes of the QT structure), but rather view the partitioning as a single process with semantic differences between the different split options (e.g., the QT split not being allowed in a node resulting from a non-QT split). This will allow to reduce to “code duplication” in the draft syntax and the reference implementation, alas the splits already show common behaviour with respect to e.g. available block sizes, quantization parameter signalling, and picture boundary.

To accomplish this change we propose to reverse the order of the qt_split_flag and mtt_split_flag (the latter becoming a more general split_flag).

The proposed adaptation introduces AI/RA/LB/LP luma BD-rate gains -0.03/-0.07/-0.06/-0.05. When the non-normative effects of split-signaling rate (i.e. speed-ups) are disabled, the gains are very similar at -0.02/-0.06/-0.07/-0.09. The adapted context modeling required for the new binarization uses a total of 2 context models more that the current draft specification.

# Binarization

Figure 1: Adapted binarization with split_flag and qt_split_flag reversed.

Figure 1: Adapted binarization with split_flag and qt_split_flag reversed.

In Figure 1, the adapted binarization is schematically presented. The order of qt_split_flag and split_flag is reversed. With this change, the split_flag indicates if any split is performed, including a QT split, which was previously signaled separately from the rest of partitioning information. The rest of the binarization was not changed.

# Context modeling

The semantic changes in the flags require adaptations in context modelling to take full advantage of the new binarization.

## split_flag

The context modeling is dependent on the split availability information for the quad-split (AQT), binary horizontal (ABH) and vertical (ABV) splits, and ternary horizontal (ATH) and vertical (ATV) splits. This information always needs to be evaluated for split signaling to remove redundant flags. Further, just as the mtt_split_flag in VTM-3.0, the context modelling is conditioned on the current and neighboring blocks’ width and height (W, H, Hleft, and Wabove).

ctxId = ( W > Wabove ? 1 : 0 ) + ( H > Hleft ? 1 : 0 ) + 3 ( ( 2AQT + ABH + ABV + ATH + ATV – 1 ) / 3 )

The number of contexts is decreased from 12 to 9.

## qt_split_flag

The context modelling is identical to VTM-3.0.

## mtt_vertical_flag

Similar to split_flag, the context modeling for the mtt_vertical_flag is conditioned on the availability of the binary and ternary splits ABH, ABV, ATH, and ATV as well as the current and neighboring blocks’ width and height.

dW = W / Wabove
dH = H / Hleft
d = dW  = =  dH ? 0 : ( dW < dH ? 1 : 2 )
ctxId = ABH + ATH  = =  ABV + ATV ? d : (ABH + ATH < ABV + ATV ? 3 : 4 )

The number of contexts for this flag is increased from 3 to 5.

## mtt_binary_flag

The context modeling is dependent on the value of previously signaled or inferred flag mtt_vertical_flag and current BT-depth DBT.

ctxId = ( mtt_vertical_flag ? 1 : 0 ) + ( DBT < 2 ? 1 : 0 )

The number of contexts for this flag is increased from 1 to 4.

# Results

## Against VTM-3.0

## The decoder runtimes might not be reliable. The reduced decoder runtimes might be a result removed redundant operations introduced in the required refactoring.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.38% | -0.24% | 107% | 100% |
| Class A2 | -0.04% | -0.13% | -0.10% | 105% | 99% |
| Class B | -0.04% | -0.28% | -0.30% | 102% | 99% |
| Class C | -0.01% | -0.36% | -0.43% | 100% | 99% |
| Class E | -0.05% | -0.69% | -0.67% | 100% | 99% |
| Overall | -0.03% | -0.36% | -0.35% | 103% | 99% |
| Class D | 0.02% | -0.19% | -0.36% | 100% | 98% |
| Class F (optional) | 0.04% | -0.22% | -0.16% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.16% | -0.22% | 103% | 102% |
| Class A2 | -0.12% | -0.11% | -0.13% | 102% | 101% |
| Class B | -0.08% | -0.17% | -0.34% | 102% | 102% |
| Class C | -0.04% | -0.38% | -0.24% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.21% | -0.25% | 102% | 101% |
| Class D | 0.00% | -0.62% | -0.51% | 101% | 99% |
| Class F (optional) | -0.02% | -0.09% | -0.26% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.33% | -0.52% | 101% | 97% |
| Class C | -0.04% | 0.06% | 0.00% | 101% | 97% |
| Class E | -0.05% | 0.43% | -0.96% | 101% | 97% |
| Overall | -0.06% | -0.01% | -0.46% | 101% | 97% |
| Class D | 0.00% | -0.23% | -0.21% | 101% | 99% |
| Class F (optional) | 0.10% | -0.43% | -0.88% | 101% | 96% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | 0.22% | -0.40% | 100% | 96% |
| Class C | -0.02% | 0.09% | 0.06% | 101% | 97% |
| Class E | -0.03% | 0.16% | -1.24% | 101% | 95% |
| Overall | -0.05% | 0.16% | -0.46% | 101% | 96% |
| Class D | 0.00% | -0.08% | -0.10% | 101% | 97% |
| Class F (optional) | 0.01% | -1.00% | -0.95% | 101% | 96% |

## Against VTM-3.0 without split-cost based speed-ups

The current implementation of VVC – VTM-3.0 – contains a split-cost based encoder speed-up which would skip testing of split modes based on a heuristic rule. To ensure that the gain is caused by different behavior of that speed-up caused by adapted binarization and context modelling, we provide results with the speed-up in question disable in both reference and test. The results match the behavior presented over VTM-3.0.

The decoder times might be unreliable. The reduced decoder runtimes might be a result removed redundant operations introduced in the required refactoring.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0-noSpeedUp |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.09% | 0.00% | 102% | 99% |
| Class A2 | -0.03% | -0.11% | -0.12% | 102% | 99% |
| Class B | -0.03% | -0.18% | -0.21% | 102% | 99% |
| Class C | 0.01% | -0.18% | -0.12% | 101% | 99% |
| Class E | -0.03% | -0.27% | -0.33% | 102% | 98% |
| Overall | -0.02% | -0.17% | -0.16% | 102% | 99% |
| Class D | -0.01% | -0.28% | -0.26% | 101% | 100% |
| Class F (optional) | 0.02% | -0.02% | -0.10% | 101% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0-noSpeedUp |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.21% | -0.11% | 100% | 101% |
| Class A2 | -0.07% | -0.23% | -0.19% | 100% | 101% |
| Class B | -0.07% | -0.30% | -0.08% | 100% | 98% |
| Class C | -0.06% | -0.12% | 0.13% | 101% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.22% | -0.05% | 101% | 101% |
| Class D | -0.01% | -0.24% | -0.25% | 101% | 102% |
| Class F (optional) | -0.03% | -0.01% | -0.22% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0-noSpeedUp |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | -0.04% | -0.20% | 101% | 101% |
| Class C | 0.00% | 0.37% | -0.03% | 101% | 101% |
| Class E | -0.14% | -0.83% | -0.17% | 100% | 96% |
| Overall | -0.07% | -0.10% | -0.13% | 101% | 100% |
| Class D | 0.03% | 0.14% | 0.10% | 102% | 100% |
| Class F (optional) | -0.08% | -0.24% | -0.37% | 100% | 98% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0-noSpeedUp |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.16% | -0.42% | 100% | 101% |
| Class C | -0.02% | 0.22% | -0.04% | 101% | 100% |
| Class E | -0.25% | -0.64% | -0.64% | 100% | 95% |
| Overall | -0.09% | -0.15% | -0.35% | 101% | 99% |
| Class D | 0.00% | -0.57% | -0.29% | 101% | 99% |
| Class F (optional) | 0.07% | -0.13% | 0.08% | 101% | 97% |

# Acknowledgments

# The contribution has been kindly cross-checked by Sharp. The cross-check results are available in the contribution JVET-M0780.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).