| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0430 |
| --- | --- |

| Title: | AHG12: On Tiles and Tile Groups for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin, Karsten Sühring, Yago Sanchez, Thomas Schierl (HHI) | Tel: Email: | +49-30-31002-185 firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes changes to the current VVC tiling signalling in order to accommodate bitstream merging scenarios, e.g. as in MPEG-OMAF, and avoid tile_group_header rewriting.

First, the coded length of tile_group_address in tile_group headers is proposed to be explicitly signalled in the picture parameter set instead of being derived from the number of tiles per picture.

Second, the tile group addresses in picture parameters are proposed to be explicitly signalled instead of derived from scan order.

# Introduction

This contribution discusses aspects related to tiles in VVC. During the last meeting, JVET-L0686 was adopted in which slices are removed in favor of tile groups and the HEVC syntax element slice_address is substituted with tile_group_address in the tile_group_header (if there is more than one tile in the picture) as address of the first tile in the tile group.

While tiles, especially temporal motion-constrained tiles, have shown to be useful in 360° video applications, the current VVC design has some drawbacks for bitstream merging as discussed in the following.

For instance, in MPEG OMAF, there is a profile, namely the HEVC viewport-dependent baseline profile, that enables a particular usage of tiles of a 360° video at different resolutions that can be used to match the viewing orientation of users. This is achieved by downloading tiles at high resolution that correspond to the viewport of a user and others at low resolutions. The downloaded tiles can be merged into a single bitstream, which requires re-writing of parameter sets and slice segment headers in case of HEVC. The reason for this is that slice segment headers need to be re-written to signal the proper slice_segment_address within the merged bitstream.

In the current VVC, although signalling tile_group_address instead of slice_segment_address is advantageous for such a merging process, modification of the tile group header in such scenarios is still required. The main problems preventing rewriting avoidance are:

- In the tile group header: tile_group_address depend on the number of tiles in the picture (NumTilesInPic); i.e. the coded length of the syntax element is specified as Ceil( Log2 ( NumTilesInPic ) ) bits and the tile_group_address is not present in the tile group header unless NumTilesInPic>1.

- In the picture parameter set: the tile group addresses are implicitly derived by the position within the tiling grid as defined.

# Proposal

Proposed changes relative the adopted syntax in JVET-L0686.

Editorially, it is proposed to rename the address in the tile_group_header into first_tile_address. In addition, it is proposed that first_tile_address and num_tiles_in_tile_group_minus1 presence is not conditional to NumTilesInPic being larger than 1.

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_parameter_set_id | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| first_tile_group_address | u(v) |
| num_tiles_in_tile_group_minus1 | ue(v) |
| } |  |
| […] |  |
| byte_alignment( ) |  |
| } |  |

The proposed picture parameter set is as follows. The coded length of tile addresses in the tile group header and the picture parameter set is explicitly signalled and each tile in the described tiling setup is explicitly assigned a tile_address.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| transform_skip_enabled_flag | u(1) |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| uniform_tile_spacing_flag | u(1) |
| if( !uniform_tile_spacing_flag ) { |  |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| } |  |
| tile_address_length | ue(v) |
| for( i = 0; i < NumTilesInPic; i++ ) |  |
| tile_address[ i ] | u(v) |
| rbsp_trailing_bits( ) |  |
| } |  |

tile_address_length_minus1 plus 1 specifies the length of the first_tile_address in tile_group_header and tile_address[ i ] in bits. The value of tile_address_length shall be in the range of 1 to X [Ed. maximum depending on level tile number limits], inclusive.

tile_address[ i ] specifies the tile address of the i-th tile in a raster scan of the tiles of a picture. The length of tile_address[ i ] is tile_address_length_minus1 plus 1 bits. The value of tile_address[ i ] shall be in the range of 0 to (1 << (tile_address_length_minus1 + 1) − 1 ), inclusive, and the value of tile_address[ i ] shall not be equal to the value of tile_ address[ j ] for any value of i unequal to the value of j.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).