#### 7.3.2.1	Sequence parameter set RBSP syntax

| … | u(1) |
| --- | --- |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| sps_affine_mmvd_enabled_flag | u(1) |
| } |  |
| sps_gbi_enabled_flag | u(1) |
| … | u(1) |

#### Merge data syntax

…

| … |  |
| --- | --- |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( sps_affine_mmvd_enabled_flag ) { |  |
| affine_mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( affine_mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| affine_mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| affine_mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| … |  |

…

#### Sequence parameter set RBSP semantics

…

sps_affine_type_flag specifies whether 6-parameter affine model based motion compensation can be used for inter prediction. If sps_affine_type_flag is equal to 0, the syntax shall be constrained such that no 6-parameter affine model based motion compensation is used in the CVS, and cu_affine_type_flag is not present in coding unit syntax in the CVS. Otherwise (sps_affine_type_flag is equal to 1), 6-parameter affine model based motion compensation can be used in the CVS. When not present, the value of sps_affine_type_flag is inferred to be equal to 0.

sps_affine_mmvd_enabled_flag equal to 1 specifies whether the affine with motion vector difference can be used for inter prediction. sps_affine_mmvd_enabled_flag equal to 0 specifies that affine with motion vector difference is not used in the CVS. When sps_affine_mmvd_enabled_flag is not present, it is inferred to be equal to 0. When sps_affine_mmvd_enabled_flag is equal to 1, affine with motion vector difference can be used in the CVS.

sps_gbi_enabled_flag specifies whether bi-prediction with CU weights can be used for inter prediction. If sps_gbi_enabled_flag is equal to 0, the syntax shall be constrained such that no bi-prediction with CU weights is used in the CVS, and gbi_idx is not present in coding unit syntax of the CVS. Otherwise (sps_gbi_enabled_flag is equal to 1), bi-prediction with CU weights can be used in the CVS.

…

#### Merge data semantics

…

Both components of of the merge plus MVD offset MmvdOffset[ x0 ][ y0 ] are derived as follows:

MmvdOffset[ x0 ][ y0 ][ 0 ] = ( MmvdDistance[ x0 ][ y0 ] << 2 ) * MmvdSign[ x0 ][ y0 ][0]	(7-71)

MmvdOffset[ x0 ][ y0 ][ 1 ] = ( MmvdDistance[ x0 ][ y0 ] << 2 ) * MmvdSign[ x0 ][ y0 ][1]	(7-72)

affine_mmvd_flag[ x0 ][ y0 ] equal to 1 specifies that affine merge mode with motion vector difference is used to generate the affine inter prediction parameters of the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. affine_mmvd_flag[ x0 ][ y0 ] equal to 0 specifies that affine merge mode with motion vector difference is not used to generate the affine inter prediction parameters of the current coding unit.

When affine_mmvd_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

affine_mmvd_distance_idx[ x0 ][ y0 ] specifies the index used to derive AffineMmvdDistance[ x0 ][ y0 ] as specified in Table 7-xx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-xx – Specification of AffineMmvdDistance[ x0 ][ y0 ] based on affine_mmvd_distance_idx[ x0 ][ y0 ]

| affine_mmvd_distance_idx[ x0 ][ y0 ] | AffineMmvdDistance[ x0 ][ y0 ] |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |

Affine_mmvd_direction_idx[ x0 ][ y0 ] specifies index used to derive AffineMmvdSign[ x0 ][ y0 ] as specified in Table 7-xx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-xx – Specification of MmvdSign[ x0 ][ y0 ] based on mmvd_direction_idx[ x0 ][ y0 ]

| affine_mmvd_direction_idx[ x0 ][ y0 ] | AffineMmvdSign[ x0 ][ y0 ][0] | AffineMmvdSign[ x0 ][ y0 ][1] |
| --- | --- | --- |
| 0 | +1 | 0 |
| 1 | -1 | 0 |
| 2 | 0 | +1 |
| 3 | 0 | -1 |

AffineMmvdOffset[ x0 ][ y0 ] are derived as follows:

AffineMmvdOffset[ x0 ][ y0 ][ 0 ] = ( AffineMmvdDistance[ x0 ][ y0 ] << 3 ) * AffineMmvdSign[ x0 ][ y0 ][0]	(7-xx)

AffineMmvdOffset[ x0 ][ y0 ][ 1 ] = ( AffineMmvdDistance [ x0 ][ y0 ] << 3 ) * AffineMmvdSign [ x0 ][ y0 ][1]	(7-xx)

merge_subblock_flag[ x0 ][ y0 ] specifies whether the subblock-based inter prediction parameters for the current coding unit are inferred from neighbouring blocks. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When merge_subblock_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

…

#### General

…

For the derivation of the variables mvL0[ xSbIdx ][ ySbIdx ], mvL1[ xSbIdx ][ ySbIdx ], mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, numSbX, numSbY, predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ], the following applies:

- If merge_subblock_flag[ xCb ][ yCb ] is equal to 1, the following apply

  - If affine_mmvd_flag[ xCb ][ yCb ] is equal to 1, the derivation process for motion vectors and reference indices in subblock merge mode as specified in 8.3.4.10 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the reference indices refIdxL0, refIdxL1, the prediction list utilization flag arrays predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ], the luma subblock motion vector arrays mvL0[ xSbIdx ][ ySbIdx ] and mvL0[ xSbIdx ][ ySbIdx ], and the chroma subblock motion vector arrays mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ], with xSbIdx = 0.. numSbX − 1, ySbIdx = 0 .. numSbY − 1, and the bi-prediction weight index gbiIdx as outputs.

  - Otherwise (affine_mmvd_flag[ xCb ][ yCb ] is equal to 0), the derivation process for motion vectors and reference indices in subblock merge mode as specified in 8.3.4.2 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, the reference indices refIdxL0, refIdxL1, the prediction list utilization flag arrays predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ], the luma subblock motion vector arrays mvL0[ xSbIdx ][ ySbIdx ] and mvL0[ xSbIdx ][ ySbIdx ], and the chroma subblock motion vector arrays mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ], with xSbIdx = 0.. numSbX − 1, ySbIdx = 0 .. numSbY − 1, and the bi-prediction weight index gbiIdx as outputs.

- Otherwise (merge_subblock_flag[ xCb ][ yCb ] is equal to 0), for X being replaced by either 0 or 1 in the variables predFlagLX, cpMvLX, MvdCpLX, and refIdxLX, in PRED_LX, and in the syntax element ref_idx_lX, the following ordered steps apply:

…

8.3.4.9	Derivation process for motion vector arrays from affine control point motion vectors

….

[Ed. (BB): This way four 2x2 chroma subblocks (4x4 chroma block) share the same motion vector which is derived from the aveage of the four 4x4 luma subblock motion vectors. In the decoding process motion compensation is still performed on 2x2 chroma blocks which is however a motion compensation on a chroma 4x4 block because all chroma MVs inside a 4x4 chroma block are the same. I would prefer an editorial change that makes it more clear that affine chroma MC is performed on 4x4 chroma blocks.]

#### Dervivation process for affine merge with motion vector difference

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

Outputs of this process are:

- the reference indices refIdxL0 and refIdxL1,

- the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY,

- the prediction list utilization flag arrays predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbX − 1,

- the luma subblock motion vector arrays in 1/16 fractional-sample accuracy mvL0[ xSbIdx ][ ySbIdx ] and mvL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1,

- the chroma subblock motion vector arrays in 1/32 fractional-sample accuracy mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1,

- For the derivation of the variables mvL0[ xSbIdx ][ ySbIdx ], mvL1[ xSbIdx ][ ySbIdx ], mvCL0[ xSbIdx ][ ySbIdx ] and mvCL1[ xSbIdx ][ ySbIdx ], refIdxL0, refIdxL1, numSbX, numSbY, predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ], the following applies:

- the bi-prediction weight index gbiIdx.

- The variable AffMmvdCnt is set equal to 0 and AffineBaseIdx  is set equal to 0. The following applies until AffMmvdCnt is equal to 1:

  - Derivation process for subblock-based motion candidate list as in 8.3.4.2 is invoked to generate subblockMergeCandList, and the number of total available candidate numCurrMergeCand is also generated;

  - When candidate subblockMergeCandList[ AffineBaseIdx ] is not equal to SbCol, the following applies with X being 0 or 1 until AffineBaseIdx is equal to numCurrMergeCand or AffMmvdCnt is equal to 0:

      - The following assignments are made with N being the candidate at position AffineBaseIdx in the subblock merging candidate list subblockMergeCandList ( N = subblockMergeCandList[ AffineBaseIdx ] ):

refIdxLX[ 0 ][ 0 ] = refIdxLXN		(8-xxx)

predFlagLX[ 0 ][ 0 ] = predFlagLXN	(8-xxx)

cpMvLX[ 0 ] = cpMvLXN[ 0 ]		(8-xxx)

cpMvLX[ 1 ] = cpMvLXN[ 1 ]		(8-xxx)

cpMvLX[ 2 ] = cpMvLXN[ 2 ]		(8-xxx)

numCpMv = motionModelIdxN + 1	(8-xxx)

gbiIdx = gbiIdxN		(8-xxx)

      - Derivation process for affine merge motion vector difference as specified in 8.3.4.11 is invoked, with predFlagLX[ 0 ][ 0 ] and refIdxLX[ 0 ][ 0 ] as input and affine merge motion vector difference cpmMvdLX as output, with X being 0 or 1.

      - The following assignments are made:

cpMvLX[ 0 ][ 0 ] += cpmMvdLX[ 0 ]	(8-xxx)

cpMvLX[ 0 ][ 1 ] += cpmMvdLX[ 1 ] 	(8-xxx)

cpMvLX[ 1 ][ 0 ] += cpmMvdLX[ 0 ] 	(8-xxx)

cpMvLX[ 1 ][ 1 ] += cpmMvdLX[ 1 ] 	(8-xxx)

cpMvLX[ 2 ][ 0 ] += cpmMvdLX[ 0 ] 	(8-xxx)

cpMvLX[ 2 ][ 1 ] += cpmMvdLX[ 1 ] 	(8-xxx)

AffMmvdCnt += 1	 	(8-xxx)

  - Otherwise, the following applies:

AffineBaseIdx += 1	 	(8-xxx)

- Otherwise (affine_mmvd_flag[ xCb ][ yCb ] is equal to 0), AffMmvdCnt is set equal to 0.

- When AffMmvdCnt is greater than 0, the following applies:

  - For xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1, the following applies:

predFlagLX[ xSbIdx ][ ySbIdx ] = predFlagLX[ 0 ][ 0 ]	(8-xxx)

  - When predFlagLX[ 0 ][ 0 ] is equal to 1, the derivation process for motion vector arrays from affine control point motion vectors as specified in subclause 8.3.4.9 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma prediction block height cbHeight, the number of control point motion vectors numCpMv, the control point motion vectors cpMvLX[ cpIdx ] with cpIdx being 0..2, and the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY as inputs, the luma subblock motion vector array mvLX[ xSbIdx ][ ySbIdx ] and the chroma subblock motion vector array mvCLX[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1 as outputs.

  - The following assignment is made for x = xCb ..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1:

MotionModelIdc[ x ][ y ] = numCpMv − 1	(8-xxx)

#### Derivation process for affine merge motion vector difference

Inputs to this process are:

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Outputs to this process are:

- Affine control point motion vector differences cpmMvdLX[ cpIdx ] with cpIdx = 0 .. numCpMv − 1 and X being 0 or 1.

The variable currPic specifies the current picture.

The process of deriving luma affine merge motion vector difference cpMvdL0 and cpMvdL1 is as follows:

- If both predFlagL0 and predFlagL1 are equal to 1, the following applies:

  - The variables currPocDiffL0 and currPocDiffL1 are derived as following:

currPocDiffL0 = DiffPicOrderCnt( currPic, RefPicList0[ refIdxL0 ] )	(8-xxx)

currPocDiffL1 = DiffPicOrderCnt( currPic, RefPicList1[ refIdxL1 ] )	(8-xxx)

  - If −currPocDiffL0 * currPocDiffL1 is greater than 0, the following applies:

cpmMvdL0[ 0 ] = AffineMmvdOffset[ 0 ]	(8-xxx)

cpmMvdL0[ 1 ] = AffineMmvdOffset[ 1 ]	(8-xxx)

cpmMvdL1[ 0 ] = − AffineMmvdOffset[ 0 ]	(8-xxx)

cpmMvdL1[ 1 ] = − AffineMmvdOffset[ 1 ]	(8-xxx)

  - Otherwise ( −currPocDiffL0 * currPocDiffL1 is less than 0 ), the following applies:

cpmMvdL0[ 0 ] = MmvdOffset[ 0 ]	(8-xxx)

cpmMvdL0[ 1 ] = MmvdOffset[ 1 ]	(8-xxx)

cpmMvdL1[ 0 ] = MmvdOffset[ 0 ]	(8-xxx)

cpmMvdL1[ 1 ] = MmvdOffset[ 1 ]	(8-xxx)

- Otherwise, if predFlagL0 is equal to 1 and predFlagL1 is equal to 0, the following applies:

cpmMvdL0[ 0 ] = AffineMmvdOffset[ 0 ]	(8-xxx)

cpmMvdL0[ 1 ] = AffineMmvdOffset[ 1 ]	(8-xxx)

cpmMvdL1[ 0 ] = 0		(8-xxx)

cpmMvdL1[ 1 ] = 0		(8-xxx)

- Otherwise (if predFlagL0 is equal to 0 and predFlagL1 is equal to 1), the following applies:

cpmMvdL0[ 0 ] = 0		(8-xxx)

cpmMvdL0[ 1 ] = 0		(8-xxx)

cpmMvdL1[ 0 ] = AffineMmvdOffset[ 0 ]	(8-xxx)

cpmMvdL1[ 1 ] = AffineMmvdOffset[ 1 ]	(8-xxx)

### Derivation process for intra prediction mode in combined merge and intra prediction

Input to this process are:

…

#### General

…

| Table 9-4 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| … |  |  |  |
| merge_data( ) | mmvd_direction_idx[ ][ ] | FL | cMax = 3 |
|  | ciip_flag[ ][ ] | FL | cMax = 1 |
|  | ciip_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | ciip_luma_mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | merge_subblock_flag[ ][ ] | FL | cMax = 1 |
|  | affine_mmvd_flag[ ][ ] | FL | cMax = 1 |
|  | affine_mmvd_distance_idx[ ][ ] | TR | cMax = 4 |
|  | affine_mmvd_direction_idx[ ][ ] | FL | cMax = 3 |
|  | merge_subblock_idx[ ][ ] | TR | cMax = MaxNumSubblockMergeCand − 1, cRiceParam = 0 |
|  | merge_triangle_flag[ ][ ] | FL | cMax = 1 |
|  | … |  |  |

…

##### 9.5.4.2.1General

…

| Table 9-10 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| … |  |  |  |  |  |  |
| inter_affine_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| cu_affine_type_flag[ ][ ] | 0 | na | na | na | na | na |
| affine_mmvd_flag[ ][ ] | 0 | na | na | na | na | na |
| affine_mmvd_distance_idx[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| affine_mmvd_direction_idx[ ][ ] | 0 | 1 | na | na | na | na |
| ref_idx_l0[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| ref_idx_l1[ ][ ] | 0 | 1 | bypass | bypass | bypass | bypass |
| mvp_l0_flag[ ][ ] | 0 | na | na | na | na | na |
| mvp_l1_flag[ ][ ] | 0 | na | na | na | na | na |
| … |  |  |  |  |  |  |

…