| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0431 |
| --- | --- |

| Title: | CE2: Affine merge with prediction offset (Test CE2.2.4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This document reports test results of CE2.2.4 – “Affine merge with prediction offset” (Affine MMVD). Similar to MMVD, the proposed method selects an affine merge candidate as a base candidate, and applies distance offset on selected offset direction for the control point motion vectors (CPMVs). There are four subtests in CE2.2.4:

a) Affine merge with offsets and block level signaled;

b) Affine merge with offset and extension of value range;

c) POC distance based offset mirroring for Bi-prediction;

d) POC distance based offset scaling for Bi-prediction.

VTM-3.0 is used as anchor in tests. Summaries of test results are listed below:

- CE2.2.4.a: -0.16% on RA, 0.01% on LDB;

- CE2.2.4.b: -0.17% on RA, -0.02% on LDB;

- CE2.2.4.c: -0.20% on RA, 0.04% on LDB;

- CE2.2.4.d: -0.20% on RA, 0.02% on LDB.

# Introduction

In VTM-3.0, sub-block based merge list is used. It includes SbTMVP candidate and affine merge candidates.

Affine merge with prediction offset was previously proposed in JVET-L0320. This test focused on one of the proposed method methods, block level signaling of the prediction offset.

# Proposed

## Affine merge with offsets

The proposed method selects the first available affine merge candidate from the sub-block based merge list as a base predictor. It applies a motion vector offset to each control point’s motion vector value from the base predictor. If there’s no affine merge candidate available, the proposed method will not be used. A distance index and an offset direction index are subsequently signaled.

Distance index is signaled to indicate which distance offset to use from the offset table.

Table 2.1 -Default Distance-offset Table

| Distance IDX | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| Distance-offset | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel |

The direction index can represent four directions as shown below, where only x or y direction may have an MV difference, but not in both directions.

Table 2.2 -Default Offset-direction Table

| Offset Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-dir-factor | +1 | –1 | 0 | 0 |
| y-dir-factor | 0 | 0 | +1 | –1 |

If the inter prediction is Uni-prediction, the signaled distance offset is applied on the offset direction for each control point predictor. Results will be the MV value of each control point.

If the inter prediction is Bi-prediction, the signaled distance offset is applied on the signaled offset direction for control point predictor’s L0 motion vector; the offset to be applied on L1 MV is applied on a mirrored or a scaled basis, specified in each subtest.

## Subtests

### CE 2.2.4.a Affine merge with offsets and block level signaled

In this subtest, the default distance-offset table is used, and the distance index and direction index are signaled on block level. The same distance offset and offset direction are applied on all available control points for the block.

If the inter prediction is Bi-prediction, the signaled distance offset is applied on the signaled offset direction for control point predictor’s L0 motion vector. For L1 CPMV, the offset is applied on a mirrored basis, which means the same amount of distance offset with the opposite direction is applied.

Table 2.3 - Complexity Analysis for CE2.2.4.a

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of pruning stages | Max number of MV scaling | Others |
| --- | --- | --- | --- | --- | --- |
| No Change | +20 | No Change | No Change | No Change |  |

### CE 2.2.4.b Affine merge with offset and extension of value range

This subtest is based on CE2.2.4.a, in addition, the distance offset value range is extended into 3 offset tables. Each offset table is adaptively selected based on the picture resolution.

Table 2.4 -Extended Distance-offset Table

| Distance IDX | 0 | 1 | 2 | 3 | 4 | Condition |
| --- | --- | --- | --- | --- | --- | --- |
| Distance-offset 1 | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | Picture Height >= 1080 |
| Distance-offset 2 | 1/8-pel | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 720 <= Picture Height < 1080 |
| Distance-offset 3 | 1/16-pel | 1/8-pel | 1/4-pel | 1/2-pel | 1-pel | Picture Height < 720 |

Table 2.5 - Complexity Analysis for CE2.2.4.b

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of pruning stages | Max number of MV scaling | Others |
| --- | --- | --- | --- | --- | --- |
| No Change | +20 | No Change | No Change | No Change |  |

### CE 2.2.4.c POC distance based offset mirroring for Bi-prediction

### This test is based on CE2.2.4.b, and in addition, a POC distance based offset mirroring method is used for Bi-prediction. When the base candidate is bi-predicted, the offset applied to L0 is as signaled, and the offset on L1 depends on the temporal position of the reference pictures on list 0 and list 1.

### If both reference pictures are on the same temporal side of the current picture, the same distance offset and same offset directions are applied for CPMVs of both L0 and L1;

### When the two reference pictures are on different sides of the current picture, the CPMVs of L1 will have the distance offset applied on the opposite offset direction.

Table 2.6 - Complexity Analysis for CE2.2.3.d

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of pruning stages | Max number of MV scaling | Others |
| --- | --- | --- | --- | --- | --- |
| No Change | +20 | No Change | No Change | No Change |  |

### CE 2.2.4.d POC distance based offset scaling for Bi-prediction

### This test is based on CE2.2.4.b, and in addition, a POC distance based offset scaling method is used for Bi-prediction. When the base candidate is bi-predicted, the offset applied to L0 is as signaled, and the offset on L1 will be scaled based on the temporal distance of reference pictures on list 0 and list 1.

Table 2.7 - Complexity Analysis for CE2.2.3.d

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of pruning stages | Max number of MV scaling | Others |
| --- | --- | --- | --- | --- | --- |
| No Change | +20 | No Change | No Change | No Change |  |

# Experimental Results

Proposed methods in this CE are implemented based on VTM-3.0. The simulations were conducted following CE2 description [3] and JVET common test conditions [4]. VTM-3.0 is used as anchor for all tests.

Table 3.1 Result of CE2.2.4.a – Affine merge with offsets and block level signaled

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.17% | -0.20% | 103% | 100% |  |  |  |  |  |
| Class A2 | -0.31% | -0.29% | -0.21% | 104% | 100% |  |  |  |  |  |
| Class B | -0.16% | -0.09% | -0.17% | 104% | 100% | -0.06% | -0.26% | -0.11% | 106% | 101% |
| Class C | 0.02% | -0.03% | 0.01% | 104% | 99% | 0.03% | 0.35% | 0.20% | 107% | 100% |
| Class E |  |  |  |  |  | 0.10% | -0.25% | -0.35% | 107% | 98% |
| Overall | -0.16% | -0.13% | -0.14% | 104% | 100% | 0.01% | -0.05% | -0.07% | 107% | 100% |
| Class D | 0.05% | 0.03% | -0.05% | 105% | 101% | 0.13% | 0.32% | 0.30% | 107% | 99% |
| Class F | 0.00% | -0.06% | -0.03% | 104% | 99% | -0.06% | 0.18% | -0.14% | 106% | 99% |

Table 3.2 Result of CE2.2.4.b – Affine merge with offset and extension of value range

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.17% | -0.20% | 103% | 100% |  |  |  |  |  |
| Class A2 | -0.31% | -0.29% | -0.21% | 104% | 100% |  |  |  |  |  |
| Class B | -0.16% | -0.09% | -0.17% | 104% | 100% | -0.06% | -0.26% | -0.11% | 105% | 101% |
| Class C | 0.00% | 0.00% | 0.05% | 104% | 100% | -0.09% | 0.20% | 0.13% | 108% | 101% |
| Class E |  |  |  |  |  | 0.16% | 0.50% | -0.16% | 106% | 98% |
| Overall | -0.17% | -0.12% | -0.13% | 104% | 100% | -0.02% | 0.09% | -0.04% | 106% | 100% |
| Class D | -0.09% | -0.02% | 0.10% | 105% | 101% | -0.12% | -0.42% | 0.18% | 106% | 102% |
| Class F | -0.05% | -0.03% | -0.02% | 104% | 99% | 0.01% | 0.00% | 0.38% | 104% | 98% |

Table 3.3 - Result of CE2.2.4.c – POC distance based offset mirroring for Bi-prediction

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.26% | -0.17% | 104% | 100% |  |  |  |  |  |
| Class A2 | -0.31% | -0.18% | -0.28% | 104% | 100% |  |  |  |  |  |
| Class B | -0.20% | -0.06% | -0.08% | 104% | 99% | -0.03% | -0.37% | 0.10% | 105% | 101% |
| Class C | -0.05% | 0.10% | 0.09% | 104% | 100% | -0.01% | 0.27% | 0.10% | 106% | 101% |
| Class E |  |  |  |  |  | 0.23% | 0.22% | 0.45% | 105% | 98% |
| Overall | -0.20% | -0.08% | -0.09% | 104% | 100% | 0.04% | -0.01% | 0.19% | 105% | 100% |
| Class D | -0.08% | -0.03% | 0.09% | 105% | 99% | -0.11% | 0.13% | 0.34% | 105% | 99% |
| Class F | -0.03% | -0.05% | -0.04% | 104% | 100% | -0.17% | -0.09% | -0.24% | 105% | 98% |

Table 3.4 - Result of CE2.2.4.d – POC distance based offset scaling for Bi-prediction

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.29% | -0.12% | 103% | 100% |  |  |  |  |  |
| Class A2 | -0.33% | -0.18% | -0.27% | 104% | 100% |  |  |  |  |  |
| Class B | -0.19% | -0.06% | -0.08% | 104% | 99% | -0.05% | -0.27% | 0.09% | 105% | 101% |
| Class C | -0.05% | 0.07% | 0.08% | 104% | 99% | -0.08% | 0.15% | 0.03% | 105% | 98% |
| Class E |  |  |  |  |  | 0.26% | 0.52% | 0.08% | 107% | 98% |
| Overall | -0.20% | -0.09% | -0.09% | 104% | 99% | 0.02% | 0.07% | 0.07% | 106% | 99% |
| Class D | -0.07% | -0.09% | 0.05% | 106% | 101% | -0.14% | 0.39% | -0.06% | 105% | 99% |
| Class F | -0.04% | -0.03% | -0.01% | 104% | 100% | -0.15% | -0.13% | -0.92% | 103% | 98% |

# Conclusion

It was reportedly observed that CE2.2.4.b has 0.17% BD-rate gain on Random Access configuration, and 0.02% BD-rate gain on Low Delay B configuration, with no decoder side complexity impact.

It is proposed to adopt CE2.2.4.b into VVC and VTM.

# Reference

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3),” JVT-L1002, Macao, CN, Oct 2018.

- G. Li, X. Xu, X. Li, S. Liu, “CE4-related: affine merge mode with prediction offsets,” JVET-L0320, Macao, CN, Oct 2018.

- Y. He, C.-Y. Chen, C.-C Chen, “Description of Core Experiment 2 (CE2): Sub-block based motion prediction,” JVT-L1024, Macao, CN, Oct 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010 Macao, CN, Oct 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).