| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0434 |
| --- | --- |

| Title: | CE2-related: Constraint on constructed affine merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this document, a constraint on constructed affine merge candidates is proposed to limit the constructed affine model parameters such that the control point motion vectors are within a reasonable range. The proposed method was implemented on VTM-3.0. It was reportedly observed that the method has negligible changes on BD-rate performance, at -0.02% on RA, and 0.00% on LDB.

# Introduction

In VTM-3.0, a sub-block based merge list is used, including the SbTMVP candidate and affine merge candidates. The maximum allowed size of the sub-block based merge list is 5. In affine merge candidates, there are 3 possible types of candidates as following:

- Inherited affine merge candidates, which inherit the affine model from neighboring block to generate control point motion vectors (CPMVs) for the current block;

- Constructed affine merge candidates, which are derived from translational motion vectors from neighboring blocks;

- Zero CPMVs.

In affine AMVP mode, there are 4 possible types of candidates for affine CPMV predictors:

- Inherited affine AMVP candidates, which inherit the affine model from neighboring block to generate control point motion vector predictors (CPMVPs) for the current block;

- Constructed affine AMVP candidates, which are derived from translational motion vectors from neighboring blocks;

- Translational MVs from neighboring blocks;

- Zero MVs.

Figure 1 - Constructed Affine control point motion vectors

For both constructed affine merge and AMVP, due to the fact that the CPMVs may be derived from neighboring blocks with large difference on motion information, there’s possibility that the constructed CPMVs represent an affine model consists of affine parameters with very large values. In such case, the candidate’s CPMV may be impractical to use. More importantly, when such a CPMV is selected by the encoder for a block, the very large CPMV may be used as the MV predictor for the future blocks. Such a propagation can trigger large MVD values which further leads CABAC coding failure.

Figure 2 - 6-parameter affine model

Affine models can be described by the following equations.

For 6-parameter affine, with affine parameters a,b, c, d, e, f, Motion vectors may be represented as:

MVx=ax+by+cMVy=dx+ey+f											(1)

For the three control points of the block, their CPMVs can be denoted as:

CPMV0:c, f

CPMV1:a∙W+c, d∙W+f

CPMV2:(b∙H+c, e∙H+f)

Where W and H denotes the block width and height, respectively.

Figure 3 - 4-parameter affine model

For 4-parameter affine, with affine parameters a, b, c, f, Motion vectors may be represented as:

MVx=   ax+by+cMVy=-bx+ay+f										(2)

For the two control points of the block, their CPMVs can be denoted as:

CPMV0:c, f

CPMV1:a∙W+c, -b∙W+f

Where W and H denotes the block width and height, respectively.

Take 4-parameter affine case as example, the affine parameter a and b may also be considered as a combination of rotation parameter and scaling parameter, such that:

a=ρ∙cosθ												(3)

b=ρ∙sinθ												(4)

Since the value of cosθ and sinθ are in the range of [-1, 1], the value range of a and b are primarily reflecting the range of the scaling parameter ρ. Practically the scaling parameter for a block could not be too large, as it’s reflecting the scaling of an object between the reference picture and the current picture.

# Proposed

It’s proposed to apply a constraint on the range of the affine parameter values for the constructed affine merge candidates, so that candidates with impractical affine models will not be used.

Take 4-parameter affine as an example. CPMV0 only has the translational portions of the affine model, which are represented by c and f. By subtracting the horizontal and vertical components of CPMV1 from CPMV0 separately, the delta part will be consists of only the affine parameters and the block width:

MVx1-MVx0=a∙WMVy1-MVy0=-b∙W								(5)

Getting the ratio of the absolute values of deltas in (5) with the width, it can represent the range of the values of affine parameters a and b:

R1x=MVx1-MVx0W=aR1y=MVy1-MVy0W=b								(6)

Similarly, for 6-parameter affine, we can derive the ratios as:

R1x=MVx1-MVx0W=aR1y=MVy1-MVy0W=dR2x=MVx2-MVx0H=bR2y=MVy2-MVy0H=e								(7)

In this proposal, the threshold to limit the maximum ratio in {R1x, R1y, R2x, R2y} is set to 2.

maxR1x,R1y,R2x,R2y<2							(8)

If any of the derived ratio is not less than the threshold, the constructed affine merge/AMVP candidate will be considered as invalid and will not be used.

# Experimental Results

The proposed method is implemented based on VTM-3.0. The simulations were conducted following JVET common test conditions [2]. VTM-3.0 is used as anchor for the test.

Table 3.1 Simulation Result

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.06% | 98% | 100% |  |  |  |  |  |
| Class A2 | -0.04% | 0.08% | 0.11% | 99% | 100% |  |  |  |  |  |
| Class B | -0.02% | 0.04% | 0.03% | 98% | 96% | 0.01% | -0.32% | 0.05% | 101% | 101% |
| Class C | -0.01% | -0.01% | 0.05% | 99% | 97% | 0.00% | 0.34% | 0.14% | 99% | 97% |
| Class E |  |  |  |  |  | 0.00% | -0.04% | -0.26% | 100% | 97% |
| Overall | -0.02% | 0.02% | 0.06% | 98% | 98% | 0.00% | -0.03% | 0.00% | 100% | 99% |
| Class D | 0.00% | -0.02% | 0.08% | 100% | 100% | 0.05% | 0.26% | 0.07% | 100% | 96% |
| Class F | 0.00% | -0.04% | -0.03% | 99% | 98% | 0.07% | 0.12% | 0.06% | 99% | 96% |

# Conclusion

It was observed that the proposed constraint has negligible impact on coding performance. BD-rate change is -0.02% and 0.00% on Random Access and Low Delay B configurations, respectively. The constraint can avoid impractical affine models to be used, and potentially avoid large motion vector values or large MVD values to be generated by propagations and resulting CABAC coding failure. It’s proposed to adopt this constraint into VVC and VTM.

# Reference

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3),” JVT-L1002, Macao, CN, Oct 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010 Macao, CN, Oct 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).