| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0449-v1 |
| --- | --- |

| Title: | CE8-related: BDPCM entropy coding with reduced number of context coded bins |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Meng Xu, Xiang Li, Xiaozhong Xu, Min Gao, Shan Liu | Tel: Email: | {mengxxu, xlxiangli, xiaozhongxu, maxwellgao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Block DPCM (BDPCM) is tested in CE8.3 and shown noticeable gains. For each residue, up to 12 bins are coded with context. A restriction is proposed to improve the entropy coding throughput, where up to 2 context coded bins are allowed for each residue on average. It shows almost no loss on CTC for AI and RA. It shows 0.09% and 0.13% BD-rate loss on average for class F in AI and RA configurations, respectively, and 0.12% and 0.32% BD-rate loss on average for class SCC in AI and RA configurations, respectively.

# Introduction

Block DPCM (BDPCM) is tested in CE8.3 and have shown noticeable coding performance gain on the screen content. In BDPCM, the residual is not applied with transform, and coded differently with a transformed block. For each residue, the absolute value is coded with up to 12 bins coded with context. The remaining bins and the sign are coded with equal probability. It may be a big burden for the entropy coder in the hardware codec.

# Proposed method

To improve the syntax parsing throughput, it is proposed to restrict with up to 2 context coded bins for each BDPCM residue. That is, for an MxN block in a single luma or chroma channel, only the first 2xMxN bins of the absolute residue are coded with context, and the remaining bins are coded with equal probability. Note that this is in line with current coefficient coding where up to 32 context coded bins are allowed for each 4x4 coefficient group.

# Simulation results

The proposed method is implemented on top of CE8.3.2 on top of PLT in CE8.2.1.

1st Reference: VTM-3.0

2nd Reference: CE8.3.2 + CE8.2.1

Test: CE8.3.2 + CE8.2.1 + proposed method

Compared with CE8.3.2 with PLT, it shows almost no loss on average for AI and RA. It shows 0.09% and 0.13% BD-rate loss on average for class F in AI and RA configurations, respectively. It shows 0.12% and 0.32% BD-rate loss on average for class SCC in AI and RA configurations, respectively.

# Conclusion

The entropy coding throughput for BDPCM residue coding is improved from 12 context coded bins per coefficient to 2 context coded bins per coefficient with the proposed method. It shows almost no loss on CTC for AI and RA. It shows 0.09% and 0.13% BD-rate loss on average for class F in AI and RA configurations, respectively, and 0.12% and 0.32% BD-rate loss on average for class SCC in AI and RA configurations, respectively. It is suggested to adopt this proposal.

# References

- “JVET common test conditions and software reference configurations for SDR video,” F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET-L1010.

- “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, JVET-L1028.

- “Versatile Video Coding (Draft 3)”, B. Bross, J. Chen, S. Liu, JVET-L1001.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).