| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0461-v2 |
| --- | --- |

| Title: | CE2-related: Alternate ALF filter shapes for luma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Daniel Socek Atul Puri | Tel: Email: | daniel.socek@intel.com atul.puri@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

In the contribution, it is proposed to modify luma filter shape for adaptive loop filter (ALF). In VTM3.0, the ALF shape for luma is fixed to 7x7 diamond (25 taps and 13 unique coefficients). Two modified luma ALF shapes are considered: (1) combined 7x7 diamond and 9x9 cross filter with fully symmetric outermost cross coefficient (29 taps and 14 unique coefficients, Figure 1a), and (2) combined 7x7 diamond and 9x9 cross filter with fully symmetric two outermost cross coefficients (29 taps and 13 unique coefficients).  Alternate shapes are combined with TC offsets of -2 for LB and RA, and -6 for AI. It is reported that, compared to VTM3.0, the first alternate ALF luma shape (1) yields -0.57%, -0.17%, and -0.36% luma BD-rates for AI, RA, and LB, respectively, while the second ALF luma shape (2) yields -0.57%, -0.15%, and -0.34% luma BD-rates for AI, RA, and LB, respectively.  There are minor runtime changes in the performed tests.

# Proposed methods

In VTM3.0, there is a single luma ALF filter used: 7x7 diamond with 25 taps and 13 unique coefficients (Figure 1). In this contribution, two tests are evaluated and presented. In the first test, the reference luma shape is replaced with the modified luma shape: combined 7x7 diamond and 9x9 cross filter with fully symmetric outermost cross coefficient, 29 taps and 14 unique coefficients (Figure 2a). In the second test, the reference luma shape is replaced with the second modified luma shape: combined 7x7 diamond and 9x9 cross filter with fully symmetric two outermost cross coefficients, 29 taps and 13 unique coefficients (Figure 2b).

Figure 1. VTM3.0 ALF filter shape for luma

Figure 2. The proposed alternate ALF filter shapes for luma

The for the above experiments TC offsets of -2 for LB and RA, and -6 for AI were used, as in [1].  Table 1 shows comparison between the tests.

Table 1. Summary of the performed tests

|  | Number of taps in luma ALF filter | Number of unique coefficients in luma ALF filter |
| --- | --- | --- |
| VTM3.0 | 25 | 13 |
| Test 1 | 29 | 14 |
| Test 2 | 29 | 13 |

# Experimental results

The proposed methods have been implemented on top of VTM3.0 [2], including SIMD optimizations.  Table 2 and Table 3 show results of the two alternate ALF luma shapes, with 14 and 13 unique coefficients, respectively. For the purposes of experiments both the anchor as well as the tested ALF filters used scalar mode without SIMD optimizations for ALF (rest of the codec had SIMD optimization enabled). A future version of this document will include runtimes with SIMD optimizations enabled for ALF (both the reference and our experiment).

# Conclusions

In this contribution, changing the shape of ALF filter for luma to the proposed filter from Figure 1a achieves -0.57%, -0.17%, and -0.36% luma BD-rates for AI, RA, and LB, respectively, while replacing the shape of ALF filter for luma to the proposed filter from Figure 1b yields -0.57%, -0.15%, and -0.34% luma BD-rates for AI, RA and LB, respectively.  There is no major runtime change.

# Patent rights declaration

Intel Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] M. Karczewicz, N. Shlyakhov, N. Hu, V. Seregin, W.-J. Chien, “CE2.4.1.4: Reduced filter shape size for ALF,” Document of Joint Video Experts Team, JVET-K0371.

[2] VTM3.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

Table 2. Results for alternate luma ALF filter shape from Figure 2a

Table 3. Results for alternate luma ALF filer shape from Figure 2b