| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0469 |
| --- | --- |

| Title: | CE7-related: unified Rice parameter derivation for coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Muhammed Coban | Tel: Email: | martak@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a unified Rice parameter derivation for coding of abs_remainder and dec_abs_level syntax elements by introducing HEVC style Rice parameter derivation that is used for coding of abs_remainder syntax element for coding of dec_abs_level syntax element instead of current local template- based Rice parameter derivation. The simulations results show -0.02% AI, -0.04% RA, 0.XX% LB BD-Rate versus the VTM-3.0 anchor for common test conditions for Rice parameter derivation method.

# Introduction

In VTM-3.0 the coefficients that are completely coded in the bypass mode of the arithmetic coding engine using a Golomb-Rice code, i.e. dec_abs_level syntax element; the Rice parameter is derived based on the sum of absolute levels in a local template around the current transform coefficient level. The mapping of quantization index 0 depends on the state (for dependent quantization) and the sum of absolute levels in a local template. The Rice parameter for coding the remainder (for coefficients that are not completely coded in bypass mode), i.e. abs_remainder syntax element, is derived similar to HEVC without referring to neighboring transform coefficient levels [1].

# Proposal

The proposal is integrated on top of CE7.4 SW, that incorporates gt2 flag (abs_level_gt3_flag) into the first coding pass [4].

## Rice parameter derivation

The Rice parameter ricePar for coding the non-binary syntax element dec_abs_level is derived similar to HEVC:

- At the end of second pass for decoding of abs_remainder fields, ricePar is set equal to min(ricePar>0 ? ricePar +1 : lrg2Coded, 3), where lrg2Coded is a flag that is set to 1 once a gt2 flag equal to 1 is encountered in the first pass.

ricePar   = min(ricePar>0 ? ricePar +1 : lrg2Coded, 3)

- Before coding a syntax element remainder, the Rice parameter ricePar is modified as follows:
If ricePar is less than 3 and the last coded value of absolute coefficient level is greater than ( (3 << ricePar) – 1 ), ricePar is incremented by 1.

For coding the non-binary syntax element absLevel[k], i.e., the absolute quantization indexes that are completely bypass-coded, the following applies:

- The ricePar is determined by the method described above.

- posZero is determined by a table look-up according to

posZero = posZeroTable[ max( 0, state – 1 ) ][ ricePar ]

where the variable state represent the state for dependent quantization (it is equal to 0 when dependent quantization is disabled) and the table posZeroTable[][] is given by

posZeroTable[3][4] = {{1,2,4,8},{3,6,12,16},{4,6,12,16}}

- The intermediate variable codeValue is derived as follows:

  - If absLevel[k] is equal to 0, codeValue is set equal to posZero;

  - Otherwise, if absLevel[k] is less than or equal to posZero, codeValue is set equal to absLevel[k] – 1;

  - Otherwise (absLevel[k] is greater than posZero), codeValue is set equal to absLevel[k].

- The value of codeValue is coded using a Golomb-Rice code with Rice parameter ricePar.

At the decoder side, the non-binary syntax elements absLevel[k] are decoded as follows:

- The variables ricePar and posZero are determined as described above;

- The intermediate value codeValue is decoded using a Golomb-Rice code with Rice parameter ricePar;

- The value of absLevel[k] is derived as follows:

  - If codeValue is equal to posZero, absLevel[k] is set equal to 0;

  - Otherwise, if codeValue is less than posZero, absLevel[k] is set equal to codeValue + 1;

  - Otherwise (codeValue is greater than or equal to posZero), absLevel[k] is set equal to codeValue.

## Partial coefficient representation in first coding pass

In the first coding pass, sig_coeff_flag, abs_level_gt1_flag, par_level_flag and abs_level_gt3_flag information is coded until less than 4 regular coded bin remains until reaching the number of regular coded bins limit [2]. This condition is modified to accommodate also only coding of sig_coeff_flag and abs_level_gt1_flag for the last coefficient in the first pass within the regular bin count limit. The remainder part of the partial coded last coefficient in the first pass is coded with the coefficients whose dec_abs_level values are coded.

# Results

The proposed scheme is implemented on top of VTM-CE7.4 software [4].  The simulations were carried out under the JVET common test conditions [3][5].

The test results for JVET common test conditions is tabulated in Table 1 for Rice parameter derivation method.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 120% | 122% |
| Class A2 | -0.06% | -0.03% | -0.10% | 118% | 121% |
| Class B | -0.01% | -0.05% | -0.05% | 97% | 100% |
| Class C | -0.04% | -0.07% | -0.19% | 96% | 99% |
| Class E | 0.03% | -0.01% | 0.13% | 98% | 98% |
| Overall | -0.02% | -0.04% | -0.05% | 104% | 106% |
| Class D | 0.01% | -0.03% | -0.06% | 95% | 97% |
| Class F | 0.23% | 0.36% | 0.28% | 97% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.08% | 0.03% | 120% | 118% |
| Class A2 | -0.05% | 0.06% | 0.09% | 119% | 115% |
| Class B | -0.04% | 0.08% | -0.10% | 93% | 96% |
| Class C | -0.02% | -0.20% | -0.05% | 91% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.03% | -0.02% | 102% | 104% |
| Class D | -0.03% | -0.69% | -0.29% | 90% | 91% |
| Class F | 0.21% | 0.19% | 0.44% | 85% | 87% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.01% | 0.15% | 0.02% | 96% | 92% |
| Class E | 0.00% | 1.26% | -0.24% | 102% | 84% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.06% | 0.04% | 0.23% | 97% | 90% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 1: Average Bjøntegaard delta rate savings of the proposed method 2.1 relative to VTM-3.0.

The test results for combination of Rice parameter derivation in combination with partial coefficient representation fo JVET common test conditions is tabulated in Table 2.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.06% | 0.04% | 120% | 122% |
| Class A2 | -0.07% | -0.05% | -0.05% | 118% | 121% |
| Class B | -0.02% | -0.05% | -0.03% | 97% | 100% |
| Class C | -0.05% | -0.08% | -0.23% | 96% | 99% |
| Class E | 0.02% | -0.06% | 0.06% | 98% | 98% |
| Overall | -0.03% | -0.06% | -0.05% | 104% | 106% |
| Class D | -0.03% | -0.05% | -0.01% | 95% | 97% |
| Class F | 0.11% | 0.06% | 0.12% | 97% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.03% | 0.00% | 120% | 118% |
| Class A2 | -0.05% | 0.08% | 0.06% | 119% | 115% |
| Class B | -0.03% | 0.16% | -0.04% | 93% | 96% |
| Class C | -0.04% | -0.07% | 0.09% | 91% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.06% | 0.02% | 102% | 104% |
| Class D | -0.08% | -0.41% | -0.18% | 90% | 91% |
| Class F | 0.10% | 0.18% | 0.18% | 85% | 87% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | 0.17% | 0.02% | 96% | 92% |
| Class E | -0.03% | 1.52% | 0.84% | 102% | 84% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.04% | 0.40% | 97% | 90% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 2: Average Bjøntegaard delta rate savings of the proposed method 2.1 relative to VTM-3.0.

# Conclusion

In this document a Rice parameter derivation method for coding of dec_abs_level syntax elements is presented that is based on previously decoded coefficient value. It eliminates the neighbor template based derivation of the Rice parameter and follows the HEVC design that is used in coding of abs_remainder syntax elements. Additionaly, support for partial coding of last coefficient in the first coding pass within the regular bin count limit is added.

# References

- H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand, M. Karzewicz, M. Coban, J. Dong, “CE7: Transform coefficient coding with reduced number of regular-coded bins (tests 7.1.3a, 7.1.3b)”, JVET-L0274, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-L1027, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

- https://vcgit.hhi.fraunhofer.de/JVET-L-CE7/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).