| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0473-v2 |
| --- | --- |

| Title: | Simplified HMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Weijia Zhu Andrew Segall | Tel: Email: | +1-360-834-8054 zhuw@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp Labs of America |  |  |

_____________________________

- Abstract

A history-based motion vector prediction (HMVP) method was adopted into the VVC draft in the 12th JVET meeting.  In the HMVP scheme, a HMVP table is maintained and updated on-the-fly with a pruning process.  As part of this pruning process, the motion information candidate is compared with all candidates in the HMVP table to determine if there is a redundancy. It is asserted here that this pruning process increases complexity.  In this proposal, two modifications to the HMVP scheme are proposed to reduce the complexity of the pruning process.  First, it is proposed to update the HMVP table using a 16x16 grid, and so only inter block located at these gird point will be added to the HMVP list.  This is asserted to reduce the complexity of the HMVP operation in the worst case.  Second, it is proposed to remove the reference index comparison during the HMVP pruning process.  Simulation results show that the combination of these two methods results in a 0.02% luma BD-Rate change compared to VTM 3 for both the random-access low-delay B configurations.

Note (v2): The concept of removing the reference index comparison when operating on HMVP candidates is further reported for inserting an HMVP candidate into the merge list.

Introduction

In the 12th JVET meeting, a history-based MVP (HMVP) method was adopted into the VVC working draft.  The method introduces an HMVP candidate that is defined as the motion information of a previously coded block in the current slice. A table with multiple HMVP candidates is maintained during the encoding/decoding process, and a pruning process is used to remove redundant candidates from the table that have the same motion information. Of course, the pruning process requires additional operations and increases complexity.  In this proposal, we proposed two modifications to reduce the complexity during the pruning process

Simplified HMVP

16x16 Grid

The first proposed modification is to reduce the number of table updates, which in turn reduces the number of pruning operations.  Here, we propose to limit the update to only once within a 16x16 HMVP gird. Thus, if an inter block located at the 16x16 HMVP, it will go through HMVP pruning process. Otherwise, the motion vector for the CU will not be added to the HMVP table. Fig.1 shows an illustration of the proposed modification, where the black lines are the gird boundary. In the Figure, the red block will go through the pruning process. And if it is not a redundant candidate, it will be added to the HMVP table. The blue block will never be added to the HMVP table since it is not located at the grid.

Figure 1: An illustration of the first medication on HMVP

Reference Picture Index

A second proposal is designed to reduce the number of operations in the pruning process. In the current design, the pruning process requires the decoder to determine if a motion candidate is redundant – and this is accomplished by comparing the inter prediction direction, reference picture index and motion vectors between the candidate an entries in the list.  Here, we propose to remove the comparisons between the reference picture indexes of different candidates during the pruning process. Thus, only the inter prediction direction and motion vectors are compared.

Combined Analysis

Table 1 compares the number of comparisons without the proposed medication and with the proposed modifications in the worst case.  (In the worst case, the HMVP table is full thus there are 6 candidates in the HMVP list).

Table 1 Number of comparisons needed by HMVP and HMVP with the proposed modifications

|  | Number of comparisons |
| --- | --- |
| HMVP in VTM3.0 | 9*6=54 |
| The proposed modification | 7*6=42 |

Simulation Results

The objective performance is also reported by following the test condition in [2] on top of VTM3 [3]. Tables 2-3 report the simulation results with random-access and low-delay B configurations.

Simulation results show that the grid based HMVP update has a luma BD-rate change of 0.02% when compared with VTM 3 for both a random-access and low-delay B configurations, respectively.  Similarly, when removing the reference index comparison during the HMVP pruning there is a luma BD-rate change of 0.00% and 0.02% for the random-access and low-delay B configurations, respectively.

Finally, combining these two approaches results in a luma BD-rate change of 0.02 when compared with VTM 3 for both a random-access and low-delay B configurations, respectively.

Table 2 Coding Performance of 16x16 grid based HMVP update compared with VTM3

Table 3 Coding Performance of removing RefIdx comparison in HMVP pruning compared with VTM3

Table 4 Coding Performance of combining the first two modifications compared with VTM3

Conclusions

In this proposal, we propose three modifications to the HMVP process. First, we propose limiting updates to the HMVP candidate list to only CUs on a 16x16 grid.  Second, we propose to remove the reference index comparison during the HMVP pruning process. Third, we propose to apply the second modification on the pruning process when inserting a HMVP candidate to the merge list. The performance change of these three modifications is less than or equal to 0.02% compared with the VTM3 using the Common Test Conditions.  We propose to adopt these three changes into the next version of the VVC draft.

- References

- B. Bross, J. Chen, S. Liu “Versatile Video Coding (Draft 3)”, JVET-L1001, Oct. 2018, Macao, China.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, Oct. 2018, Macao, China.

- VTM 3, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_vtm.git

Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix - Additional Investigation

Given the observation that removing the reference index duplicate check provided a negligible impact on coding efficiency, we further evaluated the concept of removing the reference index check when inserting an HMVP candidate into the merge list. Thus, only the inter prediction direction and motion vectors are compared during that pruning process. Table A-1 compares the number of comparisons without the proposed modification and with the proposed modification in the worst case.

Table A-1 Number of comparisons needed when inserting a HMVP candidate to the merge list with and without the proposed modification

|  | Number of comparisons |
| --- | --- |
| HMVP in VTM3.0 | 4*6=24 |
| The proposed modification | 4*4=16 |

Simulation results show that when removing the reference index comparison during the pruning when inserting a HMVP candidate to the merge list there is a luma BD-rate change of 0.01% and 0.06% for the random-access and low-delay B configurations, respectively.

Table 3 Coding Performance of removing RefIdx comparison during pruning process when inserting a  HMVP candidate to the merge list (as compared to VTM3)