| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0476 |
| --- | --- |

| Title: | CE2: Control point MV offset for Affine merge mode (Test 2.2.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ciao Yang Yao-Jen Chang | Email: | yuciaoyang@fginnov.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of new Affine merge candidates based on the control points motion vectors (CPMVs) offsets of the first candidate in the Affine merge list in CE2.2.5. The new Affine merge candidates are created with different offset directions and different offset magnitudes to the first Affine merge candidate. In the experiment, 20 new CPMV offset candidates are added into the Affine merge list and the number of Affine merge candidates is increased from 5 to 25. Compared to VTM3.0, -0.06%/-0.02%/-0.05% YUV-BDrate with 104/101% coding time change are observed in RA condition, and -0.03%/0.06%/-0.03% YUV-BDrate with 105/99% coding time change are observed in LB condition.

Introduction

In JVET-L0389 [1], the Affine merge mode uses the MV information of the neighboring and temporal blocks to derive the Affine control points motion vectors (CPMVs) of the current CU. It was included into Core Experiment (CE) 2 [2] to evaluate the performance on top of common test conditions [3] in VTM3.0. The following table shows the description for CE test reported in this document:

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 2.2.5 | Affine MV offset merge candidates | JVET-L0389 | Y.-C. Yang (Foxconn) | H. Liu (Bytedance) |

The proposed Affine merge mode

New Affine merge candidates are generated based on the CPMVs offsets of the first Affine merge candidate. If the first Affine merge candidate enables 4-parameter Affine model, then 2 CPMVs for each new Affine merge candidate are derived by offsetting 2 CPMVs of the first Affine merge candidate; Otherwise (6-parameter Affine model enabled), then 3 CPMVs for each new Affine merge candidate are derived by offsetting 3 CPMVs of the first Affine merge candidate. In Uni-prediction, the CPMV offsets are applied to the CPMVs of the first candidate. In Bi-prediction with List 0 and List 1 on the same direction, the CPMV offsets are applied to the first candidate as follows:

MVnew(L0), i = MVold(L0) + MVoffset(i)

MVnew(L1), i = MVold(L1) + MVoffset(i)

In Bi-prediction with List 0 and List 1 on the opposite direction, the CPMV offsets are applied to the first candidate as follows:

MVnew(L0), i = MVold(L0) + MVoffset(i)

MVnew(L1), i = MVold(L1) - MVoffset(i)

In this contribution, various offset directions with various offset magnitudes are used to generate new Affine merge candidates. The following implementation was tested:

20 new Affine merge candidates corresponding to 4 different offset directions and 5 different offset magnitudes are generated as shown in the following offsets set:

Offset set = { ( 2, 0), (0, 2), (-2, 0), (0, -2),( -4, -4), (4, -4), (4, 4), (-4, 4),(8, 0), (0, 8), (-8, 0), (0, -8),           (-16, -16), (16, -16), (16, 16), (-16, 16),( 32, 0), (0, 32), (-32, 0), (0, -32) }.

The Affine merge list is increased to 25 for this design. The number of potential Affine merge candidates is 31 in total.

Experiment result

The proposed CPMV offset candidates are tested on top of VTM3.0 under the common test conditions specified in [2] and [3]. Table 1 shows the experimental results for 20 new Affine merge candidates with Affine merge list size = 25.

Table 1. Summary of experimental results (The proposed 20 new CPMV offset candidates with Affine merge list size = 25 v.s. VTM3.0 anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.13% | -0.02% | 105% | 102% |
| Class A2 | -0.11% | -0.01% | 0.00% | 105% | 106% |
| Class B | -0.08% | 0.05% | -0.15% | 106% | 99% |
| Class C | -0.03% | -0.04% | 0.01% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.02% | -0.05% | 104% | 101% |
| Class D | -0.10% | -0.06% | 0.11% | 105% | 100% |
| Class F (optional) | -0.06% | -0.04% | -0.02% | 103% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.20% | -0.11% | 105% | 101% |
| Class C | -0.03% | 0.20% | 0.10% | 104% | 97% |
| Class E | -0.04% | 0.31% | -0.07% | 104% | 100% |
| Overall | -0.03% | 0.06% | -0.03% | 105% | 99% |
| Class D | -0.12% | -0.12% | 0.45% | 105% | 101% |
| Class F (optional) | 0.03% | -0.37% | -0.29% | 103% | 103% |

Conclusion

This contribution proposes to generate new Affine merge candidates by adding CPMV offsets to the first candidate in the Affine candidate list. It is observed that the proposed CPMV offset with 20 new candidates under Affine merge list size =25 provides -0.06%/-0.02%/-0.05% YUV-BDrate with 104/101% coding time.

Reference

- Y.-C. Yang and Y.-J. Chang, “CE4-related: Control point MV offsets for Affine merge mode,” JVET-L0389, Macao, CN, Oct. 2018.

- Y. He, C.-Y. Chen and C.-C Chen, “Description of Core Experiment 2 (CE2): Sub-block based motion prediction,” JVET-L1022, Macao, CN, Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, Oct. 2018.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).