| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0479 |
| --- | --- |

| Title: | Non-CE4: On clipping of scaled motion vectors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Misra1, F. Bossen2 | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com frank@bossentech.com |
| Source: | 1Sharp Labs of America, 2Sharp Electronics of Canada |  |  |

_____________________________

# Abstract

It is reported that the current VVC WD uses a 16-bit clip for scaled motion vectors. However, the motion vectors in VVC WD make use 1/16-pel precision which limits the range of scaled motion vectors to [-2048, 2047 15/16]. The contribution proposes to increase the clipping range to 18-bits to allow the same motion vector range as HEVC for scaled motion vectors. The proposed change is asserted to be a bugfix for the VVC WD. The modification was implemented in the VTM-3.0 and its impact evaluated for the Random Access (RA) and Low Delay B (LDB) configuration of the Common Test Conditions. It is reported that the changes results in an average luma BD rate of 0.00% (RA), 0.00% (LDB).

# Introduction

The current VVC WD [1] uses a 16-bit clip for scaled motion vectors. The clipping range is asserted to be a legacy carry-over from HEVC. In HEVC motion vectors made use of 1/4-pel precision. As result the effective motion vector range after the clip was [-8192, 8191 3/4].

However, motion vectors in VVC WD make use 1/16-pel precision. As a result, the effective motion vector range after the 16-bit clip is: [-2048, 2047 15/16]

The contribution therefore proposes to increase the clipping range for scaled motion vectors from 16-bit to 18-bits to allow the same motion vector range as HEVC for the scaled motion vectors.

The VVC WD changes corresponding to this modification is listed in the Appendix of this document.

# Experimental Results

The proposed change is implemented in VTM-3.0 [3] and evaluated using Random Access (RA) and Low Delay B (LDB) configurations from the common test conditions [2]. The summary results are listed below. Detailed results can be found in excel file attached to this contribution.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.03% | 101% | 99% |
| Class A2 | 0.00% | 0.01% | 0.02% | 99% | 99% |
| Class B | -0.01% | 0.01% | -0.01% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | 0.00% | 98% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F (optional) | 0.01% | -0.07% | -0.13% | 106% | 100% |

# Recommendation

In the current VVC WD scaled motion vector are clipped to 16-bits. The contribution proposes to extend this range to 18-bits. The impact on coding efficiency of the proposed change is shown to be negligible. The proponents recommend that the proposed modification be adopted in the next version of the VVC WD and VTM.

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001 12th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, 3–12 Oct. 2018

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010 12th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, 3–12 Oct. 2018

[3] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-3.0 VTM-3.0

# Appendix

Modification to VVC WD are highlighted in yellow below:

#### 8.3.2.10	Derivation process for spatial motion vector predictor candidates

…

- When availableFlagLXA is equal to 1, DiffPicOrderCnt( refPicListA[ refIdxA ], RefPicListX[ refIdxLX ] ) is not equal to 0, mvLXA is derived as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-309)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-310)

mvLXA = Clip3( −32768 −131072, 32767 131071, Sign( distScaleFactor * mvLXA ) * 
				( ( Abs( distScaleFactor * mvLXA ) + 127 )  >>  8 ) )	(8-311)

…

- When availableFlagLXB is equal to 1, DiffPicOrderCnt( refPicListB[ refIdxB ], RefPicListX[ refIdxLX ] ) is not equal to 0, mvLXB is derived as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-325)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-326)

mvLXB = Clip3( −32768 −131072, 32767 131071, Sign( distScaleFactor * mvLXB ) *
				( ( Abs( distScaleFactor * mvLXB ) + 127 )  >>  8 ) )	(8-327)

…

#### 8.3.2.12	Derivation process for collocated motion vectors

…

      - Otherwise, mvLXCol is derived as a scaled version of the motion vector mvCol as follows:

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-337)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-338)

mvLXCol =  Clip3( −32768 −131072, 32767 131071, Sign( distScaleFactor * mvCol ) * 
				( ( Abs( distScaleFactor * mvCol ) + 127 )  >>  8 ) )	(8-339)

where td and tb are derived as follows:

td = Clip3( −128, 127, colPocDiff )	(8-340)

tb = Clip3( −128, 127, currPocDiff )	(8-341)

…

#### 8.3.3.3	Derivation process for temporal triangle merging candidates

…

- For N being equal to each of A1, B1, B0, A0, B2, ColC0 and ColC1 the following applies:

  - The variable tempMv is derived as follows:

  - If DiffPicOrderCnt( RefPicList0[ refIdxL0N ], RefPicList1[ refIdxL1N ] ) is not equal to 0:

tempMv[ 0 ] = ( mvL0N[ 0 ] + Clip3( −32768 −131072, 32767 131071, Sign( distScaleFactor * mvL1N[ 0 ] ) * 	(8-392)
	( ( Abs( distScaleFactor * mvL1N[ 0 ] ) + 127 )  >>  8 ) ) + 1 ) >> 1

tempMv[ 1 ] = ( mvL0N[ 1 ] + Clip3( −32768 −131072, 32767 131071, Sign( distScaleFactor * mvL1N[ 1 ] ) * 	(8-393)
	( ( Abs( distScaleFactor * mvL1N[ 1 ] ) + 127 )  >>  8 ) ) + 1 ) >> 1

where distScaleFactor is derived as follows:

td = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicList0[ refIdxL0N ]  ) ) 	(8-394)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td	(8-395)

tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicList1[ refIdxL1N ] ) ) 	(8-396)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 ) 	(8-397)

…