| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0495 |
| --- | --- |

| Title: | CE3: Intra mode coding (Test 3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the experimental results of CE3-3.2 on intra mode coding. It is reportedly shown that, Test 3.2 achieved -0.03% and 0.02% luma BD-rates with around 97% and 99% encoding time, and 100% and 100% decoding time for All Intra (AI) and Random Access (RA) configurations, respectively.

Introduction

As shown in Figure 1, there are Planar, DC and 93 angular intra prediction modes used in VTM3.0 [1]. The number of available intra prediction modes is always 67 for each block. To code an intra mode, a most probable mode (MPM) list of size 6 is constructed by using the intra prediction modes of two neighboring blocks. Firstly, one MPM flag is signaled to indicate whether current mode is MPM or not. Then, if MPM flag is true, truncated unary coding is used to signal the MPM index, otherwise, truncated binary coding is used to signal the remaining modes.

Figure 1: Intra prediction modes in VTM3.0

Test description

In this test, it is proposed to signal 6 MPMs and only 32 remaining modes for each coding unit. The generation of 6 MPMs is the same as that in VTM3.0.

32 remaining modes are derived by adding offset values to the angular neighboring modes in MPM list. Offset values are selected from offsetList1 = {1, 2, 3, 4, 6, 8, 10, 12}. For each offset value in offsetList1, each angular neighboring mode is added to that offset as a remaining mode if not already included. After the derivation process, the following default mode list is used in case the remaining mode list is not full.

{50, 18, 2, 34, 66, 10, 26, 42, 58, 6, 14, 22, 30, 38, 46, 54, 62, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 52, 56, 60, 64, 17, 19, 49}

To code an intra prediction mode, an MPM flag is firstly signaled. If the MPM flag is true, then truncated unary coding is used to signal the MPM index, which is the same as that in VTM3.0. Otherwise, 5-bit fix length codeword with bypass coding is used to signal the mode index of the remaining modes.

Experimental results

This test is implemented on top of VTM3.0 software [1]. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-L1010 [2].

Table 1: Coding performance of CE3 Test 3.2 over VTM-3.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.11% | -0.01% | 98% | 98% | 0.03% | 0.15% | 0.25% | 99% | 98% |
| Class A2 | -0.02% | -0.01% | -0.06% | 96% | 99% | 0.03% | 0.19% | 0.21% | 100% | 99% |
| Class B | -0.02% | -0.03% | 0.01% | 97% | 101% | 0.02% | 0.13% | 0.03% | 100% | 99% |
| Class C | -0.06% | -0.04% | -0.14% | 96% | 100% | 0.01% | 0.11% | 0.10% | 99% | 99% |
| Class E | 0.01% | -0.05% | -0.04% | 97% | 99% |  |  |  |  |  |
| Overall | -0.03% | -0.04% | -0.05% | 97% | 100% | 0.02% | 0.14% | 0.13% | 99% | 99% |
| Class D | -0.07% | 0.03% | -0.19% | 97% | 101% | 0.01% | -0.24% | 0.10% | 99% | 99% |
| Class F | -0.21% | -0.20% | -0.27% | 96% | 100% | -0.14% | -0.11% | -0.05% | 99% | 98% |

Conclusion

In CE3 Test 3.2, it is proposed to only signal 6 MPM and 32 remaining modes for each coding unit. The proposed method has small encoding time decrease and average -0.03% and 0.02% luma BD-rates on top of VTM3.0 under AI and RA configurations.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1010, Macao, CN, 3–12 Oct. 2018.

Acknowledgements

We would like to thank Qualcomm for cross-checking this CE test.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).