| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0513-v1 |
| --- | --- |

| Title: | CE7-related: Context modeling of pred_mode_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao Seungwook Hong Haitao Yang Jianle Chen | Email: | yin.zhao@huawei.com Seungwook.Hong@huawei.com haitao.yang@huawei.com Jianle.Chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes 2 methods to improve the context modeling of CU-level intra/inter coding mode flag pred_mode_flag by referring the prediction mode of its neighbor CU. The method1 refers prediction mode of above and left CU. The method2 further takes into account of the size of CU when both neighbor CU are not available or inter mode. The result of method1 shows {-0.09%/-0.02%/-0.05%} BD-rate gain, respectively for RA, LDB and LDP configurations, and method2 shows {-0.09%/-0.03%/-0.10%} BD-rate gain, respectively for RA, LDB and LDP configurations over VTM-3.0.

Introduction

The pred_mode_flag is per CU syntax element specifies whether the current coding unit is coded as intra/inter prediction mode. pred_mode_flag equals to 1 specifies that the current coding unit is coded in intra prediction mode and 0 specifies inter prediction mode or not available[1]. This pred_mode_flag uses single context using regular coding.

Proposed Method

Method1 uses 3 contexts bins for coding pred_mode_flag depending on the prediction mode of above and left CU.

ctxId = (a available and intra?1:0)+(b available and intra?1:0)

Method2 further classify the case by the size of CU when the ctxId is zero.

sizeC = widthCU * heightCU

ctxId = (a available and intra?1:0)+(b available and intra?1:0)

ctxId += (ctxId == 0) ? (sizeC > 256) ? 4 : ((sizeC > 64) ? 3 : 0) : 0

Table 1 shows the summary of each methods

Table 1: Context Modeling Method of pred_mode_flag. (a) method 1 (b) method 2

| Left | Above | CtxIdx |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 2 |

| Left | Above | sizeC | CtxIdx |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | > 64 | 3 |
| 0 | 0 | > 256 | 4 |
| 0 | 1 |  | 1 |
| 1 | 0 |  | 1 |
| 1 | 1 |  | 2 |

(a)

(b)

Result

The propose methods were implemented in VTM 3.0 and the results shows in table 2 and table 3.

Table 2: performance of method1 over VTM 3.0

|  | Random Access over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.08% | -0.03% | 100% | 100% |
| Class A2 | -0.09% | 0.08% | 0.08% | 100% | 100% |
| Class B | -0.09% | 0.09% | -0.07% | 100% | 100% |
| Class C | -0.08% | -0.05% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.02% | -0.02% | 100% | 100% |
| Class D | -0.03% | -0.01% | -0.06% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Low Delay B over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.11% | 0.09% | 100% | 100% |
| Class C | -0.01% | 0.29% | 0.14% | 100% | 99% |
| Class E | -0.03% | 0.41% | -0.13% | 100% | 99% |
| Overall | -0.02% | 0.15% | 0.05% | 100% | 100% |
| Class D | 0.00% | 0.19% | 0.16% | 100% | 99% |
| Class F (optional) |  |  |  |  |  |
|  | Low Delay P over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.15% | 0.42% |  |  |
| Class C | -0.08% | 0.08% | 0.08% |  |  |
| Class E | 0.00% | -0.22% | -0.64% |  |  |
| Overall | -0.05% | 0.03% | 0.04% |  |  |
| Class D | 0.01% | -0.27% | 0.30% |  |  |
| Class F (optional) |  |  |  |  |  |

Table 3: performance of method2 over VTM 3.0

|  | Random Access over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.06% | 0.01% | 100% | 100% |
| Class A2 | -0.08% | 0.03% | 0.09% | 100% | 100% |
| Class B | -0.11% | 0.16% | 0.03% | 100% | 100% |
| Class C | -0.08% | -0.01% | 0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.04% | 0.06% | 100% | 100% |
| Class D | -0.02% | -0.02% | 0.04% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Low Delay B over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.24% | -0.10% | 100% | 100% |
| Class C | -0.06% | 0.25% | -0.04% | 100% | 98% |
| Class E | 0.06% | 0.05% | -0.29% | 100% | 99% |
| Overall | -0.03% | -0.01% | -0.13% | 100% | 99% |
| Class D | -0.04% | 0.00% | 0.30% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Low Delay P over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.24% | 0.06% |  |  |
| Class C | -0.05% | 0.28% | 0.03% |  |  |
| Class E | -0.20% | -0.56% | -0.95% |  |  |
| Overall | -0.10% | 0.05% | -0.20% |  |  |
| Class D | -0.01% | -0.08% | 0.12% |  |  |
| Class F (optional) |  |  |  |  |  |

References

[1] J. Chen, Y. Ye, S.H. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3)”, JVET-L1002-v1, 12th JVET Meeting, Macau, China, 3–12 Oct, 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).