| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0513-v2 |
| --- | --- |

| Title: | CE7-related: Context modeling of pred_mode_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao Seungwook Hong Haitao Yang Jianle Chen | Email: | yin.zhao@huawei.com Seungwook.Hong@huawei.com haitao.yang@huawei.com Jianle.Chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes 3 methods to improve the context modeling of CU-level intra/inter coding mode flag pred_mode_flag by referring the prediction mode of its neighbor CU. The method1 refers prediction mode in above and left CU. The method 2 further takes into account of the size of CU when both neighbor CU are not available or in inter mode. The method3 excludes the above CU from the context modeling when it is not in the same CTU as the current CU. Method 3 could be also applicable to other syntax elements which include above CU in context modeling so that the dependency of the syntax elements on the line buffer could be removed. The result of method1 shows {-0.09%/-0.02%/-0.05%} BD-rate gain,   method2 shows {-0.09%/-0.03%/-0.10%} BD-rate gain, and method3 shows {XXX/YYY/ZZZ} BD-rate gain, respectively for RA, LDB and LDP configurations over VTM-3.0.

# Introduction

The pred_mode_flag is per CU syntax element specifies whether the current coding unit is coded as intra/inter prediction mode. pred_mode_flag equals to 1 specifies that the current coding unit is coded in intra prediction mode and 0 specifies inter prediction mode or not available[1]. The pred_mode_flag uses single context using regular coding.

# Proposed Method

Method1 uses 3 contexts bins for coding pred_mode_flag depending on the prediction mode of above and left CU.

ctxId = (a available and intra?1:0)+(b available and intra?1:0)

Method2 further classify the case by the size of CU when the ctxId is zero.

sizeC = widthCU * heightCU

ctxId = (a available and intra?1:0)+(b available and intra?1:0)

ctxId += (ctxId == 0) ? (sizeC > 256) ? 4 : ((sizeC > 64) ? 3 : 0) : 0

Table 1 shows the summary of each methods 1 and 2

Table 1: Context Modeling Method of pred_mode_flag. (a) method 1 (b) method 2

| Left | Above | CtxIdx |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 2 |
|  | (a) |  |

| Left | Above | sizeC | CtxIdx |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | > 64 | 3 |
| 0 | 0 | > 256 | 4 |
| 0 | 1 |  | 1 |
| 1 | 0 |  | 1 |
| 1 | 1 |  | 2 |
| (b) |  |  |  |

Method3 extends the method2 to exclude the CU in above from the context modeling when it is out of CTU boundary. It could apply not only to the pred_mode_flag but also to the syntax elements in below so that the dependency of the syntax elements on the line buffer could be removed.

- Affine flag

- Skip flag

- IMV(Adaptive MV precision Mode) flag

- Triangle flag

Formula for the new context modeling of pred_mode_flag includes the syntax elements in above could be written as

ctxId = (a available and intra?1:0) + (b available and intra?(b in same CTU ? 1 : 0) : 0)

# Result

The proposed methods were implemented in VTM 3.0 and the results are showing in table 2, 3 and 4.

Table 2: performance of method1 over VTM 3.0

|  | Random Access over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.08% | -0.03% | 100% | 100% |
| Class A2 | -0.09% | 0.08% | 0.08% | 100% | 100% |
| Class B | -0.09% | 0.09% | -0.07% | 100% | 100% |
| Class C | -0.08% | -0.05% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.02% | -0.02% | 100% | 100% |
| Class D | -0.03% | -0.01% | -0.06% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Low Delay B over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.11% | 0.09% | 100% | 100% |
| Class C | -0.01% | 0.29% | 0.14% | 100% | 99% |
| Class E | -0.03% | 0.41% | -0.13% | 100% | 99% |
| Overall | -0.02% | 0.15% | 0.05% | 100% | 100% |
| Class D | 0.00% | 0.19% | 0.16% | 100% | 99% |
| Class F (optional) |  |  |  |  |  |
|  | Low Delay P over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.15% | 0.42% |  |  |
| Class C | -0.08% | 0.08% | 0.08% |  |  |
| Class E | 0.00% | -0.22% | -0.64% |  |  |
| Overall | -0.05% | 0.03% | 0.04% |  |  |
| Class D | 0.01% | -0.27% | 0.30% |  |  |
| Class F (optional) |  |  |  |  |  |

Table 3: performance of method2 over VTM 3.0

|  | Random Access over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.06% | 0.01% | 100% | 100% |
| Class A2 | -0.08% | 0.03% | 0.09% | 100% | 100% |
| Class B | -0.11% | 0.16% | 0.03% | 100% | 100% |
| Class C | -0.08% | -0.01% | 0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.04% | 0.06% | 100% | 100% |
| Class D | -0.02% | -0.02% | 0.04% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Low Delay B over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.24% | -0.10% | 100% | 100% |
| Class C | -0.06% | 0.25% | -0.04% | 100% | 98% |
| Class E | 0.06% | 0.05% | -0.29% | 100% | 99% |
| Overall | -0.03% | -0.01% | -0.13% | 100% | 99% |
| Class D | -0.04% | 0.00% | 0.30% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Low Delay P over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.24% | 0.06% |  |  |
| Class C | -0.05% | 0.28% | 0.03% |  |  |
| Class E | -0.20% | -0.56% | -0.95% |  |  |
| Overall | -0.10% | 0.05% | -0.20% |  |  |
| Class D | -0.01% | -0.08% | 0.12% |  |  |
| Class F (optional) |  |  |  |  |  |

Table 4: performance of method3 over VTM 3.0

|  | Random Access over VTM-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.13% | -0.01% | 100% | 99% |
| Class C | -0.04% | 0.12% | 0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.03% | 0.07% | 0.06% | 100% | 99% |
| Class F (optional) |  |  |  |  |  |
|  | Low Delay B over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.01% | 0.27% | 0.16% | 100% | 99% |
| Class E | 0.18% | 0.23% | -0.28% | 100% | 97% |
| Overall |  |  |  |  |  |
| Class D | -0.03% | 0.61% | -0.10% | 100% | 97% |
| Class F (optional) |  |  |  |  |  |
|  | Low Delay P over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.03% | 0.10% | -0.17% |  |  |
| Class E | 0.04% | -0.28% | -0.15% |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.05% | -0.23% | 0.20% |  |  |
| Class F (optional) |  |  |  |  |  |

# References

[1] J. Chen, Y. Ye, S.H. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3)”, JVET-L1002-v1, 12th JVET Meeting, Macau, China, 3–12 Oct, 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).