| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0516-v2 |
| --- | --- |

| Title: | Non-CE9.2.1.e: Non-local-mean-based MRSAD and Row-subsampled Search Pattern for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen1, Wei-Jung Chien2, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +886-3-566-1980 1chunchic@qti.qualcomm.com, 2wchien@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal presents a modified MRSAD with non-local mean and a row-subsampled search pattern to reduce the computational complexity of DMVR. The former is the same MRSAD as for CE9.2.1.e, except that the mean value of MRSAD is replaced by a constant shared over all searched blocks. This constant is the mean value of the block at the center of the search area. The latter is to confine the searching granularity of integer-interval motion vector refinement from 1-pel interval to 2-pel interval for vertical motion vector component. With row-subsampled MRSAD as in CE9.2.1.e, only the samples with even row indices on the search area are involved in the process of motion vector refinement, and thus the search area size is reduced by half. In addition, DMVR with 8x8/4xN CUs disabled were tested on top of the proposed methods with or without pre-fetching extra samples for motion compensation. As reported from the experimental results, non-local mean and row-subsampling search provide 1.05% and 0.95% of Y BD-rate saving relative to the VTM-3.0 anchor, respectively. The number becomes 0.87% when both are combined. When 8x8/4xN CUs are disabled with and without extra pre-fetched samples, the Y BD-rate savings are reported to be 1.06% and 1.08%, respectively. All the methods are combined to produce 0.84% of Y BD-rate saving.

# Introduction

Decoder-side Motion Vector Refinement (DMVR) [1] is a bi-prediction technique for Merge blocks with two initially signalled motion vectors (MV) that can be further refined by using bilateral matching prediction. In CE9.2.1.e [2], during the matching process of DMVR, the sample values on the search area are interpolated by using 2-tap bilinear filter. The cost function used in the matching process is row-subsampled MRSAD, in which the block sample mean is computed based on the whole samples within matching blocks. After the matching process is done, the refined MVs are used for motion compensation in the prediction stage, boundary strength calculation in deblock filter, temporal motion vector prediction for subsequent pictures and cross-CTU spatial motion vector prediction for subsequent CUs. DMVR can be switched on and off adaptively without signalling overhead. Its MV refinement process operates only when the following conditions are all satisfied.

- CU size is with at least 64 pixels and the CU height is with at least 8 pixels.

- The initial block difference between Pred L0 and Pred L1 falls within a threshold.

- The coding unit is a Merge block.

- Uni-directional, ATMVP and Affine mode are all excluded.

- Only one of (POCref0 – POCcur) and (POCref1 – POCcur) values is negative.

# Technical Description

## MRSAD with Non-local Mean

MRSAD with non-local mean is proposed to decrease the decoding latency during the bilateral matching process as each matching block need to be scanned twice (one for computing mean and the other for computing per-sample mean-removed difference) in order to produce MRSAD. Specifically, this contribution takes the mean value from the samples at the center of the search area. It is noted that this mean value is identical to the one computed when DMVR performs bilateral matching with a refinement motion vector equal to (0, 0). Then, this mean value is shared over all local blocks on the search area during bilateral matching. Accordingly, the two-scanning process at local block level for MRSAD can be avoided.

## Row-subsampled Search Pattern

The row-subsampled search pattern imposes a restriction on the vertical direction of integer-interval motion vector refinement. With the restriction, the searching granularity of integer-interval motion vector refinement toward vertical direction is doubled from 1-pel interval to 2-pel. The equivalently searchable points on the search area are (∆x, ∆y), where ∆x=0, ±1, ±2 and ∆y=0, ±2. It is noted that 0 would becomes the only option for ∆y at the second iteration of 6-point search [2] if the selected ∆y is not equal to 0 at the previous iteration. With

row-subsampled MRSAD, only the samples with even row indices (i.e. 0, ±2, ±4, …) on the search area are involved in the process of motion vector refinement (including the computation of block mean values and bilateral matching errors). Thus, this contribution only interpolates the samples with even-row indices on the search area. It is noted that this also affects the prediction-based early termination as in CE9.2.1.e. Its threshold value has to be further divided by 2.

As in Figure 1, only the reference samples marked inside the red dash-line boxes are involved in bilinear interpolation. These reference samples can be logically mapped together and thus the bilinear interpolation can be directly applied, except that the vertical phase for interpolation should be divided by two because the spatial distance between two rows is doubled.

The parametric fractional-pel MV refinement [1] needs to be adjusted according to the doubled spatial distance between two rows. Specifically, the definition of ∆yfrac is given as follows while the definition of ∆xfrac is left unchanged.

∆xfrac = floor( (16 * (E-1,0 – E1,0)) / (2(E-1,0 + E1,0 – 2E0,0)) ),

∆yfrac = floor( (2 * 16 * (E0,-2 – E0,2)) / (2(E0,-2 + E0,2 – 2E0,0)) ),

where E0,-2, E0,2, E0,0, E1,0 and E-1,0 are the bilateral matching error at (∆x,∆y)=(0,-2), (0,2), (0,0), (1,0) and (-1,0), respectively. It is noted that ∆yfrac is always set equal to 0 when ∆y≠0.

Figure 1. The samples inside the dash-line box are the filter inputs for horizontal interpolation. The effective number of filter input samples for horizontal interpolation is the same as (w+5)*(h/2 + 3). The number of filter output samples is (w+4)*(h/2 + 3). (e.g. 4x4 CU and DMVR search range from -2 to +2).

## DMVR with 8x8 and 4xN CUs Disabled

This contribution additionally disables the use of 8x8/4xN CUs for DMVR to evaluate the coding performance impact. Extra reference samples may be prefetched to fill the padded area before bilinear interpolation and the final motion compensation.

# Experimental Results

The experiments of the proposed technique were conducted using the CE9.2.1.e software [2] and the common test conditions [3] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio relative to the VTM-3.0 anchors. The VTM anchor results were generated using the VTM-3.0 software and the common test conditions. Parallel encoding and decoding were applied to Random Access cases for anchors and tests.

Six testes are conducted, and the respective performance results were summarized in Table 1. In addition, the performance results of CE9.2.1.e are also provided for reference.

(Test a) CE9.2.1.e + MRSAD with non-local mean;

(Test b) CE9.2.1.e + Row subsampled search pattern;

(Test c) CE9.2.1.e + Row subsampled search pattern + MRSAD with non-local mean.

(Test d) CE9.2.1.e + No 8x8/4xN CUs for DMVR;

(Test e) CE9.2.1.e + No 8x8/4xN CUs for DMVR + Reference sample prefetching;

(Test f) Test c + Test e.

- BD-rate results and processing time ratios relative to VTM-3.0.

|  | (a) CE9.2.1.e + Non-local Mean |  |  |  |  | (b) CE9.2.1.e + Row Subsampled |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.20% | -1.25% | -1.38% | 102% | 113% | -1.19% | -1.21% | -1.43% | 103% | 111% |
| Class A2 | -1.68% | -1.93% | -1.86% | 107% | 119% | -1.51% | -1.75% | -1.74% | 102% | 114% |
| Class B | -0.81% | -0.91% | -1.00% | 103% | 112% | -0.70% | -0.85% | -0.91% | 102% | 110% |
| Class C | -0.76% | -0.88% | -0.91% | 103% | 114% | -0.66% | -0.81% | -0.82% | 102% | 113% |
| Overall | -1.05% | -1.17% | -1.22% | 103% | 114% | -0.95% | -1.09% | -1.15% | 103% | 112% |
| Class D | -0.71% | -1.00% | -0.91% | 102% | 115% | -0.62% | -0.82% | -0.80% | 102% | 113% |
| Class F | -0.31% | -0.47% | -0.41% | 104% | 117% | -0.28% | -0.40% | -0.32% | 103% | 116% |
|  | (c) CE9.2.1.e + Row Subsampled + Non-local Mean |  |  |  |  | (d) CE9.2.1.e + No 8x8/4xN |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.06% | -1.14% | -1.25% | 106% | 114% | -1.29% | -1.29% | -1.46% | 103% | 109% |
| Class A2 | -1.40% | -1.63% | -1.60% | 104% | 116% | -1.72% | -1.90% | -1.85% | 103% | 112% |
| Class B | -0.66% | -0.79% | -0.86% | 104% | 111% | -0.81% | -0.87% | -0.97% | 103% | 107% |
| Class C | -0.60% | -0.72% | -0.73% | 103% | 114% | -0.72% | -0.81% | -0.82% | 103% | 112% |
| Overall | -0.87% | -1.01% | -1.05% | 104% | 113% | -1.06% | -1.14% | -1.21% | 103% | 110% |
| Class D | -0.57% | -0.87% | -0.69% | 102% | 113% | -0.62% | -0.75% | -0.78% | 102% | 114% |
| Class F | -0.26% | -0.38% | -0.29% | 104% | 116% | -0.32% | -0.44% | -0.36% | 104% | 117% |
|  | (e) CE9.2.1.e + No 8x8/4xN + Prefetching |  |  |  |  | (f) Test c + Test e |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.30% | -1.31% | -1.53% | 102% | 111% | -1.07% | -1.10% | -1.25% | 102% | 110% |
| Class A2 | -1.73% | -1.93% | -1.86% | 102% | 113% | -1.36% | -1.58% | -1.51% | 101% | 110% |
| Class B | -0.82% | -0.96% | -1.00% | 102% | 109% | -0.63% | -0.72% | -0.82% | 102% | 107% |
| Class C | -0.74% | -0.81% | -0.88% | 101% | 109% | -0.54% | -0.71% | -0.69% | 101% | 108% |
| Overall | -1.08% | -1.19% | -1.25% | 102% | 110% | -0.84% | -0.97% | -1.01% | 102% | 109% |
| Class D | -0.64% | -0.88% | -0.78% | 102% | 113% | -0.50% | -0.71% | -0.55% | 102% | 111% |
| Class F | -0.30% | -0.41% | -0.35% | 103% | 116% | -0.22% | -0.36% | -0.31% | 103% | 115% |

- BD-rate results and processing time ratios of CE9.2.1.e relative to VTM-3.0.

|  | CE9.2.1.e |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.30% | -1.32% | -1.52% | 103% | 112% |
| Class A2 | -1.78% | -1.95% | -1.96% | 103% | 116% |
| Class B | -0.86% | -0.97% | -1.04% | 103% | 112% |
| Class C | -0.82% | -1.00% | -0.95% | 103% | 114% |
| Overall | -1.12% | -1.25% | -1.30% | 103% | 113% |
| Class D | -0.77% | -0.96% | -0.90% | 102% | 115% |
| Class F | -0.37% | -0.48% | -0.44% | 102% | 114% |

# Conclusion

In this report, a non-local-mean MRSAD and a row-subsampled search pattern with block size restriction and extra samples prefetched for motion compensation are presented. It is recommended for the committee to consider further study in the context of a CE.

# References

- S. Sethuraman, “CE9: Test 9.2.6 (combines CE9.2.15/9.2.16 with elements of 9.1.4 and 9.2.5),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L0173, Oct. 2018.

- S. Sethuraman, “CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0147, Jan. 2019.

- F. Bossen, J. Boyce and et. al., “JVET Common Test Conditions and Software Reference Configurations for SDR Video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1010, Oct. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).