| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0525-v2 |
| --- | --- |

| Title: | CE10-related: Simplification of intra prediction in CIIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Po-Han Lin Yao-Jen Chang | Tel: Email: | PohanLin@fginnov.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

In VTM3.0, combined inter and intra prediction (CIIP) combines an intra prediction and a merge indexed prediction. When it applies the CIIP, the available luma intra prediction modes are set to 4, which are Planar, DC, horizontal and vertical modes. This contribution proposes a selection scheme to simplify the CIIP by keeping only 1 available intra prediction mode for a CU. Whether the PDPC and reference smoothing filter are enabled or not is also studied. The results show that 0.07%/0.04%/0.06% BD-rate change in RA condition and 0.15%/0.09%/0.29% BD-rate change in LB condition with slightly encoding time decreasing are observed if the PDPC and the reference smoothing filter are enabled, and 0.12%/0.03%/0.01% BD-rate in RA condition and 0.19%/0.21%/0.26% BD-rate in LB condition with slightly encoding time decreasing are observed if the PDPC and the reference smoothing filter are disabled.

Introduction

The CIIP in VTM-3.0 combines an intra prediction and a merge indexed prediction. A syntax is signalled to indicate an extra intra prediction is enabled/disabled when the current CU is merge indexed prediction. If CIIP is applied, an additional syntax is signalled to indicate the luma intra prediction mode. The luma intra prediction mode is selected from {DC, Planar, HOR, VER} and the chroma intra prediction mode is set as DM mode without signalling. In the current software, HOR mode is not available when the ratio of width to height of a CU is larger than 2, and VER mode is not available when the ratio of height to width of a CU is larger than 2.

The proposed simplifications

The Planar mode requires a pixel-wise predictor computation which is complicated for implementation of the CIIP. Therefore, it is proposed to remove the Planar mode in the CIIP. A selection scheme is also proposed to keep only 1 available luma intra prediction for the CIIP. This available intra prediction mode is adaptively decided by the CU shape. If the ratio of width to height of a CU is larger than 2, VER mode is selected for intra prediction. If the ratio of width to height of a CU is less than 1/2, HOR mode is selected. Otherwise, DC mode is selected for intra prediction. If the HOR or VER mode is selected, the adaptive weighting method in VTM3.0 are also applied to combine the inter and intra predictor. The syntax of the intra mode and the MPM derivation process for the proposed scheme are removed accordingly. Whether the PDPC and the reference smoothing filter are enabled is also evaluated in this contribution. In summary, the two simplification methods for CIIP are studied in this contribution:

- Automatically select 1 luma intra prediction mode out of {DC, HOR, VER } without explicit signaling.

- Disable PDPC and reference smoothing filter for the selected intra prediction mode.

Experiment result

The simplifications are implemented on top of VTM3.0 and the software is performed under common test conditions [1]. There are two tests conducted:

Test 1: Automatically select 1 luma intra prediction mode out of {DC, HOR, VER } without explicit signaling

Test 2: Test 1 with PDPC disabled and reference smoothing filter disabled

Table 1 shows the experimental results of the Test 1. It shows that 0.07%/0.04%/0.06% BD-rate change in RA condition and 0.15%/0.09%/0.29% BD-rate change in LB condition. Table 2 shows the experimental results of the Test 2. It shows that 0.12%/0.03%/0.01% BD-rate change in RA condition and 0.19%/0.21%/0.26% BD-rate change in LB condition.

Table 1. Summary results of Test 1 over VTM3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.06% | -0.05% | 99% | 98% |
| Class A2 | 0.04% | 0.07% | 0.07% | 99% | 100% |
| Class B | 0.04% | 0.06% | -0.03% | 99% | 100% |
| Class C | 0.11% | 0.06% | 0.25% | 99% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.04% | 0.06% | 99% | 101% |
| Class D | 0.04% | -0.06% | 0.06% | 96% | 94% |
| Class F (optional) | 0.04% | -0.04% | 0.05% | 94% | 89% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.17% | -0.04% | 0.17% | 97% | 101% |
| Class C | 0.15% | 0.31% | 0.46% | 99% | 98% |
| Class E | 0.12% | 0.01% | 0.27% | 98% | 101% |
| Overall | 0.15% | 0.09% | 0.29% | 98% | 100% |
| Class D | 0.01% | 0.23% | 0.31% | 99% | 95% |
| Class F (optional) | 0.02% | 0.15% | 0.23% | 97% | 91% |

Table 2. Summary results of Test 2 over VTM3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.03% | 0.01% | 99% | 95% |
| Class A2 | 0.07% | 0.00% | 0.14% | 99% | 99% |
| Class B | 0.12% | 0.05% | -0.07% | 98% | 96% |
| Class C | 0.11% | 0.04% | 0.00% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.03% | 0.01% | 99% | 97% |
| Class D | 0.05% | -0.01% | 0.00% | 95% | 93% |
| Class F (optional) | 0.04% | 0.03% | 0.05% | 93% | 86% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.23% | 0.00% | 0.34% | 96% | 103% |
| Class C | 0.16% | 0.30% | 0.39% | 98% | 97% |
| Class E | 0.16% | 0.44% | -0.03% | 96% | 102% |
| Overall | 0.19% | 0.21% | 0.26% | 97% | 101% |
| Class D | 0.17% | -0.07% | 0.20% | 99% | 92% |
| Class F (optional) | 0.11% | 0.42% | 0.48% | 97% | 90% |

Conclusion

This contribution proposes a simplification method of intra prediction in the CIIP. Test 1 shows that 0.07%/0.04%/0.06% BD-rate change in RA condition and 0.15%/0.09%/0.29% BD-rate change in LB condition over VTM3.0. Test 2 shows that 0.12%/0.03%/0.01% BD-rate change in RA condition and 0.19%/0.21%/0.26% BD-rate change in LB condition. Also, a slightly encoding time decreasing is observed. Since the complexity of the proposed design is relatively lower than the other proposed designs, it is suggested to consider it for the adoption into VVC.

Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, Oct. 2018.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).